Utility "NEC-2 for MMANA" (Ver. 1.2) is intended for calculation of antenna models made and optimized in program MMANA. Calculation is made with use of  NEC-2, including calculations above a surface of the ground in Sommerfeld-Norton model and calculations with use NEC-2 Extended thin wire kernel. Also in "NEC-2 for MMANA" it is possible to make calculation in a case when some wires in MMANA model are in dielectric isolation. A maximum quantity of processable segments – 3000, NEC-2 engine compiled by Arie Neede, Netherlands. All files of the program are placed in one directory.

"NEC-2 for MMANA" reads a file created MMANA with expansion *.maa, and converts it in acceptable for calculation on NEC-2 the form. Restrictions at converting the following:

- Sets of the combined wire in MMANA and sets of stacks are ignored. The combined wire and stacks can be counted with manual description in MMANA for all wires of combined wire antenna or stacks.

- Description of shifted position of sources or loadings from the beginning, the end or the center of a wire is ignored. I.e., for example, if in MMANA position of a source as w2b7 "NEC-2 for MMANA" will imagine position of a source as w2b is given and will set it in the first segment of wire N 2. For calculation in view of shifted source it is possible to split a wire in MMANA on two parts and to establish a source in the beginning or the end of one of these two parts.

- Loading in S-parameter is not acceptable.

Segmentation for NEC-2 calculations.

For calculation in NEC-2 the establishment of segmentation occurs as follows.

- For a wire in MMANA segmentation such as "-1", "-2", "-3" program sets for calculation in NEC-2 linear segmentation with density lambda / (DM1*EC), i.e. with the greatest, offered MMANA, density of segments on a wire is given.

- For a wire in MMANA segmentation such as "0" program sets for calculation in NEC-2 linear segmentation with density lambda/DM2, i.e., actually, according to segmentation in MMANA.

- For segmentation with parameter > 0, splitting into segments occurs according to MMANA.

For converting the segmentation of wires in MMANA with parameter "0" is seemed optimal. With parameters of segmentation in MMANA "-1", "-2", "-3" establishment of segmentation for NEC-2 frequently turns out unfairly dense, that increases in "NEC-2 for MMANA" time of calculation over necessary. Also the common number of segments can become more allowable, limited used version NEC-2, and the length of segments can become too small for accurate calculations.

Calculation above real ground.

"NEC-2 for MMANA" sets for calculation external conditions for antenna models (Free Space, Perfect Ground, Real Ground) according MMANA specification registered in corresponding *.maa file. For a case of the real ground the Sommerfeld-Norton model of the ground is established by default, however, if in MMANA model some wires are touching ground calculation with the Sommerfeld-Norton ground can not be made. In this case there is an opportunity to specify ground model as Perfect Ground.

Parameters of the real ground (dielectric constant and conductivity) via MMANA-file are not transferred. They are necessary to input manually in " NEC-2 for MMANA ". The surface of the ground is supposed in "NEC-2 for MMANA" unlimited and homogeneous.

Calculation of model of the aerial with wires in dielectric isolation.

The metal filament (vein) of wire is supposed as having the cylindrical form, isolation on it is located also as a cylindrical layer. MMANA model radii are encountered as radii of metal filament of wires. For taking in account of influence of isolation loading by additional inductance along all isolated wire is introduced with its thickening up to radius with isolation. Inductance calculated under the formula:

L=μ0⋅1−1ε⋅kabs2⋅lnRr, Hmeter

Where R- radius of wire with isolation,

r- radius without isolation,

ε- relative dielectric constant of isolation,

kabs- velocity factor.

As before calculation kabs precisely is not known, but its values for practical cases of wire antennas on are not far from 1. For calculation it is assumed kabs=0.95. Even at its value is 1 the accuracy of calculation is not lower than about 0.3-0.4 %. With value used in the program accuracy of calculation is really fit for practical use.

The given formula is received in the assumption that R, r << λ, d2Hϕ(R)dz2⋅1Hϕ(R)≈d2Izdz2⋅1Iz≈const, where - Hϕ, Iz a magnetic field around of wire (on an external surface of dielectric isolation, z - changes along a wire) and a current in wire, the length of wire also should be much more than its radius with isolation. For overwhelming majority of practical wire antennas these conditions can be counted fulfilled.

Idea of isolated wire calculation belongs to Alexander Yurkov, RA9MB and based on determination of surface impedance Zsurf=EH at dielectric surface and boundary condition for fields on edge between medias.

Calculation options.

"Find MMANA file".

This button user can find the *.maa file for calculation. Its name together with path is highlighted in the appropriate window.

"Refresh".

By button "Refresh" model data of antenna in program are updated. This button is useful when working with antenna model proceeds simultaneously in MMANA and in "NEC-2 for MMANA". It is possible to make changes of model of antenna in MMANA and to save them in a file. If the name of MMANA file is not changed, by button "Refresh" the utility "NEC-2 for MMANA" updates earlier read data from MMANA a file. In the present version (ver.1.2) updating is necessary and is made for the data about an environment of the antenna and a sample of impedance for SWR calculation.

"Calculate at original of MMANA file".

By the button "Calculate at original of MMANA file" converting and calculation on NEC-2 is performed for models of antenna as it is given in *.maa a file. Results of calculation and the diagram can be looked on a sheet "RP plots" and "Table of results".

"Angles for radiation pattern calculation".

In a window "Angles for radiation pattern calculation" user can set an angles needed to calculate the azimuth diagram (elevation angle needed) and for the elevation diagram (azimuth angle needed).

"Ground model".

In a window "Ground model" the type of the ground and its parameters are set.

"Dielectric coated wire calculations".

In the given window parameters for calculation of antenna with isolated wires are set, calculation is made by button "Calculate" and "Calculate with wires in dielectric coat" (sheet " Tables of results ").

In a field "Radius of wires to be coated" the radius of wires needed to be dressed in isolation (i.e. this radius should be one of designated in MMANA) is set. At calculations it is supposed, that one or several wires in MMANA have the radius appropriate to radius of a metal vein of an isolated wire, - these wires and will be coated in isolation.

In a field "Radius of wire with coat" the radius of a wire with isolation is set (radius of its metallic vein is set "Radius of wires to be coated").

In a field "Dielectric constant of coat" relative dielectric constant of isolation is set.

In a field Frequency" frequency of calculation is set. In this field value of frequency from MMANA file after calculation by the button "Calculate at original of MMANA file" appears.

In a field "Make all wires shorter on" (and scroll bar connected to it) sets how shorter it is needed to make ALL wires of antenna. At its difference from 0 all wires will be shorted for calculation, without dependence from their radius (i.e. without dependence all of them in isolation whether or not). Value of this field not changes contents of MMANA file.

Calculation is made by button "Calculate" or "Calculate with wires in dielectric coat" (on a sheet "Tables of results").

Making calculation several times at different frequencies it is possible to find resonant frequency for the antenna with wires in isolation.

If all wires in MMANA a file have identical radius all of them can be designed as isolated (this radius it is necessary to designate in a field "Radius of wires to be coated"). Then several calculations with different parameter in field "Make all wires shorter on" it is possible to find how wires must be geometrically shorter in isolation. If wires in MMANA have different radiuses (in isolation can be designed only separate of them) to change length of isolated wires better directly in MMANA and to make calculation in "NEC-2 for MMANA" with parameter in field "Make all wires shorter on" equal 0.

"Use NEC-2 Extended thin wire kernel".

Sets in NEC-2 calculation with use of Extended thin wire kernel.

"Show NEC-2 input".

On this option in a window on a sheet "Data sheet" NEC-2 input information is shown.

The sheet "RP plots".

On it diagrams (azimuth and elevation) and results of last calculation are shown. In a window with results it is possible to make remarks and comments. On Popup menu it is possible to save diagrams and results.

Through radio-buttons in a window "Polarization components" it is possible to sketch diagrams for polarization components of far field. In MMANA the component "Phi" is designated as horizontal, "Theta" as vertical.

Results of calculation are deduced also in a window on a sheet "Data sheet". It is possible to copy its contents to clipboard on a Popup menu.

Dimitry Fedorov, UA3AVR,

ua3avr@au.ru