**********

FaxCode ST

**********

Copyright (c) 1998 Dave J Miller

All Rights Reserved

FAXCODE ST IS A SHAREWARE PROGRAM

=================================

FAXCODE ST is protected under UK and International copyright law, and is

NOT public domain software. Dave J Miller owns the copyright to this

program and the document files.

You are hereby granted a limited licence to use this copy. You are not

allowed to alter the program in any way, to do so is an infringement of my

rights. SHAREWARE is NOT Public Domain!

If you use this program more than once then you obviously have a need for

it. You are therefore obliged to pay me the small sum of 6.00 (six UK

pounds). See "Registering FaxCode ST" for full details

FaxCode ST - inventory

======================

When distributed through any means whatsoever, this total package must not

be separated.

FAXCODE.PRG	The program

FAXCODE.DOC	The documentation as standard ASCII text file

FAXCODE.RSC	The resource file

REGISTER.DOC	Registration form

CIRCUIT.PI3	Interface circuit diagram in PI3 format

CIRCUIT.IMG	Interface circuit diagram in IMG format

About FaxCode ST

================

FaxCode ST is a decoding program for the short wave listener. It uses

software tone decoding (FSK) techniques to allow the use of a very simple

hardware interface and to simplify the tuning process. It will decode fax

and RTTY at all the standard speeds and settings and allow the user to

print pictures and text or save them to disk.

System Requirements

===================

FaxCode ST should work with any standard Atari ST computer in high

resolution mode. It will work with 512K memory but 1M or more is

recommended to view complete fax charts. It is running at the limits of the

STs speed so GEM menus are not used and any programs which run in the

background such as time displays should be disabled or removed. For the

same reason it is not possible to run it on a colour monitor using a high

res emulator. The fax print routine is designed to run with Epson

compatible 24 pin printers.

Hardware Interface

==================

FaxCode uses a simple comparator interface powered by the ST. Details  of

the circuit diagram can be found in the files CIRCUIT.IMG and CIRCUIT.PI3.

Experienced constructors should be able to build this on strip board and

fit it in a D-type shell. Commercial interfaces designed for use with JVFAX

or HAMCOMM should be compatible if DSR (pin 6) is connected to CTS (pin 5)

Fax Mode

========

From the initial dialogue box select FAX and OK to enter fax mode. The top

line shows various settings and a spectrum tuning aid. e.g.

IOC=576 RPM=120 PAGE:CURRENT=1 USED=1 MAX=19 APT:ON MODE=APT  _____________

^      ^

---------------------------------------------------------------------------

IOC: This defines the aspect ratio of the received fax and the 2 standard

settings of 288 and 576 are available.

RPM: This is the speed at which the fax image is scanned and defines the

time to print one line. 120 rpm gives a scan line of 0.5 second.The

standard settings of 60, 90, 120 and 240 are available.

PAGE: Each page of a fax occupies two screens (left & right) and a fax

chart will occupy several pages. The CURRENT page is the one being viewed,

the USED pages are how many pages in memory are in use and where the

incoming data is printing to and the MAX page is the maximum amount of

pages available. The MAX figure depends on how much memory is installed up

to a maximum of 19 pages.

APT: This stands for automatic picture transmission and when enabled the

program will automatically respond to various tones transmitted by the fax

station. The tones are used to start and stop fax reception and set the IOC

and RPM.

Tuning

------

Fax is transmitted with a shift of 800Hz and the receiver should be tuned

so that the 2 tones on the spectrum display peak over the markers. The

black tone is on the left and white on the right. The BFO offset should be

set to +1900Hz if you have this feature otherwise set to USB and tune

1900Hz low of the published frequency.

Keyboard Commands

-----------------

A - toggle APT mode on/off

I - set IOC (index of co-operation). Toggle between 288 & 576

R - set RPM. Cycles through 60,90,120,240 rpm

F1 - re-synch (wait for start signal and/or phasing signal)

F2 - manual synch

F10 - exit fax mode

P - print screen. Must be halted

S - save screen(s) to disk. Must be halted

F - toggle spectrum display on/off

H - halt

space - resume when in halt mode

up arrow,down arrow - page up/down

right arrow, left arrow - move to right/left screen

ctl right arrow, ctl left arrow - shift screen right/left

del - clear all screens

help - view help screen

Modes

-----

APT - APT in use. Waiting for start tone.

SYNC - Waiting for phasing signal

FAX - fax receive mode

HALT - reception halted

END - all pages full and decoding stopped. Need to press del to continue

Explanation of APT/SYNC modes

-----------------------------

APT ON:-  APT is the fully automatic mode. When enabled the decoder will

wait for the start tone. When this is detected it will change to SYNC mode

and wait for the phasing signal.The phasing signal is used to synchronise

to the edge of the fax and then it will start displaying the data and

change to FAX mode. When the fax has finished a stop tone is sent and the

decoder will detect this, stop displaying data and revert to APT mode ready

for the next fax. The start and phasing signals also tell the decoder what

IOC & RPM is going to be used for the fax. The decoder interprets this and

sets the appropriate IOC and RPM.

APT OFF:-  The decoder can still sync to the phasing tone but does not

respond to the start and stop tones so will not set IOC/RPM or stop

reception at end of fax. To synch when you hear the phasing signal or wait

for it press F1 to switch to SYNC mode. F2 will over-ride this and start

receiving. If the fax is not aligned at the left hand edge it can be

shifted right or left using control right/left arrows.

If the signal is noisy or distorted APT mode may not work properly and it

is possible that a false stop signal will be detected. In this case it is

best to disable APT mode.

RTTY Mode

=========

From the initial dialogue box select RTTY and OK to enter RTTY mode. The

display at the top shows the current settings and a spectrum tuning aid.

e.g.

SPEED=50   N/R=N   UNSHIFT ON SPACE=N   CODES=N   P=N

___________________

LOGFILE:                                                 ^          ^

--------------------------------------------------------------------------

SPEED: This shows the baud rate. The speeds 45.5, 50, 75 & 100 are

available

N/R: Normal or reverse

UNSHIFT ON SPACE: If set to Y(es) then the decoder will switch to letter

shift mode after every space.

CODES: When set to Y(es) will also display the codes for line feed,

carriage return, letter shift & figure shift.

P: Output to printer enabled Y(es), N(o)

LOGFILE: If saving to logfile is enabled will show the path and filename.

Tuning

------

RTTY is transmitted with varying shifts between 170 Hz & 850 Hz depending

on the station. Luckily FaxCode automatically detects the shift and no

settings are required. Even the centre frequency is not critical. Just tune

the receiver in data mode (+1000 Hz BFO offset) or USB 1000 Hz below the

published frequency so that the spectrum is displayed somewhere between the

2 pointers. You will see the 2 tones as peaks on the spectrum display.

Keyboard commands

-----------------

B - set baud rate. Cycles through 45.5, 50, 75 & 100 bd

R - toggle normal/reverse

C - toggle codes display on/off

U - toggle un-shift on space on/off

L - toggle letter/figure shift

H - halt

space - resume when in halt mode

O - open logfile

E - end logfile

P - toggle printer on/off

V - enable view mode (i.e. review data already received in the buffer)

Esc - exit view mode

up arrow,down arrow - scroll up/down when in view mode

Registering FaxCode ST

======================

FaxCode ST is a shareware program. If you find it useful you are obliged to

register your copy. This will cost 6 and I will send you a master disk

registered in your name with the nag screens disabled. I will also include

some extra programs to decode CW, SITOR & RTTY using a sampler cartridge

and RTTY tuning unit. For registration outside Europe add 1. All

registered users will be entitled to a free up-grade (return master disk or

send 1 for new disk) to version 2.0 which will also decode CW (morse

code). Payment is by cash, cheques (drawn on UK banks only) and postal

orders. Eurocheques, American Express money orders and bank drafts are

accepted but only in UK POUNDS STERLING. Add an extra 1.00 (one UK pound)

for postage outside of Europe (total 7.00 seven UK Pounds).

Please see REGISTER.DOC for full details.

Contacting the author

=====================

My address is:

Dave J Miller

33 Springfield Park

Twyford

Reading

Berkshire

RG10 9JG

England

email:		davemiller@zetnet.co.uk

Future improvement ideas

========================

CW mode

Complex modes - SITOR A, SITOR B etc

Full screen spectrum display for signal analysis

Decode time and set computers clock from standard time & frequency station

Fixed shift settings in RTTY for when reception is poor or station pauses

between transmissions.

References

==========

Amateur Radio Computing Handbook, Joe Pritchard

Radioteletype Code Manual, Klingenfuss

Atari ST Internals, Bruckmann, Englisch, Gerits

Atari ST GEM Programmer's Reference, Szczepanowski & Gunther

Introducing Atari ST Machine Code, R Pearson & S Hodgson

Atari ST Machine Language Programming Guide, S Field

Acknowledgements

================

Thanks to Jerry Glenwright & Mike Richards, both of Short Wave Magazine for

their encouragement in developing this software

Thanks to Mark Baines for help with the documentation.