F T V		Version 1.1b

Multimode software for Sound Blaster compatible hardware.

Supports WEFAX, FAX, SSTV, NAVTEX, RTTY and CW

Copyright (C)1994-2001, Brian E. Cauchi, 9H1JS. All rights reserved.

FTV can be used to receive images from Weather Satellites and Terrestrial Facsimile Stations, text transmissions from radio telex services, Morse code, as well as Amateur Radio FAX, SSTV, RTTY and CW signals. It can also transmit Amateur Radio FAX and SSTV. No external hardware, other than a radio, is required, as all the necessary signal processing is implemented in software.

A. TABLE OF CONTENTS 1. INTRODUCTION 4 1.1 Supported image communication modes 5 1.2 Supported data communication modes 5 2. SYSTEM REQUIREMENTS 5 3. INSTALLATION 5 3.1 Windows 95/98 6 3.2 MS-DOS 6 3.3 Plug & Play Sound Cards Under MS-DOS 6 3.4 PCI Sound Cards 6 4. CALIBRATION SETTINGS 7 5. PROGRAM START UP AND TROUBLESHOOTING 7 6. EXPLORING THE PROGRAM 8 6.1 Menus 8 6.2 Help screen and short cut keys 8 6.3 Cursor keys & the Tab menu 9 6.4 Exercises 10 6.4.1 Pan and zoom 10 6.4.2 Loading a picture 10 6.4.3 Video control functions 10 6.4.4 Image processing 11 6.4.5 Image manipulation 11 6.4.6 Loading a NOAA weather satellite image 12 6.4.7 Loading a colour SSTV picture 12 6.4.8 Loading a tape-recorded colour SSTV picture 13 7. OPERATION 13 7.1 Something on Sound Cards 13 7.1.1 Interconnections 13 7.1.2 Mixer control 13 7.1.3 Limitations with Hardware Version 1.xx 14 7.2 The Tuning Scope display 14 7.2.1 Adjusting your radio for WEFAX, FAX and SSTV 15 7.2.2 Adjusting your radio for RTTY, NAVTEX and SITOR-A 15 7.3 WEFAX reception 15 7.4 FAX reception 16 7.5 SSTV reception 16 7.6 FAX / SSTV transmission 17 7.7 Receiving data modes 18 7.8 CW reception 19 8. ADVANCED FEATURES 19 8.1 Automatic reception using APT 19 8.2 Automatic reception using Squelch 20 8.3 Scheduled FAX reception 20 8.4 FAX browser 21 8.5 Concurrent operations 21 8.6 Offline image processing and conversion 22 External commands 23 9. GENERAL INFORMATION 23 9.1 Conditions for use 23 9.2 Registration and ordering 23 9.3 Contact information 24 9.4 About the author 24 9.5 Acknowledgments 24 10. APPENDICES 25 10.1 Shortcuts 25 10.2 Quick menu reference guide 26

B. INDEX OF TABLES AND FIGURES Table 1 General information to get you started with your radio. 4 Table 2 List of supported image communication modes. 5 Table 3 Typical extended memory utilization. 5 Table 4 Configuration checks. 6 Figure 5 Images resulting from incorrect line rate settings. 7 Table 6 Menu colours in FTV. 8 Table 7 The result of mode settings. 9 Table 8 Functions assignable to the cursor keys from the Tab menu. 9 Table 9 Filters and their effect on the image. 11 Figure 10 Tuning Scope display of various FAX signals. 16 Figure 11 SSTV signal. 16 Figure 12 RTTY signal. 18 Table 13 Data modes function keys 18 Figure 14 CW signal. 19 Table 15 Morse (CW) function keys. 19 Table 16 APT (automatic picture transmission) format. 19 Table 17 Squelch trigger conditions & minimum duration. 20 Table 18 Scheduled reception file syntax. 20

# INTRODUCTION

FTV is a multipurpose radio signal decoding package that can be used to receive images from Weather Satellites and Terrestrial Facsimile Stations, text transmissions from radio telex services, Morse code, as well as Amateur Radio SSTV signals. It can also transmit Amateur Radio FAX and SSTV images. No external hardware, other than a radio, is required, as all the necessary signal processing is implemented in software. The main features include:

- WEFAX, FAX, SSTV, NAVTEX, RTTY and CW reception.

- Transmits 8 FAX modes (2 in colour) and 18 SSTV modes (10 in colour).

- FAX operation supports APT signal recognition and scheduled reception.

- No external decoder or converter box is required.

- Signal analysis is provided for both image and data modes.

- Recovery from wrong mode settings is possible without loss of data.

- Images may be loaded and prepared for transmission while receiving another.

- Raw data streams may be saved and reloaded for offline analysis.

- Program works in real time.

If you are new to these modes, the table below contains some general information to get you started with your radio.

| FTV mode | Signal origin | Frequency/MHz | Radio mode | Bandwidth |
| --- | --- | --- | --- | --- |
| NAVTEX | Terrestrial | 0.518 | SSB | 2 kHz |
| FAX | Terrestrial | 2-20 | SSB | 2 kHz |
| SSTV | Amateur radio | 7.040, 14.230, 21.240, 28.680 | SSB | 2 kHz |
| WEFAX | Low orbiting satellite Geostationary satellite | 137-138 1695 | FM | 25 kHz |

Table   1    General information to get you started with your radio.

## Supported image communication modes

| Operating Mode | Reception | Transmission |
| --- | --- | --- |
| Monochrome WEFAX | 60, 90, 100, 120, 180, 240 lpm | No |
| Monochrome FAX | 60, 90, 100, 120, 180, 240 lpm | Yes |
| Colour FAX | 120, 240 lpm | Yes |
| Monochrome SSTV | SC-1 8, 16, 32s Robot 8, 12, 24, 36s | Yes |
| Colour SSTV | Martin 1, 2, 3, 4 Scottie 1, 2, 3, 4, DX Wrasse SC-2 180s | Yes |

Table   2    List of supported  image communication modes.

## Supported data communication modes

Baud rates: 30 to 500 baud

Modes: Baudot / RTTY, SITOR-A, SITOR-B / NAVTEX, Morse code / CW

# SYSTEM REQUIREMENTS

The minimum system requirements are:

- 80386 processor with hard disk

- 256k free XMS memory

- VESA compatible SVGA graphics card

- Sound Blaster compatible sound card

XMS memory required depends on the sampling window and the duration of the image. The AM sampling window is fixed, while the FM sampling window is accessed through Setup Demodulator Window. Generally speaking, 4Mb free XMS memory is recommended.

| Mode | Sampling window | XMS used per minute | XMS per 15 minutes |
| --- | --- | --- | --- |
| FAX | 0.35ms | 170k | 2.5Mb |
| WEFAX | 0.2ms | 290k | 4.3Mb |

Table   3    Typical extended memory utilization.

# INSTALLATION

The files supplied on the floppy disk are compressed, and have to be installed onto your hard disk before FTV can run. To install the software, insert the floppy disk in drive A: and type ‘A:INSTALL’. The software will be installed in a sub-directory named C:\FTV.

## Windows 95/98

FTV runs satisfactorily under Windows 95/98 with the following limitations:

- Direct-X must be installed.

- Shelling to DOS while receiving results in signal loss.

- Task-switching (Pressing Alt-Tab) while receiving or transmitting results in abnormal program termination.

Direct-X is a set of software drivers provided by Microsoft to optimize gaming performance under Windows 95/98. If Direct-X is not installed, you will need to restart your computer in MS-DOS mode to be able to use all the features of FTV.

After installing FTV, two new shortcuts, named FTV-DOS & FTV-WIN, will be added onto your Desktop. FTV-DOS runs the program in MS-DOS mode, while FTV-WIN runs it under Windows.  These shortcuts may be conveniently manipulated into the Start menu, as well as used directly to launch FTV.

In the event that you have to run FTV in MS-DOS mode, and have problems using the shortcut provided, you may restart your computer in MS-DOS mode as follows: enter the ‘Start Menu’, select ‘Shut down …’, then select ‘Restart in MS-DOS mode’ and click OK.

## MS-DOS

If you are using MS-DOS 5.0 or 6.0, no specific configuration is needed in most cases. The driver HIMEM.SYS is required, and the use of SMARTDRV is recommended to improve performance when loading and saving files. To run the program from DOS, type ‘FTV’ in the C:\FTV sub-directory.

| File to edit | Line to add, if not already present |
| --- | --- |
| CONFIG.SYS | DEVICE=C:\DOS\HIMEM.SYS |
| AUTOEXEC.BAT | SMARTDRV C+ |

Table   4    Configuration checks.

## Plug & Play Sound Cards Under MS-DOS

If you have a Plug & Play Sound Card, make sure the card is initialized properly before loading FTV. This may require the use of a Plug & Play Configuration Manager and/or other utilities supplied with your card. For example, if you have a Creative Sound Blaster AWE64, you must run the program CTCM, which is a Plug & Play Configuration Manager.

## PCI Sound Cards

PCI soundcards commonly supplied with new PC's (e.g. PCI64V, PCI128V, PCI512 and their clones) may not work in reception mode due to lack of support for recording in the legacy emulation drivers provided by the vendor.

# CALIBRATION SETTINGS

Many sound cards have crystal oscillator blocks which provide excellent stability, but their frequency can be ±5% of the value marked. Because of this, an initial calibration procedure needs to be carried out before FTV can successfully decode images. If this is not done, received pictures may be severely slanted, and the Automatic Mode Recognition facility may not work.

The idea is to start by receiving a known signal, then trimming the Line Rate manually, until the image is perfectly straight. Then use Setup Demodulator ClockCal to memorize the calibration setting.

| The line rate is far too slow. Note how the lines are slanting to the left. | Image is discernible, but the line rate is still too slow. | The original picture, with the correct line rate. | The Line Rate is too fast. Note how the lines are slanting to the right. |
| --- | --- | --- | --- |

Figure  5    Images resulting from incorrect line rate settings.

The recommended procedure is to decode a FAX station known to transmit at 120 LPM. If you prefer to use SSTV, make sure you are using Free-Run mode (Shift-F8) before effecting Line Rate adjustments.

The procedure is then repeated for WEFAX, and again, make sure you are using Free-Run mode (Shift-F8) before effecting Line Rate adjustments.

Calibration data is stored in the configuration file (AmCal, FmCal, TxCal & DataCal). Calibration for data modes and for transmit mode is only possible by manually editing the relevant parameter in the configuration file FTV.CFG.

# PROGRAM START UP AND TROUBLESHOOTING

Start the program by typing FTV. The program starts by testing the audio hardware & XMS memory manager. If errors are encountered with the sound card, the program will warn you about it, but it will not abort, making it possible to try out most features (except reception / transmission itself).

In the unlikely event that your computer does not have XMS memory, you will have to add HIMEM.SYS to your CONFIG.SYS file, as outlined in section 3.

If all goes well, the program will switch the display to graphics mode. The default file ‘ftv.tga’ will be automatically loaded and displayed. The default graphics mode is 640 x 480, 256 colours. If you press Enter, a blue menu bar with yellow text will appear at the top of the screen.

If none of this happens, it is possible that your graphics card is not compatible with the program. You may press File, followed by Quit, to exit the program. If the display is unstable, press Ctrl-Alt-Del immediately to minimize the risk of damaging your monitor.

Some Super-VGA BIOS do not have VESA support built in, in which case the program may report an error like ‘Mode not supported on this card’. If this happens, check your graphics card drivers disk for a VESA BIOS Extension utility. This is a resident program which you install before using applications that need VESA support. I know this is necessary for OAK VGA's, for example.

# EXPLORING THE PROGRAM

In FTV, there are several ways to issue commands. Most commands are accessed through a Menu Bar, but some are accessed using special shortcut keys. Some frequently done adjustments are handled using a special menu, the Tab menu, which assigns the cursor keys to specific tasks.

## Menus

In general, the entire screen is used for image display. The main menu is displayed on the top of the screen when it is invoked by pressing Enter. Menu selections are made by pressing the relevant highlighted capital letter. The main menu and its sub-menus may be closed by pressing Escape. Commands in the main menu may also be accessed directly, that is, by pressing the relevant letter, without having to first invoke the main menu by pressing Enter.

All menu bars have a blue background, but the text changes colour according to the type of menu.

| Type of menu | Text colour | Notes |
| --- | --- | --- |
| Regular menus | Yellow | Shortcut keys guaranteed to work where possible. |
| Special menus | Green | Often include parameters which may be changed using the cursor keys. |
| Transmit menus | Cyan |  |

Table   6    Menu colours in FTV.

## Help screen and short cut keys

F1 (Help) will display the help screen, outlining all short cut keys. Additional help is provided through Pop-Up Help Windows that may be toggled using Shift-F1. Most of the short cuts are self explanatory, but note that many of them have no direct menu equivalent.

F2 (Mode select) is the Mode control shortcut. The main operating mode is controlled by using the Up/Down cursor keys, while the secondary mode setting is controlled by using the Left/Right cursor keys.

In FTV, there are in effect, 3 distinct mode settings, depending on which window is displayed, and what function is being used.

| Mode setting | Condition | Result |
| --- | --- | --- |
| RX | When in Rx window | Sets the Rx mode & the display mode |
| DISPLAY | When in Tx window | Sets the local display mode |
| TX | When in Tx menu | Sets the Tx mode |

Table   7    The result of mode settings.

F3 (View as Fax) allows you to view the current image as a plain fax without actually modifying any parameters. A second invocation restores to the former mode. This function comes in very useful when recovering badly synchronized signals.

F4 (Toggle view) allows you to toggle between the Tx window, which contains loaded images, and the Rx window, which contains received data.

## Cursor keys & the Tab menu

The cursor keys are assigned different functions by using the Tab key. When used in conjunction with the Control key, a coarse adjustment is applied.

| Function | Description | Left/right | Up/down |
| --- | --- | --- | --- |
| Rate | Adjust line rate | Yes | No |
| Phase | Adjust horizontal shift | Yes | No |
| Frame start | Adjust frame start | No | Yes |
| Frame end | Adjust frame end | No | Yes |
| Pan | If zoomed, pan left, right, up or down | Yes | Yes |

Table   8    Functions assignable to the cursor keys from the Tab menu.

## Exercises

### Pan and zoom

Here are some things you can try with the default picture loaded:

- Press Home a couple of times to zoom out,

- Press End a couple of times to zoom in (magnify),

- Press Tab, and select Pan,

- Use the cursor keys to pan around the image,

- Press Ctrl-End to fit the whole image on the screen.

The X and Y keys may be used to adjust the aspect ratio. Ctrl-Home resizes the image so that it covers as much of the screen area as possible while preserving the aspect ratio. Ctrl-End stretches the image to cover the entire screen area.

### Loading a picture

If the main menu is not displayed, press Enter until it does. Press File Load,  select the file ‘testcard.raw’, and press Enter.

### Video control functions

This picture contains special patterns which will allow us to explore various features of FTV. Try out the following video control functions:

- Press F5 to increase the brightness, Shift-F5 to decrease,

- Press F6 to increase the contrast, Shift-F6 to decrease,

- Press F7 to invert the picture, and once more to restore,

- Press Ctrl-F6 to set to Lithographic mode,

- Press Ctrl-F5 to restore Brightness and Contrast to default setting.

### Image processing

FTV provides various image processing tools. Filters provide noise reduction, low pass, high pass & edge detection functions. Histogram equalization is used to optimize the Brightness & Contrast settings for the portion of the image being displayed. Sub-frame arithmetic is provided for images from NOAA weather satellites, which consist of two images, side by side.

Use File Load, select the file ‘lady.raw’, and press Enter. Use Process Histogram (Ctrl-F7) to optimize the Brightness & Contrast settings. These settings may be reset to their default state once again by using Ctrl-F5.

As a result of the contrast enhancement following the Histogram Equalization operation, the image appears a little noisy, so we shall apply the ‘Median N.R.’ filter. To do this, use Process Filter, then select the ‘Median N.R.’ filter using the arrow keys. Use Apply to process the image.

Load the file ‘testcard.raw’ once more, so that we may explore the ‘Impulse N.R.’ filter. Note how the third block from the top, inside the circle, consists of a black background, filled with simulated noise, with increasing intensity, from left to right. Such impulse type noise can be filtered by FTV.  Try it out. You may want to try out some of the other filters, too.

| Image manipulation Filter | Effect on image |
| --- | --- |
| Impulse N.R. | Removes isolated spikes |
| Median N.R. | Removes HF noise, cleans up image |
| Soften | Low pass filter, blurs image |
| Sharpen | High pass filter, sharpens image |
| Edge enhance | Sharpens edges on the image |
| Edge detect | Only edges remain visible |

Table   9    Filters and their effect on the image.

It is possible to carry several image manipulation procedures, concerning the orientation of the image, alignment with the display, and cropping.

The way the image is displayed is accessed through the View menu:

- View Orientation Rotate-90° will turn the picture 90°,

- View Orientation Upside-down will turn the picture 180°,

- View Orientation Mirror will mirror the picture from left to right,

- View Orientation Flip will flip the picture upside down,

- View Orientation Default will restore the normal view,

- View Left displays the left half of the image,

- View Right displays the right half of the image

- View All display the entire image once more.

The following Line Rate adjustment procedure is fundamental to FAX operation as well as when fixing badly synchronized WEFAX & SSTV signals:

- Press Tab, and select Rate,

- Press the right cursor key a few times, and note how the picture slants. Using Ctrl-Right results in a coarse adjustment.

- Press the left cursor key a few times, to restore the picture. Don’t forget that you can use Ctrl-Left for a coarse adjustment.

The image may also be shifted horizontally:

- Press Tab, and select Phase,

- Press the right cursor key a few times, and note how the picture shifts. Using Ctrl-Right results in a coarse adjustment.

- Practice restoring the picture as above.

Sometimes, the top and/or bottom parts of the picture need to be cropped. Frame Start and Frame End (Tab menu) provide a way to crop the top and bottom of the picture.

On startup, FTV assigns the Left/Right cursor keys to the Line Rate adjustment, and the Up/Down cursor keys to the Frame Start adjustment, as these are the ones most commonly used.

### Loading a NOAA weather satellite image

Load the file ‘noaa.raw’. The image consists of two monochrome frames, as sensed by two separate channels on the satellite, visible on the left, and infra red on the right. The picture was received in a South-North pass (upside down) and shows a clear view of Europe, the Mediterranean, and North Africa.

To quickly select the left portion, use View Left. As the image is a bit weak in contrast, use Process Histogram, and note how the image is enhanced. To view the right portion, use View Right. As the image is flooded, use Process Histogram (or Ctrl-F7) once again to enhance the image.

### Loading a colour SSTV picture

Load the file ‘lady.raw’.  This picture was received off-air and saved directly to disk. The sync response can be set by using Shift-F8, to Free-run, Standard or Super. Set to Free-run mode and note its benefits - the ragged edges, particularly around the callsign banner and left margin, smoothen up. Free-run mode is recommended for off-air operation. Its disadvantage is that synchronization will not be achieved unless the Line Rate is precisely set.

Press Ctrl-Right once, to temporarily uncalibrate the Line Rate. You will notice that the bottom left part of the picture changes colour. Further uncalibration will of course result in a completely garbled picture. FTV has a special function to the rescue, Phase/sync lock (Ctrl-F8), which will analyze the image and trim the Line Rate accordingly.

For those of you who do not know how colour SSTV works, press F3 (View as Fax). This will redisplay the picture as a FAX, and three separate frames, corresponding to green, red and blue, appear. Inverting the image (F7) will reveal a bright white vertical band at the screen margins, which is the synchronization (sync) signal. Pressing F3 once more will restore the former mode.

The sync signal is used as a marker at the start of each line, and the primary colour segments follow in sequence. A composite colour picture is then assembled by FTV.

### Loading a tape-recorded colour SSTV picture

Load the file ‘sealand.raw’. This picture was recorded on a tape cassette. In the old days, it was only possible to store images on tape! Unfortunately, when playing back tape recorded SSTV images, they sometimes acquire characteristic wavy effects. This is due to wow & flutter and tape stretch. You may use Shift-F8 to set the Sync Response to ‘Super (for tape recordings)’.

You can use F3 (View as Fax) once more just to see how warped the picture really is! Press F3 again to restore the former mode.

# OPERATION

## Something on Sound Cards

### Interconnections

If you are using a stereo sound card, it is a good idea to apply the signal to both channels. This may be easily done by shorting the tip & ring of the stereo jack plug that goes into the sound card.

The connection may be direct, or through a 1:1 transformer. Using a transformer reduces the likelihood of RF noise from the computer getting into the radio. In any case, use screened leads, and route these away from strong magnetic fields arising from monitors etc.

### Mixer control

All Sound Blaster compatible cards should work with this program. In some cases, problems are encountered due to non-standard mixer chips used on supposedly compatible cards. In addition, FTV does not support a mixer in cards with Hardware Version prior to 3.xx. In such cases, it is still possible to use FTV by using external mixer control.

Before you can use external mixer control, edit the files ‘ftv-rx.bat’ & ‘ftv-tx.bat’ so that the appropriate mixer control utility is called. Then, run FTV using the X command line option. The mixer control panel in the Tuning Scope will become inoperative, and instead, these batch files will be called by FTV as needed.

### Limitations with Hardware Version 1.xx

If FTV reports a Hardware Version of 1.xx, then one thing that in rare cases may need setting up is the Setup Demodulator Grid filter.  Incorrect setting of this parameter will result in a visible grid of very widely spaced dots. The coarseness of these dots increases with slower PC's. On my computer, which has a 486DX 33MHz CPU, it is set to 2. Increase this parameter until the grid no longer appears when a 1.9 kHz sine wave of about 1V r.m.s. is injected into the sound card.

## The Tuning Scope display

When receiving signals, the Tuning Scope display is the most useful tool in FTV. It is automatically shown when starting reception, and may be redisplayed by using F9.

The Tuning Scope offers these facilities:

- Real-time monitoring of the input audio waveform (Direct)

- Real-time monitoring of the demodulator waveform (Baseband)

- Real-time monitoring of the spectrum with tuning markers (Spectrum)

- Mixer control panel, level monitor & clipping detector

- Accurate center frequency display

- Rx/Tx memory monitor

- Quick restart feature

Most settings are accessed by using the first letter of the command. For example, to toggle from one tuning Display mode to another, press D.

The Filter feature affects only the display; it does not affect the internally demodulated signal. It is used to smooth the displayed waveform. The Holdoff feature is used to slow down the trace refresh. The Restart feature clears all received data and restarts reception.

If your soundcard contains a supported mixer, the Mixer controls will allow you to select the input source and to set the input level. If the Clip indicator (red) is shown, the input level is too high. Correct input level setup is important for optimum performance, particularly for WEFAX operation. If the Mixer controls in FTV don’t work with your card, external control is possible.

Note that on pressing Escape to quit the ‘Tuning Scope’, there will be no signal loss as long as the Restart feature is not used.

### Adjusting your radio for WEFAX, FAX and SSTV

When working with image communication modes, use the Baseband display mode. The region relevant to the current demodulator setting is shown in gray, while the SSTV sync region is shown in blue. Illegal or out of band regions are shown in red. To the right of the main dislay window is the Analyzer window, which gives a visual indication of the distribution of the signal.

### Adjusting your radio for RTTY, NAVTEX and SITOR-A

When working with data modes, use the Spectrum display mode. The demodulator center frequency is indicated by a vertical yellow line. Tune your radio so that the two peaks that make up the FSK signal lie on either side of this center-line. The mark/space distribution is shown in the Analyzer window.

## WEFAX reception

The WEFAX video sub-carrier is a 2.4 kHz AM signal. As a consequence, it requires careful gain settings, unlike FM. AGC (automatic gain control) will distort the signal, precluding the use of the Microphone input. The Line Input will have to be used instead.

To receive WEFAX signals, select a WEFAX mode by using F2, then press Rx to start reception. The ‘Tuning Scope’ is displayed when starting reception, and may be redisplayed by using F9.

The gray region in the Tuning Scope represents a range of 0% (bottom) to 100% (top) modulation. Set the input level  so that the signal fills the gray region without reaching into the red regions. This may be done by adjusting the volume knob on your radio or by using the relevant Mixer controls. If the Clip indicator is shown, the input level is too high.

On leaving the ‘Tuning Scope’, the received signal is displayed in real-time. You may work on the image while it is still being received, for example, the Rate, Phase, Brightness, Contrast and Orientation may all be adjusted, and the signal may be analyzed too. When reception is complete, press Stop, and you may then save the image.

Keep in mind that as long as the Demodulator (AM / FM / FFT) is not changed, the received signal is not lost when the Mode is changed. The signal is simply processed once more and displayed using the new mode.

## FAX reception

The FAX video sub-carrier is a 1.9kHz FM signal, with a deviation of ±400Hz. Unlike AM, gain settings are not critical, and AGC will have a beneficial effect. The use of the Microphone input is recommended, but using the Line Input is also possible.

To receive FAX signals, select a FAX mode by using F2, then press Rx to start reception. The ‘Tuning Scope’ is displayed when starting reception, and may be redisplayed by using F9, in very much the same manner as for WEFAX.

Check visually that the signal is not clipped, and is located squarely in the gray region. The gray area represents a range of 1.5kHz (bottom) to 2.3kHz (top).

| The start signal, where the carrier is modulated by a 300 Hz tone | The phasing signal, consisting of a white line, interrupted by a black pulse. | A FAX chart, with white symbols on a black background. |
| --- | --- | --- |

Figure 10   Tuning Scope display of various FAX signals.

On leaving the Tuning Scope, the received signal is displayed in real-time. You may work on the image while it is still being received, in the same way as for WEFAX. Phase/sync lock (Ctrl-F8) may be used to automatically center the image if the phasing signal has been received.

If the wrong side-band is selected, a negative image will result, however this may be easily rectified by pressing F7 (Invert).

It is recommended to save Colour FAX frames in RAW format, if you intend to view or manipulate them with FTV.

SSTV reception

Figure 11   SSTV signal.

Tuning SSTV signals requires more care than FAX. The correct side-band must be selected, such that the sync pulses appear in the blue region of the ‘Tuning Scope’. A staircase signal is shown opposite; this is a commonly encountered pattern used to test the gray scale.

Phase/sync lock (Ctrl-F8) may be used to automatically trim the noise band preceding the image, and to provide synchronization. When Mode Analysis (F8) or Phase/sync lock (Ctrl-F8) are conducted on a static image (raw file loaded from disk), or when reception is stopped, both preceding and trailing noise bands are cropped, if both are within 20% of the image size. The line rate is also trimmed accordingly.

SSTV line sync mode (Shift-F8) controls the way the image is synchronized. See sections 6.4.7 & 6.4.8 for further information.

If a frame starts with noise, automatic Phase / sync lock (Ctrl-F8) may not be enough to achieve line lock. This is particularly true for colour modes, when the noise in question consists of fragments of other transmissions. In such a case, trim the top of the frame manually until line lock is achieved.

It is recommended to save Colour SSTV frames in RAW format, if you intend to view or manipulate them with FTV.

## FAX / SSTV transmission

Only TGA files may be transmitted directly, but it is also possible to transmit a RAW image or an unsaved received picture. In such cases, FTV will convert the image to TGA format before proceeding to the transmission menu.

The typical sequence of events followed during FAX/SSTV transmission is:

- Load an image and press Tx to display the transmission menu.

- Add banner on top of the image, overlay text on the image. When overlaying text, position with cursor keys, and size it using the +/- keys. The background colour is transparent; if changed, it remains solid.

- Select a mode to use (setting is inherited from Rx mode).

- Press Enter to start transmission (press any key to abort the transmission).

Set the output power level on your transmitter conservatively. FAX and SSTV are 100% duty cycle modes, with transmissions lasting several minutes; solid state transmitters will get quite warm in the process.

Care must also be taken not to over-modulate your transmitter. Adjust the Mic Gain control while keeping an eye on the ALC meter. If output level from the sound card exceeds the level needed by the transmitter, an external potentiometer may need to be inserted between the sound card audio output and the transmitter microphone input.

## Receiving data modes

Figure 12   RTTY signal.

Press Mode daTa. The display reverts to text mode, and FTV is in a state of continuous reception. Logging appends to the file ‘ftv-data.log’. A single RTTY character is shown opposite. The initial (idle) state is Mark. This is followed by a start bit, followed by five symbols ‘10110’, followed by a stop bit (shown extended).

When working with data modes, use the Spectrum display mode for tuning. The Spectrum display facilitates both the actual tuning and the process of setting up your radio to reduce adjacent interfering signals if it has notch filtering and passband shifting capabilites.

It is also possible to export and import raw data stream files, for offline identification and processing.

Most operations, including changes to decoding parameters such as the baud rate, polarity, and data mode, there is no loss of data. Data in memory simply reinterpreted using the new settings. If you need to clear the data in memory, use F5 (Restart).

| Key | Label | Description | Settings |
| --- | --- | --- | --- |
| F1 | Baud rate | Set baud rate | 30-500 baud |
| F2 | Polarity | Set keying polarity | Normal / Reverse |
| F3 | Unshift | Controls how shift is released | Normal / On space |
| F4 | Display | Controls the data display mode | Normal / Ctrl / Raw |
| F5 | Restart | Clears buffer and restarts reception |  |
| F6 | CatScan | Shells to CatScan program |  |
| F7 | Logging | Logs data to ‘ftv-data.log’ on disk |  |
| F8 | Analyze | Signal analysis |  |
| F9 | Scope | Tuning scope display |  |
| F10 | Quit | Return to FTV image module |  |
| F11 | Morse | Switch to Morse code reader |  |
| F12 | Data mode | Switch to another data mode |  |
| / | File… | Import and export raw files |  |

Table  13   Data modes function keys

## CW reception

Press Mode daTa, followed by F11 to switch to Morse code. Operation is similar to the other data modes, however, logging appends to the file ‘ftv-cw.log’.

The Morse letter V (dot-dot-dot-dash) is shown opposite, in the gray region, which corresponds to the selected frequency band. Each band is about 250Hz wide, and all eight bands are acquired simultaneously. Data in memory is not erased on selecting a new frequency band, but simply reviewed from the beginning, while reception continues.

Figure 14   CW signal.

| Key | Label | Description | Settings |
| --- | --- | --- | --- |
| F1 | Speed wpm | Set keying rate | 5-50 wpm |
| F2 | Auto | Estimate & track keying rate | Off / on |
| F3 | Frequency | Select frequency band | 250-2000Hz |
| F4 | Errors | Controls if errors are displayed | Shown / masked |
| F5 | Restart | Clears buffer and restarts reception | - |
| F6 | CatScan | Shells to CatScan program | - |
| F7 | Logging | Logs data to ‘ftv-cw.log’ on disk | - |
| F9 | Scope | Tuning scope display | - |
| F10 | Quit | Return to FTV image module | - |
| F11 | Data modes | Switch to data mode screen | - |

Table  15   Morse (CW) function keys.

# ADVANCED FEATURES

## Automatic reception using APT

This feature permits unattended monitoring of services that makes use of this format. These include many terrestrial weather facsimile stations, as well as geostationary weather satellites. FTV relies on the 200/300/675Hz start tones and 450 Hz stop tone that are part of the APT (automatic picture transmission) format. Received images are saved automatically to disk.

| Signal | Duration | IOC 576 | IOC 288 | Remarks |
| --- | --- | --- | --- | --- |
| Start tone | 5s | 300Hz | 675Hz | 200Hz for colour fax modes. |
| Phasing signal | 30s |  |  | White line interrupted by a black pulse. |
| Image | Variable | 1200 lines | 600 lines | At 120 lpm. |
| Stop tone | 5s | 450Hz | 450Hz |  |
| Black | 10s |  |  | Neither generated nor needed by FTV. |

Table  16   APT (automatic picture transmission) format.

## Automatic reception using Squelch

Squelch mode allows unattended monitoring of services that do not make use of APT.  These include a few terrestrial facsimile stations, SSTV frequencies, and low orbiting weather satellites. Received images are saved automatically to disk as for APT. The operation of the squelch gate depends on the type of demodulator used.

| Demodulator | Trigger condition | Start | Stop |
| --- | --- | --- | --- |
| AM | Presence of 2.4kHz subcarrier (±10%) and Audio input level > 10% | 3s | 10s |
| FM | Audio input level > 50% | 3s | 10s |

Table  17   Squelch trigger conditions & minimum duration.

## Scheduled FAX reception

Scheduled FAX reception provides a way to monitor specific products by using a schedule file. The schedule file may include calls to other DOS programs, for computer aided receiver control. For example, in conjunction with the Cat-Scan program, a frequency database may be queried and the strongest signal in a group of frequencies selected automatically.

Schedule files have an SKD extension, and may be loaded either from the command line, using the F:<file> option, or by using Auto Scheduled-rx. Schedule files consist of plain text, with commands in the following format:

| Command | Description |
| --- | --- |
| T <time start> <duration> [info] | Set time (UTC) |
| X <command line> | Execute DOS command |
| R[m] <Lpm> <Modulation> [info] | Receive picture(s) |
| S <time> | Start over (typically after 00:00) |

Parameters

Time format is hhmm (or * for current time), duration is in minutes.

The R command is followed by A,D or S, which correspond to APT, Dump & Squelch respectively. If none are specified, Squelch mode will be used by default. - APT operation relies on the presence of Start & Stop tones. - Dump mode will save the entire received signal to disk for the duration specified. - Squelch mode will trigger on the presence of a signal. If using AM, it will trigger on the 2.4kHz carrier (±10%). If using FM, it will trigger on an input level exceeding 50%

Lpm is one of the standard rates: 60,90,100,120,180 or 240. Modulation is AM or FM.

The INFO field is meant for annotation purposes. It is appended to the log file entry.

Table  18   Scheduled reception file syntax.

When invoked, FTV will locate the first applicable entry in the schedule. As all times are in Zulu, the TZ environment variable must be set.

An example schedule file is shown below. An explanation of the action of each line follows.

T  0230 15 "GYA - 0230 - BROADCAST SCHEDULE"	(a)

X CatScan GYA LSB F	(b)

Ra 120 FM	(c)

T 1600 18 “NAM - 1600 - SAT Image VT 1545Z"	(d)

X CatScan 8080.0 LSB F	(e)

Rd 120 FM	(f)

S 0000	(g)

- Let us assume that the above schedule is invoked at 0100Z. FTV will wait until it is 0230Z.

- It will then invoke the CatScan program, to set up the receiver. (In this case, CatScan will look up the frequency database, extract the various frequencies associated with callsign GYA, and get the strongest signal. It will set the radio to LSB and apply the appropriate 1.9kHz shift).

- FTV will enter receive mode, at 120 Lpm, FM. Start and stop signals will be monitored and multiple pictures saved separately. At 0246Z, FTV will stop receiving, and if an incomplete picture is present, this is saved as well.

- FTV will wait for 1600Z.

- It will again invoke CatScan (it will set the radio to 8080.0kHz LSB and apply the appropriate 1.9kHz shift).

- FTV will enter receive mode, at 120 Lpm, FM. This time, start and stop signals are ignored and the entire 18 minutes are dumped to disk. At 1619Z, FTV will stop receiving.

- FTV will wait for 0000Z. The schedule will then restart.

## FAX browser

Pictures received using Automatic Reception may be conveniently browsed using the File Browse function. It is possible to delete unwanted pictures, and to manipulate and save the desired ones. The log of received pictures is kept in the file ‘ftv-auto.log’.

When manually saving pictures, it is possible to use ? as a filename. This will automatically generate a filename and add an entry to the above mentioned log file.

## Concurrent operations

It is possible to do most operations while reception is in progress, without interfering or stopping reception, and without losing any received data. Reception is on the most part, a background task. You may save the data that has been received up to a certain point, load a file for transmission, modify the Tx image, add and modify the banner, and overlay text, among other things.

When changes to the mode settings in the Rx window require a change in the demodulator type, FTV will prompt you whether to restart reception or continue in the former mode, to reduce the likelihood of accidental loss of signal.

When a TGA image has been loaded into the Tx window, changes to the mode settings in the Tx window are possible, but the resulting image may be unusable. FTV will prompt you whether to modify the loaded TGA image or not.

For obvious reasons, Tx Start will stop reception first!

## Offline image processing and conversion

It is possible to perform offline image processing by using FTV as a command line utility. This feature is aimed primarily at users of unattended FAX operation. Typically the system is left running to collect several charts during the day. When the system is attended, the received images can be processed to clean up the received images, and converted from RAW format to TGA, so that they can be viewed and printed with other software programs.

Offline image processing is activated by using the command line option Pnnn:<file>

The nnn parameter is a decimal number corresponding to the filter or combination of filters to be applied:

1=Impulse

2=Median

4=Soften

8=Sharpen

16=Edge enhance

32=Edge detect

Example 1: To convert a file from RAW to TGA without applying a filter, in this case a file called TEST in the TMP folder, use:

FTV P0:tmp\test

Example 2: To convert the above mentioned file from RAW to TGA and applying the Median filter, use:

FTV P2:tmp\test

Example 3: To convert the above mentioned file from RAW to TGA and applying the Impulse and Median filters (1+2), use:

FTV P3:tmp\test

Example 4: To convert a set of files in the TMP folder from RAW to TGA, using one command, use:

for %1 in (tmp\*.raw) do FTV %1

## External commands

You can assign external commands to the Alt-Number keys, so that you can easily launch other programs, such as satellite prediction, rig control and frequency databases.  Commands are read from the file ‘ftv.key’. Valid entires start with a number (0-9) followed by the desired command line. An example assigning a function to the Alt-1 key follows:

1	c:\radio\it\it.exe

# GENERAL INFORMATION

## Conditions for use

This program is licensed for noncommercial use only. It may be copied and used by anyone as long as it is distributed in whole and without profit.  The collection of files on the distribution disk must be distributed in an unmodified form. Unlicensed commercial distribution is prohibited.

The author provides absolutely no warranty. The program is supplied "as is", and you may use it at your own risk. In no event will the author be liable for any damages arising out of the use of this program.  Bug reports, suggestions and feedback are welcome.

## Registration and ordering

If you wish to support FTV, you may choose to register your copy of the software. Your registration, while aiding the continued evolution of the program, entitles you to free upgrades when newer versions are released.

| Item | Cost in US$ |
| --- | --- |
| Registration | 35 |
| Diskette & illustrated user's manual | 15 |

To place your order, state your callsign, full name and address, and send cash payment in US$ (or the equivalent in Sterling, Deutsche Mark, or Italian Lira) to my postal address below. Personal cheques are also accepted but are subject to bank clearance. Eurocheques must be drawn using the same currency as the issuing bank. Postal orders and credit cards are not accepted.

## Contact information

Postal address:	Brian E. Cauchi,

Gawhra, Domenico Cachia Street,

Birkirkara BKR06, MALTA.

Email addresses:	briane@geocities.com

briane@waldonet.net.mt

Web page: 		http://www.geocities.com/SiliconValley/2504

## About the author

| Here I am! My name is Brian, and I'm located in a place called Birkirkara, on the Island of Malta, right in the middle of the Mediterranean sea. Centrally situated, Birkirkara houses around 22,000 of the 370,000 inhabitants on the island. It is hemmed by industrial zones, and no more than a few minutes ride away from the island's main entertainment areas. My amateur radio callsign is 9H1JS and I have been licensed since 1989. |  |
| --- | --- |

## Acknowledgments

My sincere thanks go to:

All the users who registered, for their support.

Geocities, for providing me with a spring board on the Web.

Knut Najmann, DJ1ZN, for his suggestions and assistance in testing FTV.

# APPENDICES

## Shortcuts

SHORTCUT GROUP	ITEM DESCRIPTION	KEY

- Help

- Display help screens	F1

  - Toggle pop-up help window	Shift-F1

- Operating mode

Change operating mode	F2

View image as FAX (reversible)	F3

Toggle Rx/Tx window view	F4

- Palette controls

- Set brightness (use shift to decrease)	F5

- Set contrast (use shift to decrease)	F6

- Invert / gamma (use shift to decrease)	F7

Default brightness & contrast	Ctrl-F5

Lithographic mode	Ctrl-F6

Optimise brightness & contrast	Ctrl-F7

- Reception controls

- Analyse mode	F8

- Set sync response	Shift-F8

- Phase sync/lock	Ctrl-F8

- Display Tuning Scope	F9

- Toggle audio monitor	F10

- Current view

Rotate 90° clockwise	Ctrl-R

Upside down	Ctrl-U

Zoom out	Home

Zoom in	End

Stretch to fit	Ctrl-End

View all	Ctrl-Home

Stretch horizontally	Ctrl-X

Stretch vertically	Ctrl-Y

- Miscellaneous

- Image editing	Display menu that assigns functions to arrow keys	Tab

- External commands	Run command line specified in the file ‘ftv.key’	Alt-Number

- Quit to DOS	Alt-X

## Quick menu reference guide

MENU	ITEM DESCRIPTION	SHORTCUT

- MAIN MENU

- File	Sub-menu

  - Load	Load RAW or TGA image file

  - Save-tga	Save image as TGA file

  - Export-raw	Save image as RAW file (native format of FTV)

  - Browse	Browse images saved automatically

  - Dos-shell	Shell to DOS temporarily

  - Quit	Exit FTV	Alt-X

- View	Sub-menu

  - All	Zoom out so that the entire image fits in the screen	Ctrl-Home

  - Left	Zoom to left half of image (for NOAA WEFAX)

  - Right	Zoom to right half of image (for NOAA WEFAX)

  - Zoom	Select a region to zoom into

  - Orientation	Sub-menu

    - Default	Set origin to top,left

    - Rotate-90	Turn image 90° clockwise	Ctrl-R

    - Upside-down	Turn image 180°	Ctrl-U

    - Mirror	Mirror image from left to right	Ctrl-X

    - Flip	Flip image upside down	Ctrl-Y

- Mode	Sub-menu

  - Analyze	Analyse mode	F8

  - Modify	Modify mode without changing the line rate

  - Select	Change to a new mode	F2

  - Rate	Set the line rate (LPM) parameter

  - Ioc	Set the IOC parameter

  - Decoder	Sub-menu

    - Am	Set demodulator to AM (for WEFAX)

    - Fm	Set demodulator to FM (for FAX/SSTV)

    - Rgb-order	Set RGB order for colour FAX/SSTV

    - Sync	Sub-menu

      - Process	Lock to sync (or to phasing signal if using FAX)	Ctrl-F8

      - Hilite	Highlight sync content (in green)

      - Level	Set the sync level

      - Response	Set the sync response	Shift-F8

      - Width	Set the sync width

      - Default	Set sync parameters to default values

    - Wave-import	Load & demodulate a wave file.

  - rTty/cw	Switch to RTTY/CW reception mode

- Process	Sub-menu

  - Filter-image	Apply a filter to the image

  - Histogram	Optimise brightness & contrast settings	Ctrl-F7

  - Palette	Edit colour palette (cloud/land/sea or false colour)

  - Composite	Extract composite images from NOAA frames

MENU	ITEM DESCRIPTION	SHORTCUT

- Setup	Sub-menu

  - sVga-mode	Sub-menu

    - Low-640x480	Set screen mode to 640x480 256 colours

    - Medium-800x600	Set screen mode to 800x600 256 colours

    - High-1024x768	Set screen mode to 1024x768 256 colours

  - Demodulator	Sub-menu

    - Sample-rate	Set the sample rate parameter (in kHz)

    - Window	Set the sampling window parameter (in milliseconds)

    - Grid-filter	Set the grid filter parameter (in dots)

    - Clock-cal	Derive the calibration parameter

  - dvs-cat-Port	Set the ComPort number

- Rx	Sub-menu

  - Restart 	Clear buffer and start reception

  - Stop	Stop reception	Ctrl-F9

  - Tuning-scope   	Display Tuning Scope	F9

  - Auto-APT  	Start automatic reception using APT

  - sQuelch  	Start automatic reception using Squelch

  - sKed	Automatic reception using Schedule file

- Tx	Sub-menu

  - Banner	Sub-menu

    - Bars	Add colour bars & grating to top of image

    - Callsign	As above, with callsign

    - Edit	As above, with custom text

    - Qso-msg	As above, but ‘de Call’ is automatically added

    - Show	Show the banner

    - Hide	Hide the banner

  - Overlay	Overlay text on the image

  - Start	Start transmitting the image

  - Mode	Set the Tx mode	F2

- Help	Display help screens	F1