--------------------------------------------------------------------------

DX OnLine

KD5ON Software

Steve Brown

2940 Koop Road

Lima, Ohio 45807

(419) 999-2187

-------------------------------------------------------------------------

DX OnLine is an antenna bearing/range, signal propagation system that runs

on an IBM or compatible personal computer.

It is a result of an effort that started in 1989 and has more than 1,500

hours of effort in it. (Probably more if you ask my XYL.)

Because of the complexity of the programs, I have targeted people with

computers beyond the basic 8088 with a Hercules monitor scenario.

DX OnLine requires a minimum of an 80286 with 640K and a color EGA or VGA

monitor and a hard disk. I will warn you that even on an 80286 some

functions such as the 24 hour MUF prediction will seem slow. Even some

older 80386's will take a drag on the 24 hr. I use a 386-20 with a

co-processor and it hums right along. 80486's work great.

It is my opinion that more and more hams are progressing to more advanced

systems so I decided not to sacrifice features to accommodate older systems.

Although it is not required, a mouse (Microsoft compatible) is highly

recommended. What I feel is one of DX OnLine's nicest feature, the "ROAM"

mode requires a mouse.

Some of DX OnLine's features include:

World Map:   The primary display of DX OnLine is a full color map

of the world.

Browse mode: As you move the mouse around on the world map, the

antenna   bearing and range is constantly updated.

While in browse mode, if you click a mouse button, a window

pops up with the current signal propagation prediction.

For fine movement of the cursor, you may use the arrow keys.

Prompt mode: This mode prompts you for a specific prefix or QTH

(city,   state, country,etc.). The search is such is that it

looks for occurrences of the select parameters keyed.

I like this as I don't have to remember how to spell places

like "KYRGYZSTAN". You could key "KY","YRG","STAN",

etc. and it would find it. Note that keying "STAN" would

also find "TURKMENISTAN", STANLEYVILLE, etc.

One thing to keep in mind is that the system has a limit in

that it only likes 250 matches for the prompted data. So

don't key in "W" for a prefix with no limiting QTH prompt.

~~~~

DX OnLine

Print capability: You may print a listing a DX bearings from your

home QTH to all locations in the target data base.

Temporary home QTH override: you may temporarily override your home QTH

data to allow you to demonstrate DX OnLine for someone else

from their own home QTH or to print a DX Bearing listing

for them.

Gray Line:   A continually (every 5 minutes) updated representation

of the "Gray Line" or day/night terminator is shown on the

primary map display.

Time   The time is shown, both local and UTC.

Grid square: The grid square is shown for both home QTH andTargets.

The target grid square is shown both in prompt and

browse modes as well as on the Antenna bearing listing.

CQ Zone:     The CQ zone is shown for target QTH's while in prompt mode

and on the antenna bearing listing.

QTH data base: A data base of target QTH's is provided. You may edit or

add to this file as you desire.

Prefix cross reference: This is another of what I feel is a nice feature.

A data base of prefix to prefix cross references is provided.

Many are included. You may add to this data base as desired.

This allows you to, for example, key prefix "PP". The X-Ref

routine would determine this to be Brazil with a primary

prefix of "PY"

It also has a USA cross reference. It makes a best guess. For

example, you could key my prefix "KD5" and it would put me

in the "W5" call area.  I know I'm in the W8 area, but I have

future plans to incorporate a callbook database into the

current call assignment strategy, the KC4's in the W4 call

area will have to live with this for now.

Current period propagation prediction: You can pop up a window that will

give a propagation prediction for the current time to the

currently selected target QTH. This can be done by selecting

menu option  m(U)f then (C)urrent. While in browse mode,

clicking either mouse button will go directly to this option.

While in this screen, you may advance the time by 1 hour by

pressing "H" or by 1 day by pressing "D"

24-hour propagation prediction: Displays a full color graph predicting

the propagation for the next 24 hours from yourhome QTH

to the  currently selected target based on the current

Sunspot # or solar flux # you have entered.

~~~

-------------------------------------------------------------------------

INSTALLING  DX OnLine

-------------------------------------------------------------------------

The diskette(s) supplied are NOT in packed format so

installation is a relatively simple matter.

1. Log on to the Root directory

CD \ <enter>

2. Create DXOL directory

MD \DXOL <enter>

3. Change to DXOL directory

CD \DXOL <enter>

4. Put DX OnLine diskette in diskette drive

5. Load diskette to hard drive

Copy a:*.* <enter> (if copying from A drive)

or

Copy b:*.* <enter> (if copying from B drive)

6. If more than one diskette, repeat steps 4 & 5

for each diskette.

(Would only apply if you requested 5-1/4" single density

diskettes [360kb])

~~~

---------------------------------------------------------------------------

DX Online

OPERATING INSTRUCTIONS

---------------------------------------------------------------------------

1. Change to the directory that contains the DX OnLine programs and data

files. If this directory is "DXOL", key  CD \DXOL and press the enter key.

2. Run the program by keying DXOL and pressing the enter key.

The program will go thru a short initialization process after which a

map of the World will be displayed. If you have a Microsoft compatible

mouse installed, you will be placed in the "Browse" mode and the mouse

cursor will be at approximately 0 degrees longitude and 0 degrees latitude

and the  antenna bearing and distance from the home QTH will be displayed

under the map. Note that the initial home QTH information is from my home

QTH of Lima Ohio.

Try moving the mouse and watch the display update. Press a mouse button

and watch what happens.

Notice that just above the map is a menu bar similar to the following:

(M)ode  m(U)f  (P)rint  (F)ile  e(X)it

To select a menu option, key the letter enclosed in parentheses. For example,

the File menu is selected by keying "F". You do not have to press enter.

The first thing you will want to do is configure the system to your home

QTH.To do this:

1. Select the File menu by keying "F"

2. Select edit home QTH by pressing "H"

You may now enter your Call, QTH, Latitude, Longitude and hours to adjust

for UTC. As I said, the initial information shown is for my home QTH so you

may see how the entries should be made.

The UTC hours adjustment is enter as East or West rather than + or -. This

is to coincide with the longitude.

For the continental U.S.:

Time Zone              Enter

EST             5 hr 0 min W

CST             6 hr 0 min W

MST             7 hr 0 min W

PST             8 hr 0 min W

For daylight savings time add 1 hour. You will have to manually adjust this

field when the time changes.

Once you have made your changes, save the Home QTH information and exit.

You will now be returned to the main program and everything will be relative

to your own home QTH.

~~~

DX OnLine

You will now want to configure DX OnLine for your printer.

To do this, select Menu Option (P)rinter. Then select option (C)onfigure.

You will be shown a list of available printers. You may scroll back and

forth through the list to find your printer. Once you have found it,enter

the # that corresponds to your printer.

If your printer is not listed, try "Epson FX" for a dot matrix or H-P

Laserjet for a laser. It is virtually impossible to put every make and

model of printer in the master printer definition file.

If nothing works, all may not be lost. The printer configuration file is

in pretty plain language. The name of this file is "DXOLPRTR.CFG". You may

use any editor to edit this file. The format is as follows:

The file must have EXACTLY 9 fields and in EXACTLY the following order.

(If your printer doesn't use one of the control strings, insert a blank

line).

1.  Printer text description

2.  Printer reset code

3.  Compressed print OFF

4.  Compressed print ON

5.  Proportional print OFF

6.  Enhanced print OFF

7.  Enhanced print ON

8.  ASCII character for Degree symbol

9.  ASCII character for Minute symbol

You may enter any data into the control string fields. To enter a character

that is not available (such as an ESC code), enter a backslant (\) followed

by the 3 position decimal ASCII code for the desired character. Any keyboard

character may be keyed directly. For example, you could key "\027@". The

system would recognize this as esc@ which is the reset control code on

Epson FX printers. If you enter a character with a backslant, be sure to

enter exactly 3 numeric characters following. Don't leave off any leading

zeros.

Because of the multitude of different character sets available on many

printers, the degree and minute symbols may not always be correct. They

are initially set to correspond with the standard PC character set. This

should work in most cases. If it doesn't, look in your printer manual to

find the proper ASCII character needed and edit file DXOLPRTR.CFG as

described above.

If you make manual changes to the printer configuration file, you might

want to incorporate these changes into the master printer definition file.

This file is called "DXOLPRTR.DEF". It is in exactly the same format as

DXOLPRTR.CFG except that it contains information for all available printers.

Again, be very careful that any printer definition records added to this

file contain exactly 9 fields (or lines).

~~~

DX OnLine

The following is a list of the Primary Menu options and the secondary

prompts that will appear as a result of selecting the primary option.

(M)ode

(R)oam mode    Changes to ROAM mode (if you have a mouse)

(P)rompt mode  Changes to the prompt for target QTH mode

(T)emp QTH     Toggles the use of a temporary Home QTH.

If going into a temporary QTH, displays a screen

to enter the temporary QTH information.

m(U)f

(C)urrent      Displays propagation predictions as of the current

time. This is the same as hitting a mouse button in the

ROAM mode.

24 hour (P)rediction  Computes and displays a graph of the

predicted signal propagation from the current time

thru the next 24 hours from your home QTH to the

currently selected target QTH.

Change (S)sn  Change the Sunspot # (Solar Flux) to the current

values. This is available from a number of sources

including WWV and ARRL bulletins. A SS# of 175 is

set as an itial value.

(P)rint

dx (A)ntenna bearing list  Prints and antenna bearing  chart from

your home QTH (or a temporary QTH) to alllocations

in the target QTH data base. The information shown

is Prefix,QTH,Longitude,latitude,long path bearing,

short path bearing,short path range in statute miles,

nautical miles and kilometers,the target Grid Square

and the CQ zone.

(R)egister   Prints a form to register DX OnLine. Since this is

a registered copy, this is here as a courtesy in case

you or a friend wish to order another copy.

(D)ocumentation  Prints this documentation.

(C)onfigure printer  Configures DX OnLine to function with your

printer.

e(X)it

(Y)es        Exit DX OnLine and return to system

(N)o         No, do not exit yet

~~~

DX OnLine

Note:  while in prompt mode, an additional menu option is shown.

(T)arget           This brings up a prompt to select a prefix and/or

target QTH.

When prompting for a target, keep the following in mind:

The system looks for occurrences of the keyed data. The following

are examples of prefix and/or QTH prompt selections:

PREFIX               QTH                            RESULT

------            ---------               ------------------------------

F                                        All prefixes starting with "F"

(F,FG,FH,etc.)

LON                  London,England, etc.

G                   LON                  London England

PORT                 Lisbon,Portugal  Portland,OR

Portland,ME  etc.

UGAL                 Lisbon, Portugal  etc.

GX                                       All "G" prefixes - used prefix

X-ref to convert

If you have any major problems. let me know.

73 es Gud DX

Steve, KD5ON