Introduction:

It is suggested that the user have a good knowledge of bpq aand radio

Networking before jumping into this windows version of the G8BPQ Switch.

BPQAXIP.DLL supports the encapsulation of AX.25 frames

in IP, as defined in RFC1226.

Environment.

The program uses Winsock Version 2. This is provided

as standard in NT 4 and Windows 98, but must be installed

on Win 95 before the program can be used. The Winsock2 for 95

runtime should be included with this program - if not, you

will need to download it from Microsoft (ws2setup.exe).

The driver is loaded when BPQ32 initialises the port.

This occurs when the first program to use the DLL is

loaded. Other BPQ32 programs may be loaded and unloaded,

but if the first program exits before all the others,

the AXIP dll will be deleted, and the other programs

will crash. To get round this problem, I've written

a simple application (BPQ32.EXE) which can be run first,

and does nothing but keep the switch and AXIP dlls in

memory. This program should be run before any other

programs that use the BPQ32 interface.

Configuration.

The driver is defined to BPQ32 as an External port:

PORT

ID=AXIP Link

TYPE=EXTERNAL

DLLNAME=BPQAXIP.DLL

QUALITY=200

MAXFRAME=4

FRACK=7000

RESPTIME=1000

RETRIES=10

PACLEN=120

ENDPORT

Note the DLLNAME parameter, which is new to BPQ32.

The DLL reads a configuration file BPQAXIP.CFG, which

defines the mapping from AX.25 callsign-ssid, to an IP

address.This file needs to be in the current directory when

BPQ32 is loaded.

#	Config file for BPQAXIP

#Command	Last digi		Remote IP address

MAP		G8BPQ-7 		10.2.77.1

MAP 		BPQ7			10.2.77.1

MAP 		NODES			10.2.77.1

The software at the moment only allows IP addresses -

it does not yet support lookup of host names.

The software will work with an IP broadcast address,

but if the destination is on the same net as the switch,

it will receive its own broadcast frames.

The latest version fixes the ssid-15 problem, and allows wildcards. If you specify an ssid, only that will match. If you dont, all ssids will match. So if you only want to send stuff for g8bpq, and not other ssids, you must specify g8bpq-0 in the config.

The code also contains the sysop functions from the standalone and data engine versions of the old bpqcode. This is basically an embedded version of sysoph. It reads the same password.bpq. Sysop mode is entered using the command PASSWORD. If you are on a local connection (eg using bpqterm), you get sysop mode immediately. if you are connected over a link, you get a prompt with 5 random numbers. You enter PASSWORD again with a parameter of the corresponding chars from password.bpq.

There is a new sysop command, to cause the axip driver to re-read its config file:

EXTRESTART P 1		Where P is the port number of the axip port.

Example if port 4 is te axip port:

EXTRESTART 4 1

I've tried most of the other sysop commands and they seem ok. But I suggest if you want to use any you try them on a test system first. I've changed the axip module to log config file errors to the bpqapp console window rather than using a message box - the latter caused the system to hang.