TERMINAL PROGRAM V1.20

Documentation - S.COLLINGS, G4SGI (APR. 91)

1. INTRODUCTION

1.1. Welcome to TERMINAL

Welcome to TERMINAL.  This is a host mode program for the AEA

PK-232MBX multimode modem.  Individual operating environments are

provided for all current modes (as available in January 1991) with

this software.  There are separate screens for Packet, Baudot (RTTY),

ASCII (RTTY), Morse, AMTOR, SIAM, NAVTEX, TDM and FAX (using the "Y"

cable") modes.  PakMail operation can be obtained using the Dumb

Terminal facility of the software.

TERMINAL may be used to provide output for LOGBOOK (a separate

program also written by G4SGI) that provides simple everyday logging

facilities using a text based window for input.  Automatic LOGBOOK

entries may be made with TERMINAL when transmitting in Morse, BAUDOT,

ASCII, Packet and AMTOR modes.  Information about a QSO may be entered

at the end of the contact with some of the details automatically

entered by the system.  The full LOGBOOK software can also be run from

within TERMINAL so that log entries can be maintained without having

to leave the main program.

Since this is a host mode program there is no need to remember

if the PK-232 is in command or converse modes.  This is all taken care

of by the software automatically.  Note that the Dumb Terminal mode of

TERMINAL operates using the command mode of the PK-232 so that

facilities not available in Host mode can still be obtained.

1.2. Minimum system requirements

TERMINAL has a number of minimum requirements in order to run

properly on an IBM PC compatible.  These are:

- IBM PC/XT/AT/386/486 compatible computer

- 512 kBytes of RAM (available for programs)

- Hercules/CGA/EGA/VGA text display

- Two floppy disk drives

OR

One floppy disk and a hard disk

- One serial port (COM1)

- AEA PK-232MBX (updated in 10/89 or later)

- Amateur Radio transceiver or general coverage receiver

- 80 column or 132 column text printer

1.3. Conventions used in this manual

Most changes to the operation of the modem can be made using

simple keyboard operations.  For example, when in SIAM mode it is

possible to issue the OK command by pressing <ALT-O>.  Here "<ALT-O>"

means hold down the alternative key and press the "O" key at the same

time.  All keystrokes are enclosed in angled brackets (eg <ESC>).

2. INSTALLING AND RUNNING TERMINAL

2.1 Copying the distribution disks

Whenever a new software package is to be used it is best to

make a copy of the distribution disks before trying to use it.

Copying the distribution disks on a dual floppy machine:

To make an exact copy of the distribution disks place the

TERMINAL distribution disk (disk 1) in disk drive B (with your DOS

disk in A) and type "DISKCOPY B: B:".  When prompted replace disk 1

with a blank floppy disk and press any key.  When copying is complete

label the disk in drive B "TERMINAL INSTALLATION DISK".

Repeat the procedure described above so that the LOGBOOK

distribution disk (disk 2) is copied and a "LOGBOOK INSTALLATION DISK"

is created.

Copying the distribution disks on a hard disk machine:

The distribution disks should be copied by placing the TERMINAL

distribution disk in disk drive A of your machine and typing "DISKCOPY

A: A:".  Replace the TERMINAL distribution disk with a blank floppy

disk when prompted.  When copying has finished remove the disk from

your machine and label it "TERMINAL INSTALLATION DISK".

Repeat the procedure described above so that the LOGBOOK

distribution disk (disk 2) is copied and a "LOGBOOK INSTALLATION DISK"

is created.

When you have successfully copied the distribution disks the

original disks should be stored in a safe place.  The complete

TERMINAL software package should be installed using the TERMINAL

INSTALLATION DISK and LOGBOOK INSTALLATION DISK that have just been

created.

There are two batch files (with the extension ".BAT") on the

installation disks.  "FDISK.BAT" can be used to install the software

onto floppy disks and "HDISK.BAT" can be used to install the software

onto a hard disk.  You should now choose which sort of installation is

needed and follow the corresponding instructions below.

2.2. Installing TERMINAL onto floppy disks

A total of three blank formatted floppy disks are needed to

install the software onto a machine with dual floppy disk drives.  Two

disks will be used to hold working copies of the programs and the

third will be used for LOGBOOK data.  These disks should be prepared

before proceeding with the installation.

Step 1:

Insert the TERMINAL INSTALLATION DISK into floppy disk drive A

and type "A:" to change to this drive.

Step 2:

Place a blank formatted disk into floppy disk drive B.

Step 3:

Type "FDISK" at the DOS prompt and follow any instructions that

appear on the screen.   Normally you should enter your callsign (or

receiving station number) and the model number of the radio you will

be using with the software.  This information is used to personalise

operation of the package.

Step 4:

When TERMINAL has been installed onto the first disk it should

be removed from the disk drive and labeled "TERMINAL WORKING DISK".

Place another blank floppy disk into drive B and press a key to

continue with the installation.

Step 5:

The second disk will contain the LOGBOOK software and should be

labeled "LOGBOOK WORKING DISK" when the software has been copied onto

it.

Step 6:

Label the last floppy disk "LOGBOOK DATA DISK".  Place the

"LOGBOOK WORKING DISK" into drive A and the "LOGBOOK DATA DISK" into

drive B.  Copy the prefix table file onto the "LOGBOOK DATA DISK" by

typing "TRANSFER A: B:" at the DOS prompt.

2.3. Installing TERMINAL onto a hard disk

A blank formatted floppy disk is needed in order to install the

software onto a machine with a hard disk.  This disk will be used to

hold LOGBOOK data.

Step 1:

Insert the TERMINAL INSTALLATION DISK into floppy disk drive A

and type "A:" to change to this drive.

Step 2:

Type "HDISK" at the DOS prompt and follow any instructions that

appear on the screen.  Normally your callsign (or receiving station

number) are used to personalise operation of the software.

CAUTION: the installation routine will create a directory

called "TERMINAL" in the root directory of your hard disk.  If there

is already a directory, or file in the root directory, of this name on

your disk then you should use a text editor (such as EDLIN) to modify

the batch file so that a different directory will be created.

Alternatively rename the file or directory that is causing the problem

if this will not cause problems with your existing software.

Step 3:

Swap distribution disk 1 with disk 2 when asked to do so and

press a key to continue with the installation routine.

Step 4:

Place the blank formatted floppy disk into disk drive A.  Copy

the prefix table file onto the new logbook disk by typing "TRANSFER C:

A:" at the DOS prompt.  The disk in drive B should be then labeled

"LOGBOOK DATA DISK".

2.4. Changing the installation

The installation of the TERMINAL software can be changed by

typing "TERMINAL/C <ENTER> at the DOS prompt instead of the usual

command to start the program (see below).  This causes the

installation routine to be run instead of the main program.

Normally the file "INSTALL.DAT" is read by TERMINAL when the

program starts.  This file contains information similar to that shown

below:

G4SGI                       - Your callsign

TS-440S                     - Your transceiver model number

COM1                        - Serial port used for communications

DO NOT DELETE THIS LINE

DO NOT DELETE THIS LINE

C:\TERMINAL                 - Directory for help information

A:                          - The drive used for the logbook data disk

100                         - Default power output (in watts)

WIDE                        - Type of print format

27, 109, 3                  - escape sequence for IBM characters.

DO NOT DELETE THIS LINE

Note:-

The lines containing the message "DO NOT DELETE THIS LINE" are

included in the file "INSTALL.DAT" so that it is compatible with the

normal LOGBOOK installation file.  In this way the same installation

information can be used to configure both LOGBOOK and TERMINAL.

This file may be edited with a text editor that does not add

extra formating characters to the file, or it may be edited using the

installation routine.

Your callsign and the model number of your transceiver appear

on the first two lines of the file.  This information is used to

personalise operation of TERMINAL and LOGBOOK.

The third line of the file indicates which serial port of the

PC will be used to communicate with the PK-232 modem.  It is possible

to use ports COM1 to COM4.

The next line of text that is used gives the DOS directory name

of the location on your computer system where help information used by

LOGBOOK is located.

The DOS name of the disk drive (and directory) where the

logbook files will be kept is on the next line of the file (drive A is

used in the example above).  Other disk drives or directories may also

be used.  Note that the directory names must not end in a "\"

character.

The next line contains the default transmitter power (in watts)

(in the example above the default power is 100 watts).

The next line indicates which kind of text printer will be

used, "NOPRINTER", "NARROW" and "WIDE" are valid printer types.

"NARROW" refers to an 80 column printer and "WIDE" refers to 132

columns.

The last line of information used by the system contains the

escape sequence needed to place your printer into a mode which allows

IBM extended characters to be printed.  These characters are used to

produce a box during QSL label printing.  The escape sequence should

be entered as decimal values separated by commas and spaces.  If your

printer does not produce correct printout and it is not possible to

use an escape sequence then try altering the DIL switch settings

inside the printer using the handbook provided with your printer.  If

this does not help then ordinary characters can be used to produce the

box by entering "NONE" on this line of the installation file (ie

"NONE" is no escape sequence).

If a text editor is being used to edit the installation file it

should not introduce any extra special control characters into the

file.

2.5. Running TERMINAL

When the TERMINAL software package has been correctly installed

the hardware should be carefully checked.  The PK-232 should be

connected to the serial port selected during the installation process

using the cable that was supplied with the modem.  The default serial

port is COM1 but any serial port between COM1 and COM4 may be used.

If you are using a PC with dual floppy disk drives place the

TERMINAL PROGRAM DISK in drive A and the LOGBOOK DATA DISK in drive B

before starting the software.  Change to drive A by typing "A:" at the

DOS prompt.

If you are using a PC with a hard disk then change to your hard

disk (normally drive C) by typing "C:" at the DOS prompt and place the

LOGBOOK DATA DISK in drive A before running the program.

TERMINAL can then be run by typing "TERMINAL <ENTER>" at the

DOS prompt.  If the hardware is connected to the PC correctly and the

software is correctly installed a banner should appear on the screen

and a message, indicating that the modem is being initialised, should

appear after a few seconds.

The contents of the file "INIT.CMD" will then be transferred to

the PK-232.  As this is happening each command will appear inside a

window displayed over the top of the banner.  If TERMINAL could not

find the PK-232 and restart it properly the program will terminate and

you should check the configuration of your system carefully.

TERMINAL communicates with the PK-232 at a fixed rate of 4800

bauds.  If battery backup is being used it is possible that TERMINAL

will not be able to initialise the PK-232 properly since another baud

rate may well be setup as the default.  The automatic baud rate

algorithm is used by TERMINAL to establish communications before

initialising the modem with the list of commands in the file

"INIT.CMD".  If the PK-232 fails to initialise then either remove

battery backup or change the baud rate of the PK-232 to 4800 baud

using a dumb terminal.

2.6. Selecting program options

All the modes of the software are displayed on the main screen

when TERMINAL has successfully initialised the PK-232.  A highlight

bar is always placed over one of the options.  This highlight bar can

be moved up or down using the <UP> or <DN> arrow keys, or sideways

using the <LFT> or <RGT> arrow keys.

When the required option is highlighted it can be selected by

pressing <ENTER>.  It is possible to exit from TERMINAL using the last

option.

3. FACILITIES COMMON TO ALL MODES OF TERMINAL

3.1. Help prompt

It is possible to display a prompt of the functions provided,

using the function keys, in any of the program's operating modes by

pressing <F1>.

3.2. Sending a text file

Any transmitting mode may be used to send a text file by

pressing <F2>.  Enter the name of the text file to be sent in the

window that appears in the bottom part of the screen and press <ENTER>

to begin sending.

Transmission of the text file can be stopped by pressing any

key.  The transmit buffer of the PK-232 can be cleared by entering the

TCLEAR command (using the facility described below) if required.

3.3. Entering a command

If one of the parameters of the modem needs to be changed and

the required function is not listed in the options at the bottom of

the screen then it can be entered as if the PK-232 is in command mode

by pressing <F3> and entering the command by hand.

For example, to change the monitor level in Packet operation:

press <F3> and enter "MONITOR 4" at the "cmd:" prompt to change the

monitor level to 4.  Other parameters may be changed in a similar way.

Please be very careful when using this facility since it is

possible to upset the operation of the modem and software very easily.

For instance, entering a command to change the mode of the PK-232

using this facility would have disastrous results.

3.4. Sending short text

This facility is very useful for sending 3x3 CQs or your

station details without having to type them in manually every time.

Short text can be obtained using two methods.  Press <F4> to

obtain a list of the texts that are available and select the required

one using a number key.  Alternatively, if a short text is used

regularly it may be more convenient to use one of the number keys

along the top of the keyboard prefixed with the ALT key.  For example,

to obtain short text number one press <ALT-1>.  Up to nine texts may

be used with each mode.

A file must be provided by the user containing the required

short texts.  This file is read by TERMINAL so that the required text

can be extracted and sent when required.

The file is named "mode.MSG" where "mode" is the name of the

operating mode.  At present "PACKET.MSG", "BAUDOT.MSG", "ASCII.MSG",

"MORSE.MSG" and "AMTOR.MSG" can be used with the system.  The user

must produce these files using a text editor in a special format.

The format comprises a text description that allows the user to

identify the contents of the short text, without having to read it,

then the text itself.  There can be up to nine texts numbered 1 to 9

and the software finds the required text by searching for a hash then

a number in the message file.

DO NOT INCLUDE ANY # CHARACTERS IN ANY OF THE SHORT TEXTS.

The format of a short text file is as shown below:

#n text description n

short text to be sent

#n+1 text description n+1

more short text to be sent

The file "EXAMPLE.MSG" is an example of short texts for BAUDOT

mode and may be edited by the user and copied to any of the short text

files named above.

Please note that the user must provide these files since the

text they contain will be different for each installation.  If no

short text files are available when operating the software then

nothing will happen when this option is selected.

3.5. RX and TX buffers

Two text buffers are provided with every mode of the software

except PACKET mode.  When text is received from a distant station the

characters are placed in an RX buffer so that they can be reviewed

using the <PGUP> and <PGDN> keys if the text has scrolled off the top

of the screen.  Press <HOME> to resume normal scrolling of the screen.

The RX buffer can be cleared by pressing <F5>.

A smaller buffer is provided for text to be sent to the distant

station.  The TX buffer can be cleared by pressing <F6>.

3.6. Printing

It is possible to have text copied to an optional printer as it

is received by pressing <F7>.  The printer should be connected to the

"PRN" or "LPT1" port of the PC.  Press <F7> again to stop printing

text.

3.7. Spooling to disk

Text may also be spooled to disk as it is received by pressing

<F8>.  Spooling can be stopped by pressing <F8> again.

3.8. Making LOGBOOK entries

TERMINAL allows LOGBOOK entries to be made while each operating

mode of the software is being used.

Time and date information is entered by the system

automatically in some cases so the PC system clock should be set to

read UTC to avoid the need to edit the times of operation in each

LOGBOOK entry.  The PC system time can be changed using an option of

the main menu.

A LOGBOOK entry can be started manually by pressing <F9> while

one of the PK-232 operating modes is being used.  This is useful for

QSOs made in speech while also using the PK-232.  The current LOGBOOK

entry can be edited at any time during a QSO using the <F10> key.

Whenever LOGBOOK entries are being edited a window (containing

the details of the QSO) will appear on the screen.  This is the same

window that is used with the standalone LOGBOOK program.  All LOGBOOK

operations within TERMINAL are completely compatible with the

standalone program.

3.8.1. Automatic logging in Packet and AMTOR modes

In Packet mode a LOGBOOK entry is started automatically when a

connection has been made with a distant station.  The entry can be

edited at any time during the connection and it may be completed, if

necessary, when the distant station has disconnected.

In AMTOR mode a LOGBOOK entry is started automatically whenever

an FEC transmission takes place.  The date and time of the

transmission are automatically entered by the system.  At the end of

the transmission you can complete the LOGBOOK entry.  TERMINAL assumes

that all FEC transmissions are CQ calls.

When an ARQ selcall is made TERMINAL assumes that a QSO is

about to start and prepares a LOGBOOK entry with the time and date of

the selcall.  When the PK-232 returns to standby operation the LOGBOOK

entry can be completed, if required.  The entry may be

edited at any time during the QSO so the details of the distant

station can be entered as they become apparent.

3.8.3. Manually entering LOGBOOK entries for BAUDOT, ASCII and Morse

modes

In BAUDOT, ASCII and Morse modes a LOGBOOK entry should be

started whenever a QSO begins and it may be completed when it ends.  A

LOGBOOK entry can be edited, during the QSO, as the details of the

distant station become known.

The date and times of the contact are entered automatically by

the system at the start of the QSO when the <F9> key is used to start

a LOGBOOK entry.  The LOGBOOK entry can be completed using the editing

facility (<F10>).

If you forget to start a LOGBOOK entry before starting to

transmit for the first time this will be done by the system

automatically.

3.9. Character codes available during demodulation of signals

It is possible to select various character codes during

reception of signals by pressing <ALT-C> to cycle through the range of

codes available with each mode.  The following characters codes are

available:

Morse mode: Continental, cyrillic, translitterated cyrillic,

katakana, translitterated katakana

BAUDOT, AMTOR and TDM modes: ITA #2, US teleprinter, cyrillic and

translitterated cyrillic

SIAM mode: ITA #2, US teleprinter, cyrillic, translitterated

cyrillic, katakana, translitterated katakana.

4. DUMB TERMINAL

This is a basic dumb terminal facility that places the PK-232

in conventional command mode.  Certain facilities such as CALIBRATE,

HELP and DISPLAY are not provided for in host mode.  However, they can

still be obtained using DUMB TERMINAL mode.

The PK-232 can be controlled just as if a simple text based

terminal were being used with the software in DUMB TERMINAL mode.  The

full range of commands may be used.

Press <ESC> to leave DUMB TERMINAL mode and return to the main

screen.

5. PACKET

Before operating in PACKET mode your callsign must be entered

into the PK-232's list of parameters in the file "INIT.CMD".  Use a

text editor to change the reference to MYCALL in this file if it is

incorrect.

5.1. Using channels and making a connection

Eleven logical communications channels are provided in PACKET

mode.  Channels 0 to 9 may be used to connect to distant stations and

channel 10 displays monitored frames only.  It is not possible to

connect to stations using channel 10.  The <UP> and <DN> arrow keys

may be used to change channels.

To connect to a distant station select one of the channels

numbered 0 to 9 and press <ALT-C>.  Enter the callsign of the station

and any routing information in the conventional way (eg. enter "G4SGI"

to connect with G4SGI directly, or "G4SGI VIA GB7XX-2" to connect via

a digipeater).

If a connection is successful then a status message indicating

that the connection has been made will be displayed on the VDU screen.

It is possible to disconnect from the station at the end of the QSO by

pressing <ALT-D>.

Multiple connections are possible using the <UP> and <DN> keys

to switch between channels on the fly.

5.2. Callsigns heard

It is possible to obtain a display of the MHEARD list by

pressing <ALT-M>.

5.3. Band of operation

The PACKET mode can be automatically configured for Priority

Acknowledgement operation on either VHF or HF bands.  Press <ALT-B> to

switch between a P-persistence system configured for 300 baud HF

operation and 1200 baud VHF operation.

6. BAUDOT (RTTY)

6.1. Switching between RX and TX

The transmitter can be switched on by pressing <ALT-T>.  Enter

the text of the QSO and press <ALT-T> again to end an over.  Text

files and short text messages can also be sent while transmitting.

6.2. Changing baud rate

The baud rate may be changed using the <ALT-+> and <ALT-->

keys.  Only those preset baud rates that are provided with the PK-232

can be obtained.

6.3. Miscellaneous parameters

It is possible to toggle between wide and narrow bandwidths by

pressing <ALT-W>.

Figures shift can be forced by pressing <ALT-N> (has the same

effect as entering NUMS) and unshift on space (USOS) can be selected

by pressing <ALT-U>.

The TX and RX tones can be inverted using <ALT-X> and <ALT-I>

respectively.

7. ASCII (RTTY)

All the parameters for this mode are the same as for BAUDOT

mode except that figures shift and USOS are not provided for.  There

is also no provision for changing the character code since this has no

meaning while operating in ASCII mode.

8. MORSE

8.1. Switching between TX and RX

The <ALT-T> key can be used to switch between TX and RX in the

same way as BAUDOT and ASCII modes.

8.2. Changing speed and LOCK

Use the <ALT-+> and <ALT--> to change the speed of morse

characters being sent.  The demodulation of incoming characters can be

locked using the <ALT-L> key.

9. AMTOR

Before operating in AMTOR mode make sure that the selcall

listed in the file "INIT.CMD" is correct for your station.

9.1. FEC (mode B)

The PK-232 can be placed into FEC transmit by pressing <ALT-F>.

Normally FEC is used for making CQ calls.

9.2. ARQ (mode A)

It is possible to change to ARQ mode by pressing <ALT-A>.  This

allows a QSO to be made once contact with a distant station has been

established.

When the end of a QSO has been reached contact with the distant

station can be broken using the <ALT-S> key so that the PK-232 can

return to standby.  The "+?" key sequence should be used to change the

distant station to an "Information Sending Station" in the normal way.

9.3. ARQ listen (ALIST)

ARQ transmissions can be monitored by pressing <ALT-L> (has the

same effect as the ALIST command).  FEC transmissions can be monitored

by placing the PK-232 in standby mode.

9.4. Miscellaneous parameters

Figure shift can be forced by pressing <ALT-N> (same as issuing

NUMS command).  The RFEC parameter can be toggled using the <ALT-R>

key and the TX and RX tones can be inverted using the <ALT-X> and

<ALT-I> keys (as with other modes).

10. SIAM

SIAM is very useful for identifying transmissions and

attempting to demodulate them.

10.1. WIDESHIFT

The setting of the WIDESHIFT parameter can be toggled using the

<ALT-W> key.

10.2. Switching between SIAM and demodulation

If a transmission has been correctly identified by the system

the PK-232 can be switched to the required mode (if it is available)

by pressing <ALT-O>.  This is the same as issuing the OK command in

command mode.

It is possible to return to SIAM mode by pressing <ALT-S> so

that analysis can continue.

11. NAVTEX

The schedules for NAVTEX transmission on 518 kHz can be

obtained from the publication "Guide to Utility Stations" by

Klingenfuss.  Most receivers should be tuned lower in frequency by

approximately 1.4 kHz so that the tones of the NAVTEX transmission

fall correctly within the passband of the PK-232's filters.

11.1. WIDESHIFT

The setting of WIDESHIFT is normally "WIDE OFF" for NAVTEX

operation on 518 kHz.  It may be toggled using the <ALT-W> key if

required.

11.2. NAVMSG and NAVSTN parameters

These parameters may be changed using the facility for entering

commands directly to the PK-232 (<F4> key).

12. TDM

12.1. Changing channel

The channel of a TDM transmission can be selected by pressing

<ALT-T> (same as TDCHAN command).  This command has the following

effects:

1 channel : TDCHAN has no effect

2 channel : TDCHAN 0 or 2 corresponds to channel A

TDCHAN 1 or 3 corresponds to channel B

4 channel : TDCHAN 0 channel A

TDCHAN 1 channel B

TDCHAN 2 channel C

TDCHAN 3 channel D

12.2. Changing baud rate

The baud rate can be selected from any of the preset rates

available, but only the following values will be accepted by the PK-

232:

1 channel:  48,  72,  96 baud

2 channel:  86,  96, 100 baud

4 channel: 171, 192, 200 baud

12.3. Synchronising

Manual synchronisation to weak TDM signals can be obtained

using the <ALT-R> key (same as R command).

12.4. Miscellaneous functions

WIDESHIFT can be toggled using the <ALT-W> key in the normal

way.  The received tones can be inverted using the <ALT-I> key.

Unshift on space can be toggled using <ALT-U>.

13. FAX (Using the "Y" cable)

FAX operation with TERMINAL uses a parallel printer connected

to the PK-232 directly using the "Y" cable provided with the unit.

13.1. Facilities available

The Index of Cooperation (IOC) and line speed (LPM) can be

selected using the <ALT-I> and <ALT-L> keys.  The direction and sense

of the image can be selected using the <ALT-D> and <ALT-N> keys.  The

image can be justified using the <ALT-J> key.

The PK-232 is capable of synchronising itself to FAX

transmissions and the status indicator will show if the modem has

succeeded in synchonising correctly.  Synchronisation can be forced by

pressing the <ALT-S> key.

14. LOGBOOK

14.1. Introduction to LOGBOOK

This is a separate program for the IBM PC that allows the

Amateur Radio enthusiast to record information about stations worked

on the amateur bands.  This program does not offer complex facilities

for contest logging but is intended to be a simple, easy to use,

computer based logbook for everyday contacts.

It is possible to store the date and times of activity, station

callsign, report, frequency and mode, operator's name, QTH, QSL

information and a 50 character comment in a disk based logbook.

The logbook can be searched for callsigns, prefixes, dates,

frequency bands, modes, names or QTHs.  It is also possible to print

QSL card labels and analyse the logbook for HF QSOs so that a country

prefix table is built up.

An example LOGBOOK screen with typical entries is shown below:

14:04:10      STATION LOGBOOK FOR G4SGI FROM 28/05/87 TO 04/02/90      04-FEB-900

���������������������������������������������������������������������������������

# Entries    372    # Entries free    23893   # Prefixes  UNCAL

Date      Start  Stop   Callsign           Freq Mode   RprtSnt RprtRec  Pwr

���������������������������������������������������������������������������������

28/01/90  13:45  13:46  CQ              29250.0  F2D   --      --       100

"      13:48  13:49  CQ              29250.0  F2D   --      --       100

"      13:58  13:58  CQ              29250.0  F2D   --      --       100

"      15:07  15:07  CLOSED DN       29250.0  F2D   --      --       100

04/02/90  10:47  10:48  EA6XQ-2         29250.0  F2D   --      --       100

"      10:52  10:53  OH2NJR-2        29250.0  F2D   --      --       100

"      10:53  10:58  OH2NJR-1        29250.0  F2D   --      --       100

"      11:02  11:04  EA1AU           29250.0  F2D   --      --       100

"      11:06  11:06  OH5RM           29250.0  F2D   --      --       100

"      11:15  11:27  G4MEM-2        144650.0  F2D   --      --         2

"      11:36  11:46  TR8CA-2         21097.8  J2D   --      --       100

"      11:46  11:48  TR8CA-2         21097.8  J2D   --      --       100

"      11:49  12:07  TR8CA-2         21097.8  J2D   --      --       100

"      12:07  12:18  TR8CA-2         21097.8  J2D   --      --       100

"      12:22  12:38  OH5TF                28  J2D   559     599      100

"      13:25  13:28  TR8CA-1         21097.8  J2D   --      --       100

>   "      13:28  13:28  CLOSED DN       21097.8  J2D   --      --       100   <

���������������������������������������������������������������������������������

F1=ADD     F2=EDIT    F3=DELETE     F4=INSERT  F5=QSL      F6=PRINT   F7=SEARCH

F8=CLEAR   F9=TABLE   F10=ANALYSE   TAB=SHIFT  ESC=ESCAPE  ALT-H=HELP

The date and times of activity, station callsign, report,

frequency and mode, operator's name, QTH, QSL information and a 50

character comment can all be stored in the disk based log.

14.2. IMPORTANT: Backing up the logbook data

The information stored in the logbook is very important and so

it is a good idea to make regular and frequent backups of the logbook

disk.

A batch process is provided that allows backups of the logbook

disk to be easily made.  The file "BACKLOG.BAT" contains DOS commands

for backing up a log disk.  To use this file place the disk to be

backed up into floppy disk drive A and have your backup disk ready.

Type in "BACKLOG" at the DOS command prompt and follow any

instructions that appear.

It is recommended that two backup disks be used to provide a

history of backups.  To do this use the "BACKLOG.BAT" file to make a

copy of the most recent backup disk, then backup the current working

disk, using the same batch process, onto the backup disk you have just

copied.  The flow of data is shown below (where step 1 is done first

and step 2 second):

Current       2      First      1       Second

Working  ----------> Backup ----------> Backup

Disk                 Disk               Disk

In this way the chances of recovering recent data if a disk

corruption should occur are considerably improved.

14.3. What happens when a logbook disk is full ?

LOGBOOK checks the amount of room on the logbook disk whenever

a new entry is added.  If the amount of free disk space gets below 4

kBytes a message appears warning you of this fact.  Eventually

logbook entries are rejected if there is not enough room on the disk

to save them.

The prefix table should be updated by pressing <F10> when the

"low disk space" message appears and you should exit from the logbook

program by pressing <ESC> so that a new logbook disk can be prepared.

Do not attempt to change disks without exiting from LOGBOOK first.

When a blank floppy disk has been formatted the prefix table

file, called "PREFIX.TBL", should be copied from the old disk onto

it.  This can be done using the "TRANSFER.BAT" routine.  To use this

batch file type in "TRANSFER A: B:" at the DOS prompt if you want to

transfer the prefix file from a disk in drive A to a new disk in

drive B.  Other disk drives may be used, if this is more convenient,

by substituting the required disk names in place of "A:" or "B:".

14.4. Using LOGBOOK's editing facilities

The basic editing functions of LOGBOOK are selected using the

function keys and are described below.  A prompt summarising the main

logbook operations is displayed on the bottom two lines of the

screen.

You can scroll the logbook entries up and down the screen

using the <Up> and <Dn> arrow keys.  Two indicators (">" and "<")

mark an entry that can be edited or deleted.

Frequencies are normally entered in kHz (eg to enter a QSO on

14.235 MHz "14235" should be entered).  It is also possible to enter

the frequency band if a full frequency is not required (eg to

indicate operation on the 144 MHz band just enter "144" for the

frequency).  When new entries are made the previously entered

frequency and mode are used so this information need not be re-

entered every time.

14.4.1. Making a logbook entry <F1>

When you want to enter details of a station into the logbook

press <F1>.  A logbook entry form will appear on the screen.  An

individual logbook item can be selected by moving the highlight bar,

using the <TAB> or <SHIFT-TAB> keys, to the required option and

pressing <ENTER>.

The data that can be entered into the logbook are date, start

time, stop time, callsign, power output, frequency (in kHz), mode,

report sent, report received, QSL sent, QSL received, name, QTH and

comment.

When all items are correct press <F10>, if you want to abort

press <ESC>.

14.4.2. Editing an entry <F2>

This allows you to make changes to logbook entries already

entered.  One entry on the screen has the symbols ">" and "<"

enclosing it.  Scroll the logbook display so that the required

logbook entry appears on this line using the <Up> and <Dn> arrow

keys.  When you press <F2> the logbook entry form will be displayed

and the selected entry will be copied into it.  You can change any of

the fields in the same way as when you first entered the information

(see <F1>).

The modified data can be saved by pressing <F10> or rejected

by pressing <ESC>.

14.4.3. Deleting an entry <F3>

Scroll the logbook display so that the entry that is to be

deleted is enclosed by the ">" and "<" symbols.  Press <F3> and the

logbook entry form will be used to display the full contents of the

entry as a verification before deletion takes place.

You can delete the entry, or leave it intact, by answering the

prompt that is displayed.

WARNING: It is not possible to "undelete" an entry so please be

careful when deleting entries.

14.4.4. Inserting an entry <F4>

This works on the same principle as the other options.  The

logbook entry form is used to enter details as before.

The new logbook entry is inserted immediately before the ">"

and "<" indicators.

Please note that you will not be able to insert a logbook

entry onto an empty logbook.  You should use the normal add facility

(<F1> key) to start a logbook.

When all fields in the entry form have been filled in

correctly press <F10>.  You can abort by pressing <ESC>.

14.4.5. Printing QSL card labels <F5>

LOGBOOK can be used to produce a QSL card label for individual

entries in the logbook file.  You will need 4" x 1 7/16" adhesive

labels for your printer in order to use this facility.  Suitable

labels are available from most computer supplies sources.

To print a QSL card label first select a logbook entry by

using the <Up> and <Dn> arrow keys to position the indicators, ">"

and "<", next to the logbook entry you want.  Press <F5> and check

that the print head of your printer is lined up with the upper edge

of a label before printing starts.

You can print more labels by moving the indicators to the

required entries and pressing <F5>.

14.4.6. Printing out a logbook file <F6>

By pressing <F6> you can print the current logbook file on the

optional printer.  If you specified that you had no printer when

installing the program then this option will not work.

If you are using an 80 column printer part of each log entry

will not be printed as there is not enough room on one line of the

paper for all the information.  It is possible to print 132 columns

of data on an 80 column printer by selecting "compressed" text on the

printer.  If you can select "compressed" text manually on your

printer then you will be able to print logbook files by first

installing LOGBOOK for a WIDE printer and then switching your printer

before starting each printout.

14.4.7.  Searching the logbook file <F7>

It is possible to search the logbook file for QSOs with a

particular callsign, or for more than one station with the same

prefix.  In addition you can search the logbook for QSOs on one

particular band, on one particular mode, or on a certain date.  QSOs

that have not had QSL cards sent, or QSOs for which no QSL has been

received may also be listed.  Facilities for searching the logbook

for particular operator names or QTHs are also provided.

Press <F7> and choose the type of search that you want by

pressing a numbers key.

A callsign can be searched for by entering the call letters in

full, alternatively only the prefix part of a callsign may be entered

so that stations with a particular prefix are listed.  For example,

to search for station G4SGI enter "G4SGI" or to search for callsigns

starting with G4 enter "G4".

Dates may be searched for in a similar way.  The format is

dd/mm/yy so that to search for QSOs on 1 January 1990 enter

"01/01/90".  Part of the date can be entered if more general searches

are required (eg to search for QSOs in May 89 enter "05/89").

QSOs on a particular frequency band can be searched for.  If

you want to list all 80m QSOs enter "3.5" for the 3.5MHz band.

Modes may be searched for by entering the required mode (eg

"USB" or "J2D").

An operator's name or QTH can be searched for by entering the

required text in a similar way.

14.5.  Analysing a logbook for worked prefixes

LOGBOOK allows a table of worked prefixes to be maintained for

HF QSOs.  It is possible to check to see if a prefix has been worked

before on a particular band, in the range 1.8MHz to 50MHz, and to

display the table on the computer screen as a "tally chart".

When a new prefix has been worked, and the corresponding QSO

details entered into the logbook, the logbook file should be analysed

so that the QSO can be added to the prefix table.

Only QSOs that have two signal reports entered are treated as

valid by LOGBOOK.  This prevents other entries such as TVI tests,

CQs, or incomplete QSOs from being processed.  A blank signal report

appears as a "--" in a report field - if anything else appears this

is assumed to be a report.

When an entry has been processed it is marked (invisibly to

the user) to prevent it from being re-processed.  If an entry is

edited (using the <F2> option) the marker is removed so that the

entry will be re-processed when the logbook is next analysed.

Callsign prefixes which are not known to LOGBOOK are left unmarked so

that the prefix table can be changed (if required) to include a new

prefix.

There is a file called "PREFIX.DAT" on the logbook

distribution disk which contains a list of the prefixes used in the

country prefix table.  This text file can be edited using a

wordprocessor (which does not introduce extra text formating

characters) so that the prefix list can be customised by the user.

If the prefix list is edited the format used in the file

should be kept the same.  This is so that the MAKETAB program can

convert the new prefix list into an empty prefix table file

correctly.  Only one prefix can be entered on each line of the file

"PREFIX.DAT".  There is a ruler line at the top of the file which

shows which columns can be used for each prefix and country name.

The logbook clearing facility can be used to mark all logbook

entries as unprocessed so that when a new prefix list is finished all

entries in a logbook file can be built into a new prefix table.

To rebuild the prefix table do the following:

(i) Edit the file "PREFIX.DAT" with a wordprocessor so that

unknown prefixes, or corrections to existing ones can be made.  The

wordprocessor (or text editor) should not add extra formatting

characters to the file.

(ii) Make sure that the modified "PREFIX.DAT" file is on the

same disk and directory the MAKETAB.EXE program.  When you have

exited from your wordprocessor run the MAKETAB program by typing

"MAKETAB<ENTER>" at the DOS prompt (do not include the " marks).

(iii) The MAKETAB program will generate a new prefix table

file, "PREFIX.TBL", which will be empty of logbook entries.

(iv) Use the TRANSFER routine to copy the new prefix table

file onto your logbook disk.

(v) Run LOGBOOK and select the CLEAR option by pressing <F8>.

(vi) The logbook data can then be re-analysed so that a new

prefix table file is built up using <F10>.

The prefix table command keys are described in more detail

below:

14.5.1.  Clearing the logbook <F8>

All logbook entries can be cleared so that they can be re-

analysed when producing a new prefix table.  Note that this function

does not delete data - it modifies the marker on each logbook entry.

A logbook file should not be cleared unless a new prefix table file

is being created from scratch.

14.5.2.  Displaying the country prefix table <F9>

The table of worked prefixes maintained by LOGBOOK can be

displayed by pressing <F9>.

When a station is worked on one of the bands between 1.8 MHz

and 50 MHz a tick mark is displayed, next to the country name, in

a column representing QSOs on that band.  This facility can be used

to see if you have worked a prefix on a particular band before.

The data in the table is most up to date when the logbook has

been analysed (see below).

14.5.3.  Analysing the logbook file <F10>

The callsign of each station worked is analysed for its prefix

and the prefix table is updated when <F10> is pressed.

Analysis of the logbook file allows the country prefix file to

be kept up to date and also updates the display of the total number

of prefixes worked on the screen.

Only QSOs that have two reports entered and which are on the

bands below 144MHz are processed.

If there are two, or more, possible prefixes for a station

then a list of the prefixes is displayed so that the correct one can

be chosen by the user.  If no prefix can be found for an entry then a

message saying that the callsign prefix is unknown will appear on the

screen and the entry will not be processed.

When the logbook has been fully analysed the "# Prefixes"

display on the logbook screen will be updated and the prefix table

file will contain the most up to date information.

Note that it is important that the logbook file is fully

analysed before starting a new logbook disk since the prefix file,

"PREFIX.TBL", will need to be transferred to the new disk.

14.6.  Shifting the display <TAB>

It is impossible to show all of the data for a logbook entry

on the computer screen at the same time.  A display scrolling

facility has been provided to help with viewing the data.

Press the <TAB> key to scroll the screen display to the left

and more data will appear from the right.  Keep pressing <TAB> until

the display returns to normal.

15. Changing the system time

The internal clock of the PC can be changed using this option

from the main screen.  Enter the time, using the format HH:MM:SS, and

press <ENTER>.

It is possible to abort by pressing <ESC>, or <ENTER> if

nothing else has been entered.  Normally the PC system clock should

be set to read UTC so that LOGBOOK times are entered correctly.

16. Print RX buffer

The entire contents of the RX buffer can be printed out using

this facility.  Make sure that a text printer is connected to PRN

(LPT1) and that it is on-line before using this option.

17. Spool RX buffer to disk

The RX buffer can also be spooled to disk using this option.

Enter the name of the disk file that is required and press <ENTER>.

18.  Final

Thank you for trying out the TERMINAL software package.  I

hope that you find it useful.  Please use the response form (the file

"RESPONSE") to pass your comments on the software back to me.

73 Simon, G4SGI