SCANNING PROGRAM for FRG-9600

Version 1.27

Copyright 1993,1994 DWCC

Date:		1 February 1994

Author:         RSM (IRC handle) Daniel, 9V1ZV

Internet:       daniel%pandora@lugs.po.my

-1.	Miscelanny

This program is dedicated to my patient and understanding wife who put

up with my late night and all day programming and hardware building and who

did not mind my buying the FRG-9600 MK5. I would also like to say thanks to all

who have helped by providing meaningful discussions on IRC #maxwell #warez and

#hack. All you out there, 73's and happy scanning.

If you want to write your own conversion utility, note that the first

43 bytes stay the same. After that starts the 1000*19 byte records. This is

then followed by 20*9 byte record followed by 1 byte. If you have any queries

or suggestions please e-mail to me at the above address.

I now realize that the command format for the FRG8800 is different from

the one used in the FRG9600 and therefore the program will NOT work with FRG-

8800 presently. If however there are enough users who want the FRG8800 version

I do not mind writing one for it. This depends entirely on the demand.

0.	Introduction

This program is freeware, freely used, freely copied, distributed

but not to be SOLD or commercialized without permission of the author. Other

than that, please have fun. It came about because the existing program I

saw, the MULTISCAN 1.6 was way too BIG, SLOW, and short on essential

features, hence I wrote this programme for my own use. I have some more

features I would like to add but at this time I just want to get this out of

the door. My FRG-9600 is extensively modified but is still compatible with

any unit out there I should think.

This is distributed with 4 files:-

SCANNER.COM	the TSR executable

SCANNER.DAT	a sample data file

SCANNER.DOC     this documentation

SCAN88.COM      XT (8088) version of the same program

EMI.TXT         text for dealing with RFI

FRG9600.MOD	mod for extending range of FRG-9600/965

If you want to reset everything, just delete the SCANNER.DAT and when

you start SCANNER.COM it will automaticaly create a fresh data file in the

directory where SCANNER.COM is installed. The data file must be in the same

directory as SCANNER.COM for it to be found. Note that the default is still

COM3 and if you do not specify anything the program will use COM3. Also if you

specify a 2-digit port, anything after the first digit is ignored so /C34 or

/C3A is the same as /C3. From version 1.17 I have added 2 more command line

switches. These are /Xyy,xx where yy is the y-offset of the pop-up box position

from the top and xx is the x-offset of the box position from the RIGHT of the

screen. Be forewarned that no limit checks are performed on these but that the

maximum acceptable value is 255. Try to keep these within reasonable limits or

I cannot guarantee the behavior of the program. The other switch I added is the

/Pnn switch where nn is any number less than 65536. This number determines the

pitch of the beep produced by the program. A greater number corresponds to a

lower pitch and the default pitch number is 200. As usual I have not tried

numbers 0 and 65535 so I cannot tell you what will happen when you do. This is

more for simple customizations.

I have now included in the distribution an XT (8088) compatible version

because I realized that while AT is predominant, many users still have only XTs

or would like to use it with their XT. This is presently untested because at

this point of time I do not have access to a convenient XT. Please channel all

bug reports to me. Note that the AT will be able to run both versions taking

into account the changed keys for F11 and F12 which are not available for the

XT class machines. The 8088 version is slightly larger though. Other than that

it uses the same data file.

I hope you enjoy this program and interface as I have. I spent 3-days

designing and building the interface and 3-days writing the assembly code and

debugging but a long time thinking about it before I actually started.

1.	Background

This programme is written completely in assembly so please inform

me if you find and bugs so that it can ease my debugging task. Also feel free

to suggest any new features that you would like to see in this program. The

minimum requirement to run this programme is theoretically a 286, and a fast

one too I should think, but the code is 286 compatible. However I use it on

a 486DX-36 and have not tested on other speeds, again let me know. The total

memory required is under 26k so any memory configuration should be okay. As

for the interface unit, the usual YAESU FIF/IF-232-C will NOT work properly

with this program because although the program can still control the radio,

there is no way for the program to detect a signal. Accompanied with this

document is a VERY simple and CHEAP serial interface that is compatible with

YAESU's, and costs a lot less while providing more features. Many parameters

in the program has not been made configurable but if there is any interest I

might look into it or if anyone would like to have a copy of the source code.

The actual code is only about 5-6k, the rest of the space is taken up by the

database entries.

2.	Features

This program provides 1000 memories which will store the frequency, the

reception mode, and a 13 byte description field. These are organized into 10

banks of 100 channels for bank scanning. Individual channels can be marked to

be skipped during memory scanning. The use of the program is patterned after

the original FRG-9600 usage but not exactly, however it should not be too hard

to figure out its usage. 2 VFO's are provided with individual limits and steps.

There is a choice of steps from 100 Hz to 10 mHz for easy tuning. Frequencies

can be keyed in directly or retrieved from memory.

Prior to version 1.14 I had forgotten to include serial port setting

option (ack!) and so it always defaulted to COM3 which is the one I used. It

then dawned upon me that others may have difficulty with this and so with

version 1.14 onwards you can use the command-line parameter /Cx where x is a

number from 1 to 4 to specify which port you want to use. So if you are using

COM4 you will issue a SCANNER.EXE /C4 and the program will then use COM4. Note

that as from version 1.18's major bugfix on this feature, the selected port is

saved along with the database so it needs to be specified only once if a save

is performed at anytime.

Scanning is provided in 3 speeds, as is manual tuning, and can be

configured to stop or pause on a signal, squelch release or a combination of

the two. This feature is only possible if my serial interface is used.

The most interesting feature of this programme that I like is the fact

that it is a TSR that can scan in the background. It actually traps the Int 08

and Int 09 for this purpose and does some reprogramming of the timer but should

be okay with normal DOS programs and QEMM. Once a scan is started the program

can be put in the background and it will continue scanning while you can run

other normal DOS programs without much problems. This is a tricky feature to

implement so I expect problems but so far it has worked okay for me. When a

signal is detected it will stop or pause and issue a short beep to get your

attention. At the time of writing I have not tried putting the program in

high memory but there should be no problem. Nevertheless, timing is very

critical for the proper behaviour of this program. I also am wary of QEMM's

stealth mode. Quarterdeck recommends that mode only when absolutely necessary.

The state of the SCANNER, including all memories, is saved upon exit

unless explicitly specified. The usual exit with Alt-X will save the state

while the Alt-Q will not save the state. A data file is created in the

directory scanner is run from under the name of SCANNER.DAT which is auto-

matically loaded and updated upon entry and exit respectively.

I have now tested loading-high the SCANNER and it works just fine.

3.	Future features and problems

A problem of this setup is the fact that the signal response is

rather slow and scanning in memory or in large steps may result in overshooting

the exact frequency, unfortunately I am not too keen to modify the hardware at

this time and changing the software would only mean slowing down the scan.

This can usually be remedied by using a slower scan rate. Alternatively, if

you are using dial/free scan, you can elect to use smaller tuning steps which

may reduce the severity of the overscan effect.

In use, the display flickers on every command received but this is

not the fault of my program but the problem of the FRG unit. Maybe YAESU can

help us with this one. Secondly, there may be some relay clicking inside

some units, again this is not the fault of the program but of the radio. I will

look into this sometime soon.

As of version 1.11, I will be looking to add a auto-logging for any

frequency encountered with activity. In case you run into some programs which

reset the clock and results in slowed or no response to frequency change

commands, try changing the rate because each rate change will reset the clock

to the appropriate value and reset the port to 4800 bauds which is required by

the FRG CAT interface.

As I have mentioned, the simplicity of this interface is due in part

to the fact that the CAT design on the FRG-9600 internally ties the S.I. (pin

3) to the positive 5v rail. This allows me to draw power for my transistor

through the S.I. pin therefore eliminating the need for an external power-

supply. This peculiarity may not be found on other YAESU radios however, and

in that case we are left with two possibilities, to either provide an external

power supply or to draw the power from the serial port. In order to use an

external supply, get a 5v supply and tie the 0v to the 0v (ground) of the

interface circuit, and tie the S.I. of the CAT (which is also the collector of

the transistor) to the the +5v through a 10k resistor. This should do the trick

and is the easiest way to fix the problem. You should try this only if there

absolutely no response on the side of the radio to any commands sent. Do this

only after eliminating all other possibilities (which aren't many). The other

way to fix this is to draw power from the serial port which requires the port

to be appropriately programmed. Unfortunately not all controller programs do

this, but the programming is easily done by setting any of the output pins on

the serial port to low (+5v) instead of the default high (-12v). For more

information check out programming or technical docs or e-mail me.

When using this interface with other controller programs, there are a

few things to consider. The first is that programs which control the port using

the BIOS to send the data will probably timeout because it expects the DSR and

CTS to be low (tied to a +12v supply through 10k resistors). I did not do this

because my program writes to the port directly and does not check these pins

since the CAT port does not support them. Some programs which do (eg. QBASIC)

will run into problems. MULTISCAN works fine however.

The second thing to note is that the feedback sections (parts 2 and 3)

are custom designed for use with my FRGSC1xx program and is probably not

supported by other controller programs so these need not be built. The behavior

of the SQUELCH unit is probably different because it is supposed to be the BUSY

line on other YAESU units. So there you are.

4.	Usage

Only one instance of SCANNER is allowed at any one time and a marker

is set at 0040:0015 to indicate an instance. If another program is installed

after SCANNER which alters either Int 08 or 09, then SCANNER will refuse to

exit and issue a beep if an exit is attempted. You may remove that program

first before SCANNER will exit gracefully. Once invoked, the program will

pop-up upon pressing Alt-x where x is the number corresponding to the COM port

for which the SCANNER is controlling. This when SCANNER is run it defaults to

COM3 so the hot-key is Alt-3. The hot-key is otherwise not alterable at this

time but if someone feels that it should be, then let me know. I just want to

keep the program small and simple though.

F1 will bring up a crude help which gives a summary of almost all the

commands. Crude is the word however, due to lack of display space. F5 and F6

will retrieve and save the present frequency, mode and step into a buffer for

retrieval. This buffer is automatically updated upon any change from DIAL to

MEMORY or vice versa, so F6 in effect will function as a Memory to Dial key if

pressed after an F5, that is an F5 followed by F6 is equals to a mem-to-dial

function. F5 is not the Dial to Memory however, it is a seldom used feature.

F2 now sorts the current memory bank according to increasing freq-

uency but affects only the current bank. The present channel will be retained

if relocated.

F4 will save the current state of the SCANNER to file. This is used to

update the file without needing to exit from scanner.

F3 toggles the beep/silent mode. This affects ALL beeps and its state

is show in the status bar by the presence of a "note" symbol for beep active.

F7 will only work when the current VFO is in MEMORY mode and will allow

the description field of the current channel to be edited. This is a 13 byte

field which will be displayed along with the channel. If a mistake is made or

if you decide to discard the entered data, hit ESC to revert to the previous

state. (Note: I have experience a problem here once but cannot repeat it.) As

of version 1.11 the left and right arrows along with the backspace key also work

while editing a description.

F8 toggles the VFO tracking action.

F9 toggles the current VFO between DIAL and MEMORY mode.

F10 cycles through the three speeds for tuning and scanning. The

three are indicated on the status bar.

F11 cycles through the scan-stop modes. S is for Signal, Q for sQuelch

and P for Pause. Any scanning can stop or pause upon signal and/or squelch

depending of which setting is selected. S-P- will pause on a signal while SQ--

will stop on either a signal or squelch release. A last minute addition is the

"H" option which when activated will stay with the channel until the signal is

lost. On the XT version, this is activated by shift-F9.

F12 cycles through the 6 reception modes of the FRG unit.

On the XT (8088) version of the program, this is replaced by the

shift-F10 key because XT's don't normally have entended keyboards.

Shift-F1 will display the list of skipped bands. The up and down arrows

will scroll through the list which contains 20 entrys. The space bar toggles

the state of the entry. A "+" in front of the entry indicates that the skip

is active. A blank indicates otherwise. The Escape key will exit this view mode.

The left most number is the entry number followed by the beginning of the skip

band and the ending frequencies.

PageUp and PageDown will initiate scanning up and down respectively.

The scanning will be a memory scan if the current VFO is in MEMORY mode or

a free scan if current VFO is in DIAL mode. In dial mode the scan is delimited

by the set limits while in a memory scan is limited to the current bank of

100 which will be searched for any valid memory which is not designated for

skipping. Scanning rate is governed by the tuning speed defined by F10.

Not that if there are no valid channels in the current bank, the SCANNER will

revert to DIAL mode. Scanning can be stopped by any keypress while the

SCANNER is popped-up and active. During scanning an arrown appears in the

status bar to indicate direction of scanning and if paused, a diamond appears.

"\" will work only in MEMORY mode and is used to toggle the status of

the current memory channel for either Normal, Skipped or Priority as indicated

by a space, an "x" and an "!" respectively. Priority channel scanning is now

implemented as of 1.14. A channel designated for skipping will not be checked

on memory scanning. A Prioritized channel will be checked every 4 scanned

channels and this function works only during scanning. Only one priority

channel is permitted per bank.

Any numeric key will start the numeric editor from which one can enter

a frequency directly or a channel. This behaviour is similar to the original

behaviour of the FRG unit so check out the manual. If a frequency is keyed,

then a CR (Return or Enter) will set the current VFO to DIAL mode at that

frequency if it is within the set limits and is valid. If a "[" or "]" is

pressed, then the lower and upper limit is set respectively. Limits may not

exceed the absolute minimum of 60 mHz and maximum of 950 mHz respectively or

they will be rejected. Also see that the low limit is below current upper limit

and vice versa. Each VFO maintains its own limits and tuning steps.

As of version 1.1 banks can be address by alphabet so you can type

J2 instead of 272. This applies in all modes but once an alphabet is keyed in

you cannot key in another alphabet, so AA is not possible. If you enter only

and alphabet, it will be interpreted as channel 0 of that bank, ie: A = A00.

The exception to this is the new auto channel assign. If you type control-enter

after keying in just an alphabet, the frequency will be stored in the first

blank spot in that bank.

If a channel is keyed (0 to 999) then an "M" will retrieve that channel

while a control-CR will store the current VFO to the keyed channel and the

delete key will delete the keyed channel. Entering a channel will auto-

matically set the VFO into MEMORY mode while deleting the current channel will

automatically revert the VFO to DIAL mode. For the auto-assign feature to work

you need to specify bank only, without a number. When in memory mode, pressing

the delete-key delete the current memory being displayed.

If a number between 0 and 19 is keyed and followed by a "+", the current

lower and upper limits (as set by "[" and "]") will then be taken to be a band

specification which is to be skipped during tuning and scanning. The skip limits

will then be stored in one of 20 skip memories as defined by the number that was

typed before the "+" was pressed. Thus if you pressed "12+", the current limits

will be stored in the 12th memory (displayable by pressing Shift-F1) and will

automatically be skipped during tuning. When any limits are stored, it will

default to active, this can however be toggled from the Shift-F1 viewing mode.

The defined skip bands are not effective during a memory scan, and works only

during a dial scan.

The right and left arrow will increase and decrease the current VFO

frequency by the set tuning step, or if it is in MEMORY mode, it will increase

or decrease the memory channel to the next occupied one. This is not bank

delimited. If the DIAL frequency hits the upper limit, it will issue a beep and

loop to the low limit and continue, the reverse is also true. The up and

down arrow will increase or decrease the tuning steps.

The tab key will toggle between VFOA and VFOB. Each VFO maintains its

own tuning step, mode and limits.

The signal will be red when present and green when absent. The squelch

is green when on and red when off. Thus the two not necessarily equal.

The ESC key will put the SCANNER into background while. If a scan has

been initiated, the scan will continue in the background. The Alt-X and Alt-Q

will exit the programme and remove it from memory if possible (see above). If

Alt-Q is used, all changes in current session are not saved! Alt-X is the usual

method for exiting.

A last minute addition means that now the F2 key will sort the current

memory bank if the current VFO is in MEMORY mode. Other banks than the current

are unaffected and the sort will be in the increasing order of frequencies.

5.	Serial Interface

The serial interface basically inverts the TXD from the serial port to

FRG CAT plug and converts the level. Since this is a step down, a low (5v)

is required for RS232C levels. Unfortunately, this +5v is not available on the

serial port, fortunately YAESU has internally tied the S.I. to the internal +5v

supply making it possible to use an open collector without need of external

power supply.

Pin 5 provides a +5v when no signal is present and about +3v when a

full signal is present. This is very sensitive to loads however so it must be

carefully used lest the voltage is not accurately representing the signal.

What I did was basically to implement a one-bit ADC convertor which takes its

power from the DTR and/or RTS lines. The squelch signal on pin 6 of the CAT

is actually documented as the BUSY line but it does not exhibit that behaviour.

Rather it seems to be at a 5 volts when the squelch is active. Again a one-bit

ADC is employed. The AGC signal from Pin 5 is fed through a voltage divider

with a variable resistor for manual level setting.

All the above can be implemented with 3 transistors, 2 zener diodes, a

few resistors and I have built it all on a small piece of Vero or matrix board

which fits snugly into the casing of a 9-pin D-plug for the serial port and so

with no external power supply, looks just like a normal cable.

This serial interface can use the same port concurrently with the

famous HAMCOMM interface without conflict so for users of HAMCOMM, you can use

the same port or build it together with the HAMCOMM circuit.

SERIAL LEVEL CONVERTOR

Pinout on                       o---------o S.I. (Pin 3 on CAT)

9-pin D-plug                    |

10k           b   / c

TXD (3)  o--/\/\/--o-----o---(|)  BC547 or similar NPN silicon transistor

|     |     \ e

IN4148 A     Z 10k  |

|     |      |

GND (5)  o---------o-----o------o---------o GND (Pin 1 on CAT)

DTR (4)  o---o          o-----------------o AGC (Pin 5 on CAT)

|          |

Z 4.7k*    A 2.4v zener (IN5221)

|          |

CD (1)  o---o          Z 10 k

c \   b     |

BC547   (|)-------o   SIGNAL ADC

e /         |

|          Z 4.7k

|          |

GND (5)  o---o----------o-----------------o GND (Pin 1 on CAT, see above)

* This can be made a 2.7k in series with a 20k potentiometer for variable

level. If this is fixed and a circuit does not work, try using another

value, usually a slightly bigger value but variable resistors are the

easiest to use but are not compact. In fact I really strongly recommend

the 20k variable resistor. If you wish to make it compact, use a sub-

miniature preset which comes with some length of knobs. These are usually

PCB mounted types, and the build the whole circuit into a 25-pin RS-232

plug, drilling a hole in the casing for the knob to come through. This

will be compact and yet have the advantage of being really flexible in

terms of signal detection. If you still find it difficult to cram parts

into that case, you can actually fit some parts into the 6-pin DIN plug

at the radio end of the cable. This is easy if you study the circuit and

then you will have more space.

DTR (4)  o----o

|          SQUELCH ADC (I need to confirm this one!)

Z 10 k                 (I haven't checked and may be wrong!)

RI (9)  o----o

c \             36k

BC547   (|)---o--------Z-----------o BUSY (Pin 6 on CAT, middle)

e /     |

|      Z 20k**

|      |

GND (5)  o----o------o--------------------o GND (Pin 1 on CAT, see above)

** This resistor is optional but if you are experiencing "stickiness" on the

squelch behaviour you may want to try including this.

The "Z" indicates a resistor and the "A" and "|>|" indicates the diode

in the up and right pointing directions respectively.

_______

/ c b e \   Bottom view of the BC547, flat side on top.

\       /

-------

All transistors can be BC547, BC107, BC108, BC109, BC550 or any junk-

box garden variety NPN silicon transistor but NOT germanium types (just in-case

you have some old ones). Preferable plastic cased ones because metal cased ones

like BC108 may short the collector when built in very tight enclosures. Other-

wise they work just as well. The zener can be any 1/4 watt or 1/2 watt or less

zener for 2.5 volts, IN5221 is just a sample. All resistors can be 1/4 watt or

1/8 watt ones at 5% accuracy since the currents involved are really low. Note

that in all three parts there is the GND connection, in practice only one needs

to be made (if you understand what I mean!). If you are really new to this, you

may not want to built the SQUELCH ADC and stick to using the SCANNER with stop

on signals only which work fine most of the time. If you are well versed, you

can understand what I am doing and maybe build a circuit with better (faster)

response. I just cooked this up from parts I had, didn't even calculate much!

If you are getting a 6-pin DIN plug for the CAT port, make sure you get a good

look at the port because there are a variety of 6-pin DIN plug. If you can, try

using ferrite beads on all the lines to minimize RFI and use screened cable.

I would appreciate a better circuit with better RFI rejection but for now this

works for me. As you can see, it is not a critical circuit and if you already

have a HAMCOMM circuit, you can take the DTR connections to the positive side

of the bridge circuit in the HAMCOMM circuit.

There isn't anything much to calibrate except for the variable

resistor in the signal detection circuit. The preset/potentiometer is used

to adjust the threshold level of the "convertor". Tune the radio to a

section where no signal is present but where there is an appreciable noise

floor (such as around 500 mHz). Then set the preset so that the signal

indicator just goes off. This should do fine. The squelch behaviour is dependent

upon the squelch setting on the radio so you may want to adjust that to see if

the circuit is working. Note that the nature of this circuit depends a great

deal on the particular type of serial card you are using. As such you may need

to experiment with the resistor values in order to get optimum behavior.

Do not use excessively long or poor quality cable for the CAT interface

because this will result in signal degradation which may cause spurious errors

in the CPU translation of the signal and result in random errors in the commands.

Please inform me if you have this problem.

I don't mind answering queries and helping out if your circuit fails

if you will mail me at the above address.

6.	References

The Waite Group's TURBO ASSEMBLER BIBLE,

Gary Syck, SAMS Publishing

PS-2/PC ASSEMBLY LANGUAGE,

Steven Holzner, Brady

THE PROGRAMMER'S TECHNICAL REFERENCE, MS-DOS IBM PC & Compatibles,

Dave Williams, John Wiley/Sigma Press

ASSEMBLY LANGUAGE PROGRAMMING for the Intel 80XXX Family,

William B. Giles, Maxwell Macmillan

Ralf Brown's Interrupt list 37

HAMCOMM 2.2 documentation, W. F. Schroeder

YAESU FRG-9600 manual and documentation.

YAESU IF-232-C circuit diagram.

YAESU FRG-9600/965 service manual.

7.	History

v1.00	First version released

v1.10	Added bank sorting and auto assign. Also fixed some bugs and

added extra scan stop mode. Added alpha bank references. A few

other bugs with the response of the system were also corrected.

v1.11	Fixed bug to allow use of numeric pad and capitals for data

entry. Also fixed sort bug to now sort whole bank. Previously

on about 19 entries were sorted. Banks now upgraded to 10 by

50 giving total of 500 channels storage. Previous absolute

limits changed from 60-950 mHz to 0-999.999.9 mHz to include HF

range for FRG-8800. Corrected a big bug in the description

field editor, now works properly with more editing functions.

This bug previously overwrote random code and resulted in

random hangs. I hope its gone away now.

v1.12	Minor bug fix for delete key in description editor.

v1.13	Memories expanded to 10 banks or 100 to give 1000 memory

channels now. Also included a conversion utility from the

scanner.dat for version 1.11+ (500 memories) to 1000 memory

scanner.dat.

v1.14	Corrected a bug with scanning down which caused the stop on

squelch open to be permanently enable. Now behaves correctly.

Included priority channels and serial port selection. Signal

identification has also been improved to reduce misses or

overshooting. Some overshooting may still occur but this is

due to the long "hang" of the FRG scanner's S-meter signal.

I can't help this one.

v1.15	Corrected bug is scanning stop mode, was previously wrongly

implemented and behaviour of pause with hold is now improved.

The pause begins at the end of signal when hold is activated.

v1.16	A minor change to allow steps of 9kHz for Europe and 30kHz for

cellular phone use.

v1.17	Added some command line switches to customize box position and

beep pitch. Also updated some parts of the documentation.

v1.18	Corrected a bug in the serial transmission data because in the

previous versions I used 1 stop bit when it should have been 2

stop bits. It still work due to the delay I inserted between

the sent characters made up for the missing stop bit. This now

allows me to send faster but I have only increased the speed

very minutely so as to maintain the accuracy of the scan stop.

I also noticed a major error in the commandline option to use

other COM ports. The error resulted in the inability to use any

port other than COM3 (sorry!) but it should be ok now. I have

also made some provision for the program to now load 4

instances for the purpose of controlling 4 COM ports. This is

highly unrecommended however and I would stick to one instance

running at any time.

v1.19	Added band skip feature. Now bands which are known to contain

nothing important can be locked out from scanning. Great to

skip unwanted portions. The scanner.dat is slightly bigger to

accomodate the new data but is still compatible with v1.18.

v1.19/88        This is the XT (8088) version of the above. F11 and

F12 have been replaced with shift-F9 and shift-F10. Some

minor code alterations.

v1.20	Previously the numeric keypad arrow and page keys were not

supported. Now they are. A text on dealing with RFI/EMI from

computers is now included. Deleting a channel is changed from

Control-Delete to simply Delete key after typing the channel.

This is necessary because non-extended keyboards cannot

generate the control-delete combination.

v1.21	The fringe behaviour of the limits and tabs has been changed for

more consistency. Now all the limits are inclusive so that you

can get ON the limits but not beyond. Some cosmetic changes has

been made to the XT version to provide better support for mono-

chrome displays. There were some major mistakes in the schematics

especially in the squelch and signal circuits. These have now

been corrected. Tab display screen now supports numeric keypad

arrow keys. Documentation corrections and additions. Previously

auto-assign was stated as typing "M", this should be control-

enter. Note that the data file is now slightly longer but is

still backward compatible with previous version.

v1.22	Bug fix with the behaviour of the memory delete key assignment.

The delete now used to delete memory (previously ^Del). The

help screen has been improved to be more "helpful"! Some code

optimization. A rudimentary VFO tracking function has been added

which is activated frm F8.

v1.23	Minor bug corrections, the squelch display was not working in the

right way. Now corrected.

v1.24	Some modification to the interface to have proper HamComm

compatibility. Note the change of power supply pin from 7 (RTS)

to now 4 (DTR). This is because HC has 7 tied to TX on PTT. Also

a bug fix to XT version to allow XT arrows to scroll help. A

10k resistor was added to the TX circuit to protect some serial

ports from sinking too much current (and burning out). Some

attribute problems in the monochrome code debugged.

v1.25	Slight design comments added to the interface circuit. Erroneous

priority channel scan behavior rectified.

v1.26	Improved hot-key checking. Now shift-alt-3 will NOT bring up the

pop-up screen.

v1.27	Still more bugs in the priority channel scanning fixed. Previously

if no priority channel was specified in a bank, it will simply

pick the last channel of some bank and scanned that as priority.

In the dial mode, a slight pause was incurred because it still

tried to check for priority channel. Now rectified both problems.

8.	Thanks

to Frank Paxton NG0N for his help in distributing this program

to buddies in #maxwell on IRC

to respondents to my message on alt.radio.scanner

to Pekka Lempola who has been especially helpful in testing the program

to W. F. Schroeder DL5YEC for advice on interface/HamComm compatibility