WINTEL a Windows Telemetry Decoding Program

Update 30/05/94

History.

I wanted to look at a single channel on KO23 (51 Rx2 Disc) to see why

it was deaf in the West-North.

To look at the telemetry I had to use TLMDC and SPLOT  etc etc

Whilst these are beaut programs, it was a cumbersome operation.

So I decided to write a simple program to decode one channel, then

put the data in EXCEL and look at it.

Now the program has grown out of all proportion!!!!

I still have a lot of work to do on it, but it seems to work mostly OK

and does what I wanted (except for live decoding -next!!!)

At the moment it only decodes the analogue channels - no status or

digital channels are decoded - use TLMDCII etc to do these.

The appearance is a bit plain at the moment for the purposes of debugging.

As yet I haven't written the HELP file so the following will have to do.

How it works.

It requires that WINTEL.CFG reside in the same directory as WINTEL.EXE

and that you have VBRUN300.DLL (usually in C:\Windows\System [390K]),

along with KITSAT.CFG and any other satellite .CFG files you require,

where ever WINTEL.CFG says they are - on that note the FIRST thing to do

is edit WINTEL.CFG to suite your setup.

When the program starts select which satellite you are going to decode,

so that WINTEL can get all the configurations (eventualy it will be automatic).

When the file selection appears - select a file to decode i.e. 94032205.KSS .

Select a channel to display ( all channels will be decoded).

Click on 'READ SPLOT'

If there is a SPLOT file, it will be read and the data displayed in

the list box. The % of file read will be displayed as its reading on

the 'READ RAW' button. At the end of reading you will get a beep.

You can then scroll the listbox, click on 'GRAPH' to look at a graph.

When you have the graph displayed, click on the graph and the data

at that X point will be displayed. Click below the graph to return.

The checkbox 'Don't clear graph' allows you to plot a channel on top

of another channel - try it with +X & -X Array Currents.

If you want a EXCEL tab delimited file generated, then click on the

'WRITE XLS' button. If the button is not enabled then there is a

EXCEL file already existing. Once again as the file is written the %

is displayed (always on the 'READ RAW' button).

If at the start when you clicked on 'READ SPLOT' and there was no SPLOT

file, then the 'READ RAW' button will be enabled.

Click on 'READ RAW' to read the KSS file and a SPLOT file will be written.

(SPLOT files are written with the TXT extension - if none existed).

You can copy the contents of the listbox & the graph screen to the system

ClipBoard - using the Menu at the top left.

You can also send the graph to a printer.

You do not need a KSS file if you already have a SPLOT file to read.

The EXCEL files take a while to write as I have grouped the channel

data to make it easy to use, that means the SPLOT file has to be read

once for each of the number of channels.

If you have EXCEL. then open the *.XLS file and use AUTOFORMAT to format

the file into EXCEL.

A KISS file *.KSS takes about 3 minutes for 800K to decode, hopefully

I can speed that up in the near future, the SPLOT file doesn't take long.

All file regardless of their extension are saved with the same name.

Ok have fun - any comments appreciated.

I purposely didn't use any of the Professional versions VBX controls etc.

SUPPORT FILES

WINTEL.EXE	the main program

WINTEL.CFG	the user config program MUST be in same directory as WINTEL

Special note - this is a new version, delete old Wintel.cfg

*VBRUN300.DLL	Visual Basic 3 library - put it with your other DLLs

usually in C:\WINDOWS\SYSTEM

WINTEL.DOC	Your reading it! - soon to be a help file.

*KITSAT.CFG	Config file for KO-23 put location in Wintel.cfg

*KITSAT2.CFG			KO-25

*WEBCOEF.CFG			WO-18 (these are modified to be like KITSAT.CFG)

*LUCOEF.CFG			LO-19   ditto

*PACCOEF.CFG			AO-16   ditto

*DOVCOEF.CFG			DO-17  I dont think this'll work

Note: Files marked * DO NOT CHANGE

regards

Geoff Radcliffe VK2ZAZ @ KO23  KO25

Sydney  N.S.W.  Australia

AMSAT Australia 256   AMSAT NA LM854  AMSAT UK 1268