TrakBox_Turbo version installation info.

========================================

Dallas semiconductor released 80C32 code compatible high speed CPU

DS80C320.  This chip is upward compatible with Intel 8032/8052 series

CPU and three times faster with the same crystal.  There are many other

additional functions (two serial ports, watchdog timer, many interrupt

sources, high X-tal clock etc..), and now, TrakBox can use this CPU.   The

calculation speedup is 4 times faster than the ordinary T_B.

The following is needed: (1) DS80C320 CPU,  (2) 22.118400MHz X-tal

(3) 320_xxx version EPROM

[Install]

1. At first, you must verify your TrakBox is working with original

EPROM and CPU. (8031,32,51,52 CMOS or NMOS).

2. Remove the battery back up jumper (JP13)

3. Replace the original CPU with DS80C320.

Check the T_B is running with new CPU.  The speed increase is 1.5 - 2

times faster than the original.

4. Replace the EPROM with Turbo EPROM (320_xxx.ROM).

Check the T_B to verify it runs with COM port baud rate of 4800.

5. Replace the original X-tal (11.0592MHz) with 22.1184MHz.

6. Now, your TrakBox_Turbo is completed.

[Notice]

1) The new CPU with 22Mhz clock rate needs faster (70-100ns) access time

EPROM and RAM. In my case, I used 120ns EPROM (Mitsubishi

M5M27C512K-12) and 80ns RAM (62256LP-8). If your T_B doesn't run,

you may need to use faster parts as outlined above.  EPROM and RAM

markings indicates the part number and the speed of the part. A part

with -12 means 120ns access time.

Many beta testers reported turbo code ran with fast EPROM and

and RAM from various suppliers.

2) Dallas Semiconductor released preliminary DS80C320 chips. The early

lot has bugs in the micro code.  The present version has YYWW-A6 on

the label.  If you have a xxxxA3 version, this  chip is not usable for

TrakBox Turbo. NOTE: YYWW means year/week the chip was manufactured.

[Dallas Semiconductor info.]

DS80C320    $12.00

DALLAS SEMICONDUCTOR

4401 South Beltwood Parkway

Dallas,  Texas  75244-3292

tel: (214)450-0448

Fax: (214)450-3715

overseas call: (214)450-5351

Credit card sales service: 1-800-336-6933

[Parts source information by VE2ALQ]

------------------------------------

For those people out here looking for components to upgrade TrakBox to

Turbo the following was (is) my source for every component needed:

JDR Electronics

Phone 408-494-1400

FAX   408-494-1420

North America Toll Free  Phone 1-800-538-5000

Fax   1-800-538-5005

Good luck folks this is where I got all of mine, no problem with single QTY

orders.  Have a Good One  -.-  -.

Darrell, VE1ALQ

-----------------

If you have any other source information, PSE let me know. I will include it

for those having difficulty finding these parts.

de Sueo,  JA6FTL