PART B

OPTIMIZATION

Sections 14 - 20

From here on we're going to assume that you know your way around the features

of SCALC described in Part A (Sections 2-13), and that you won't need quite the

same level of detailed assistance when introduced to SCALC's two remaining

Editor functions.

That leaves us free to concentrate on higher-level issues such as the

optimization process, and what you can achieve from it.

14.   ABOUT OPTIMIZATION

14.1  Introduction

SCALC is worth using for the sake of its circuit analysis features alone. Add

the optimization features - normally only encountered in software costing

thousands of dollars - and SCALC is unbeatable value!

Optimization is what liberates today's computer-aided engineers from the

restrictions of simple 'textbook' design examples. Starting from an existing

design, you can change and improve it to meet complicated real-life performance

criteria.

A simple performance-analysis program restricts you to doing optimization 'by

hand': you have to decide for yourself what changes to make, then you run the

program and see if the network performs any better. Quite possibly it doesn't,

and even if you have made some improvement it's anybody's guess whether the

network could perform better still.

The whole process goes so much better when the program includes an optimizer,

so the computer can do more of the work. The optimization features of the

original PD program ALMOND were the main reason why we chose to develop it into

SCALC.

14.2  Optimization overview

Computer-aided optimization is not a cure-all. It is a very effective addition

to your armory of design tools, but it won't do your thinking for you.

Speed and methodical persistence are two areas in which computers excel.

Computer optimization searches rapidly and methodically for improved

performance from your network. But optimization CANNOT design the original

network for you! SCALC's optimizer can only adjust the parameter values of

network elements that YOU have decided to use.

Also, you have to decide on your own definitions of 'better' and 'optimum'. For

example, do you want an amplifier which gives the maximum possible gain at one

frequency, or do you want a broadband design? If, as sometimes happens, you

want the maximum possible gain across a specified bandwidth, you have to define

exactly what you do want, in numerical terms that the computer can understand.

SCALC calls this 'setting objectives'.

We'll say more about setting objectives in a later section, but even now we'll

warn you that that if you aim too high, your objectives may not be attainable.

This is not a fault in SCALC: the circuit you have chosen is simply not capable

of delivering everything you wished for. You may be able to settle for a lesser

level of performance, and SCALC can certainly help to achieve the best possible

compromise design. If that isn't good enough, it's up to you - as an RF

engineer - to create a better network which has a greater potential for

optimization.

When you let SCALC optimize a network towards your objectives, it does so by

simply adjusting parameter values. However, some parameters will be more

important than the rest; in other words the network's performance will be more

sensitive to changes in these parameters. SCALC incorporates a very useful

feature called Sensitivity Analysis which allows you to concentrate on the most

important network elements first. The fewer the number of parameter values

being examined, the faster the optimizer runs. You can always come back to

'tweak' some of the less-sensitive parameters at a later stage.

Optimization is like climbing a mountain to reach the summit of performance.

SCALC approaches this problem by computing the steepest 'upward' slope and

adjusting all parameter values to move in that direction. This cycle repeats

until every possible move results in reduced performance, at which point

SCALC's optimizer reckons it has reached the summit, and stops. Unfortunately

this method is inherently short-sighted, like climbing a large mountain covered

in cloud. You may reach a point where any further move is downwards, but if you

can't see very far, how do you know that you're at the true summit? You might

only be at the top of one of the foothills.

SCALC offers you two alternatives. One is to adjust parameter values 'by hand'

and then re-start the optimizer. The more effective method is to allow SCALC to

make random changes in parameter values when it seems to be approaching an

optimum. This is equivalent to taking a random leap to some other point on the

mountain and then starting to climb again (WARNING - do not attempt this

procedure outdoors). If the optimizer still returns to the same set of

parameter values after making a number of random leaps, the chances are that

you have indeed reached the true summit, and have achieved the best possible

performance from that particular network configuration.

14.3  Steps in optimization

The next few sections explain the basic steps in network optimization using

SCALC.

1.      Setting objectives using the Objectives Editor (Section 15)

2.      Deciding which network elements to optimize, and more specifically which

parameters; this uses the Optimization Editor (Section 16) and the

Sensitivity Analysis feature (Section 17).

3.      Optimization (Section 18).

All of these sections are illustrated by examples, using files provided on disk.

15.   OBJECTIVES EDITOR

15.1  Overview

You enter the Objectives Editor either from the SCALC Main Menu or via the

short-cut from the Response Menu (Section 13) - in either case, press O.

�����������������������������������������������������������������������������Ŀ

Objectives Editor

Line number  1     Frequency:

�S11� dB           Weighting

�S22� dB           Weighting

�S21� dB           Weighting

FREQ     S11       S22       S21    �  FREQ     S11       S22       S21

in GHz  dB   WGT  dB   WGT  dB   WGT � in GHz  dB   WGT  dB   WGT  dB   WGT

I - Insert line      P - Print Objectives     F - Objectives file catalog

M - Modify line      S - Save file              - Line up     - Line down

D - Delete line      L - Load file            E - Optimization Editor

�������������������������������������������������������������������������������

Figure 45:  Objectives Menu

The layout of the Objectives Menu (Figure 45) is very similar to that of the

Network Editor (Figure 2). You can construct end edit a list of optimization

objectives using similar facilities to insert, modify and delete lines.

Optimization objectives can be saved to disk as files of type .OPT, and

recalled for further editing or re-use.

If you already know how to use the Network Editor, these Optimization Editor

functions will be self-explanatory; if you're having difficulties, perhaps you

should review Section 10.

What's new in the Optimization Editor is the layout and meaning of the

objectives themselves.

The network parameters available for optimization are S11, S22 and S21. The

present version of SCALC can only optimize these three S-parameters; it does

not optimize S12 since this is seldom required, and in any case can it always

be achieved by turning the whole network around using the IMA command and

optimizing S21 for the mirror-imaged network. Also, the present version of

SCALC cannot handle stability- or noise-related optimizations.

Each line in an Objectives file sets objectives for S11, S22 and S21 at a

particular frequency. The format allows you to specify objectives either as

minimum performance targets of gain or return loss, or as a target to be

achieved precisely (gain only, since a network hardly ever needs to be

optimized to exhibit a specific return loss).

Each individual objective is given a 'weighting' to indicate how important you

consider it to be, relative to the other objectives. The scale is purely

arbitrary, so typically you might assign a weighting of 10 to an objective of

'ordinary' importance and perhaps 20 to bias the optimization more towards

achieving a more important objective. Weightings are very much a matter of

experience, and only become significant in cases where you're having difficulty

in achieving all your desired performance objectives. A weighting of zero

implies 'don't care', and should be used wherever possible to free-up the

optimization.

An example would help at this point.

15.2  A simple example

Suppose we wish to improve the performance of the 1GHz lowpass filter used as

an example in Section 3.

(Section 3 showed you how to load the file LPF.NET using the Network Editor: at

the Main Menu press E to enter the Network Editor; press L and enter LPF at the

filename prompt; then press Esc to check the network and return to the Main

Menu. Now press O to enter the Objectives Editor, and we're ready to continue.)

Let's say that we are looking for at least 20dB of stopband attenuation from

1.4GHz upwards, while at the same time improving the passband return loss to

10dB or better at 0.9GHz and below. To achieve this without making the filter

more complex, we will have to accept some passband ripple; let's say we can

accept �0.1dB.

To translate these verbal objectives into something that SCALC can work with,

we need to specify certain maximum or minimum values of the three available S-

parameters at particular frequencies. To specify 'flatness' or broadband

performance, we need to repeat the same S-parameter specifications at a number

of frequencies across the desired band (OK, so maybe the performance could run

wild between these spot frequencies, but in real-life networks that hardly ever

happens). In this case, frequencies of 0.3, 0.6 and 0.9GHz should suffice to

define the passband response. Above the cutoff frequency, we already know that

the response of this simple lowpass filter will roll off in a straightforward

manner, so all we need is to specify the desired performance at 1.4GHz and at

one more frequency for good measure; let's say 1.7GHz.

Using the Objectives Editor we construct the following list, which is also

supplied on disk as LPF.OPT:

�����������������������������������������������������������������������������Ŀ

Objectives Editor

Line number  6     Frequency:

�S11� dB           Weighting

�S22� dB           Weighting

�S21� dB           Weighting

FREQ     S11       S22       S21    �  FREQ     S11       S22       S21

in GHz  dB   WGT  dB   WGT  dB   WGT � in GHz  dB   WGT  dB   WGT  dB   WGT

.3    -10   10  -10   10  -.1   10

.6    -10   10  -10   10  -.1   10

.9    -10   10  -10   10  -.1   10

1.4    0    0    0    0   -20  -10

1.7    0    0    0    0   -20  -10

�������������������������������������������������������������������������������

Figure 46:  Optimization objectives for LPF.NET

Now what does all that mean? Let's look at the passband and stopband

separately.

All three lines specifying the passband (0.3, 0.6 and 0.9GHz) are identical;

as noted earlier, this is how we specify 'flatness' or broadband performance.

All the parameters have an equal weighting of 10 (the WGT column). The S11 and

S22 parameters are negative, as return losses always must be in S-parameter

notation, and SCALC insists on this. The value of -10dB indicates that SCALC

should try to achieve this return loss as a minimum, but we don't care by how

much or how little it can be bettered.

The passband ripple target of �0.1dB is expressed by specifying the S21

objective as -0.1dB with an unsigned weighting. Taken literally, this only

sets a target for the peak negative value of the passband ripple, but the

characteristics of a lowpass filter response strongly imply that the peak

positive excursion will also be very close to 0.1dB.

In the stopband, specified at 1.4 and 1.7GHz, we don't care about either S11 or

S22 so they are each given a weighting of zero; in this specific case, the

entries in the S11 and S22 columns become irrelevant - so why not say zero. To

tell SCALC that -20dB is a minimum target for S21, we place a minus-sign ahead

of the weighting (see next section for explanation). Without that minus-sign,

SCALC would try to optimize the network for an S21 value of exactly 20dB.

15.3  Syntax for objectives

The following table summarizes the valid combinations of parameter values and

weightings for optimization objectives.

Parameter    Value        Weighting     Meaning of weighting

S11          Always �0    Unsigned      Not valid (interpreted as negative)

0             Don't care about S11

Minus         Worst acceptable return loss

S22 - as S11

S21          Any          Unsigned      Specific gain/loss target

0             Don't care about S11

Minus         Maximum acceptable S11

A minus-sign used ahead of a weighting is a logical 'flag' meaning that the S-

parameter value to the left is a limit, not a specific target. (The minus-sign

does NOT imply that the weighting is in any sense 'negative'.)

As noted above, S11 and S22 can only be optimized for return LOSS, i.e.

negative values of S11 and S22. S11 or S22 cannot be optimized to a specific

value - SCALC always assumes you will accept a better match than the minimum

standard specified, if the network can provide it.

In practice, if a network has insertion gain you generally either require a

specific value, or else you don't care about gain. The unsigned and zero

weighting options meet these two needs.

If a network has insertion loss (negative S21), you generally either require a

specific value, or the value is the minimum acceptable attenuation; or else you

don't care. The unsigned weighting option will optimize towards a specific

value, while zero indicates 'don't care'. The minus-signed weighting for

insertion loss S21 implies that you'd accept more attenuation if the network

can provide it.

(The remaining option is a minus-signed weighting for a network with positive

S21. This will optimize towards a maximum acceptable value of insertion gain,

which is not very useful in practice.)

15.4  Choice of frequencies

As stated earlier, a complete set of S11, S22 and S21 objectives and weightings

is required for each frequency. These are entered in tabular form at the top

left of the screen and when the table is complete they are transferred to a

single line in the objectives list at the centre of the screen. You can specify

up to 20 frequencies, but the speed of optimization decreases if you specify

more frequencies than absolutely necessary.

CAUTION

The frequencies that you set for use with objectives will overwrite whatever

frequencies you might have set earlier for analysis purposes (Section 13).

TIP

If you have a powerful PC and a fairly simple optimization problem, you can

specify the same frequencies for optimization as you will wish to use for

performance analysis after optimization. You can then flick back and forth

between analysis and optimization using the same set of frequencies, and the

optimized network will be waiting for you in the Network Editor.

15.5  Leaving the Objectives Editor

The only exit from the Objectives Editor is via the Optimization Editor - press

E.

From the Optimization Editor you can return to the Main Menu by pressing Esc.

15.6   Another example

Here is another example of a very simple objectives list - only one line, in

fact. Figure 47 shows a harmonic filter designed to suppress the second and

third harmonics of a 1GHz signal. This is achieved by parallel-connected 90�

open-circuit stubs resonant at 2GHz and 3GHz. A second pair of stubs are placed

a certain distance further down the series transmission line.

� OST, 90� at 2GHz �

�                  �

�                  �

�        TRL       �

in -��������������������- out

�                  �

�                  �

� OST, 90� at 3GHz �

Figure 47:  Stub filter to suppress 2nd and 3rd harmonics of 1.0GHz

Obviously the quarter-wave stubs will produce the required notches in the

passband at 2GHz and 3GHz, so the 90� lengths require no further optimization.

But will the return loss at 1GHz be acceptable? If you analyse this network

(supplied on disk as 2F3F_LPF.NET) you'll find that the answer is NO!

Fortunately the return loss of this type of filter can be optimized by

adjusting the length of the series transmission line, and the objectives list

is extremely simple because it involves only one frequency.

This one-line objectives list is supplied on disk as 2F3F_LPF.OPT -

FREQ     S11       S22       S21

in GHz  dB   WGT  dB   WGT  dB   WGT

1      0    10   0    10   0    10

These objectives mean that the reflection parameters S11 and S22 are to be

optimized to -40dB or better. Engineering intuition tells us that optimum

matching at the input and output ports (by making S11 and S22 large and

negative) will automatically optimize the transmission parameter S21 as well,

so we can speed the optimization by setting the S21 weighting to zero.

Whenever you wish to optimize towards a perfect impedance match, -40dB is a

reasonable objective to set for S11 and/or S22. Return losses better than -40dB

are pretty meaningless, either in computational terms or in physical reality;

if you set a tighter objective such as -50dB it's merely 'playing with

numbers'.

TIP

It's worthwhile to spend some time constructing a library of basic objectives

lists suitable for your applications, because you can save them as reloadable

files which you can then edit and add to your library.

Section 16 will carry out a very simple optimization, and Sections 17 to 20

will deal with more advanced topics.

16.   A SIMPLE OPTIMIZATION

The Optimization Editor allows you to select which network elements SCALC is

allowed to alter during the course of optimization, and more specifically which

parameter values can be changed.

When the Optimization Editor lists the network, elements such as CAX and IMA

which do not correspond to real-life electronic components are shown in dim

grey because they cannot be altered in an optimization. From the remaining

elements listed at normal brightness, you can then select which elements are to

be altered, and more specifically which of their parameters.

As a very simple example, let's look further at the 2F3F notch filter.

(From the SCALC Main Menu, press E to enter the Network Editor; press L and

load the file 2F3F_LPF. Press Esc to return to the Main Menu, press O to enter

the Objectives Editor and press L to load the objectives file 2F3F_LPF. Then

press E to move on to the Optimization Editor and you'll see the following

screen.)

�����������������������������������������������������������������������������Ŀ

ELEMENT    PARAMETER      LOCAL                        Optimization Editor

type  num  num   value    GRADIENT

Element No.   #1       #2       #3    � Element No.   #1       #2       #3

OST   1    50�      90�lng   2GHz

OST   2    50�      90�lng   3GHz

TRL   3    50�      90�lng   1GHz

EQU   4    1el#

EQU   5    2el#

CAX   6    1el#     5el#

END   6

I - Insert line      P - Print opt. elements        E - Objectives Editor

M - Modify line      O - Optimize

D - Delete line      S - Sensitivity analysis       Esc - Main Menu

�������������������������������������������������������������������������������

Figure 48:  Optimization Editor screen for the 2F3F_LPF example

Note that all the transmission lines are highlighted (on the screen; not

possible in the disk-based manual), which means that any of their parameters

could be altered during optimization. Although SCALC imposes a limit of 6

parameters in total, only one parameter is relevant to this particular example

problem: the length of the series transmission line - TRL, element 3. We don't

want to change the open-circuit stubs because they are already the correct

lengths to provide notches at 2GHz and 3GHz.

To select the desired parameter to optimize, press I to insert a line, and

enter 3 to select TRL. Then enter 2 to select TRL's second parameter, length.

This has been set initially to 90 but is obviously incorrect.

Having selected the desired parameter, we can go straight to the optimization.

SCALC will adjust the length of the transmission line until S11 and S22 have

been optimized, and in this particular case there does exist a solution which

gives a perfect impedance match.

Press O to optimize, and skip the next two messages by pressing Enter twice

(we'll come back to those later). The optimization then begins; just sit back

and watch until the progress messages disappear, and the job's completely done!

The upper part of the display has changed to:

ELEMENT    PARAMETER      LOCAL

type  num  num   value    GRADIENT

1  TRL   3    2   51.877 lng  0

In other words, shortening the series transmission line from 90� to 51.9� gives

the required impedance match. (Actually, 90� wasn't a totally wild initial

guess; if you eliminate the 3GHz stubs, you'll find that the remaining network

gives a notch at 2GHz and a perfect impedance match at 1GHz.)

Notice that the final value of the local gradient is zero. This indicates that

we're at the `top of the mountain' and SCALC has found a perfect solution with

no compromises. We'll return to this topic in Section 19.

Press Esc to return to the Main Menu and then press A for a quick spot-

frequency analysis at 1GHz, which was the single frequency for which the

network was optimized. The impedance match is near-perfect, limited only by the

numerical accuracy of computation, and as expected the elimination of mismatch

losses has brought S21 up to its desired value of 0dB. You can then re-analyse

the network over a range of frequencies to obtain more complete performance

data.

17.   SENSITIVITY ANALYSIS

As mentioned earlier, some network parameters are more important than others in

determining performance. Since the time required for an optimization depends

strongly on the number of parameters being varied, it's very useful to be able

to eliminate parameters that are of little relevance. Also, the present version

of SCALC only allows a maximum of 6 parameters to be varied at the same time.

When you have used the Optimization Editor (Section 16) to identify up to 6

parameters, you can use the Sensitivity Analysis feature to determine how

strongly they are likely to affect the performance of the network.

As an example, re-load the 1GHz lowpass filter example. (From the SCALC Main

Menu, press E to enter the Network Editor; press L and load the file LPF. Press

Esc to return to the Main Menu, press O to enter the Objectives Editor and

press L to load the objectives file LPF. Then press E to move on to the

Optimization Editor and you'll see the following screen.)

�����������������������������������������������������������������������������Ŀ

ELEMENT    PARAMETER      LOCAL                        Optimization Editor

type  num  num   value    GRADIENT

Element No.   #1       #2       #3    � Element No.   #1       #2       #3

CPP   1    2.2pF

INS   2    10nH

CPP   3    4.7pF

INS   4    10nH

CPP   5    2.2pF

CAX   6    1el#     5el#

END   6

I - Insert line      P - Print opt. elements        E - Objectives Editor

M - Modify line      O - Optimize

D - Delete line      S - Sensitivity analysis       Esc - Main Menu

�������������������������������������������������������������������������������

Figure 49:  Optimization Editor listing for the 1GHz lowpass filter

The network will be listed in the centre of the screen with the non-optimizable

elements such as CAX and IMA greyed-out. The remaining network parameters are

all available for optimization, so you have to choose the ones you wish to work

with. in this example, we will select the three capacitor values as variables

for optimization.

To include element 1, CPP, in the list, press I (insert a line) and enter 1,

the element number. In response to the Parameter # prompt, enter 1 again - CPP

only has one parameter. Repeat the procedure for elements 2 (INS), 3 (CPP) and

5 (CPP). The upper part of the screen should now look like this:

ELEMENT    PARAMETER      LOCAL

type  num  num   value    GRADIENT

1  CPP   1    1   2.2pF

2  CPP   3    1   4.7pF

3  CPP   5    1   2.2pF

Figure 50:  Optimization parameter list for the 1GHz lowpass filter

If you make a mistake, you can press D to delete a line and then I to enter it

again.

NOTE

With a network element that has more than one parameter, you also have

the option to press M - modify a line. SCALC will ask you which of its

parameters you wish to modify.

If you wish to optimize more than one parameter for the same network

element, you'll have to press I first to insert another line. Remember

that the maximum limit of 6 applies to the total number of parameters,

not the number of different elements.

Now do a sensitivity analysis to determine the relevance of each parameter to

meeting the performance objectives for the 1GHz lowpass filter. Just press S,

and after a few moments, numbers will appear in the LOCAL GRADIENT column:

ELEMENT    PARAMETER      LOCAL

type  num  num   value    GRADIENT

1  CPP   1    1   2.2pF      -58.46

2  CPP   3    1   4.7pF      -293.67

3  CPP   5    1   2.2pF      -58.4

Figure 51:  Sensitivity analysis for the 1GHz lowpass filter

The term 'gradient' comes from our hill-climbing analogy for the optimization

process. Changing a parameter which has a large gradient will take us 'uphill'

faster than changing one with a smaller gradient. A negative local gradient is

not important; it merely means that the performance will be moved 'uphill' by

making a negative change in the parameter value.

The values of gradients vary considerably between different networks. What's

important is the relative magnitude of the gradient of each parameter, compared

with the gradients of the others in the optimization list. In this example

element 2, parameter 1 has the highest gradient, followed by elements 1 and 3

(the network is symmetrical).

In some cases we might wish to eliminate a parameter from the optimization list

because sensitivity analysis has shown its gradient to be small (press D to

delete a line, and enter the line number). If your network has more than the

limit of 6 parameters requiring optimization, your strategy should be to

concentrate first on the parameters showing the highest gradients. After these

major parameters have been substantially optimized, you can bring the less-

sensitive parameter back into the optimization list. The Sensitivity Analysis

feature will show you whether any parameter may require a final 'tweak'.

In this example we will keep all three parameters in the optimization,

described in the next section.

18.   OPTIMIZATION PROCEDURES

So far we have identified which parameters are to be optimized, and have

analysed the network's sensitivity to their parameter values. Next is the

optimization itself.

Press O to begin - but before the calculations start, there are two further

choices to make. The first is:

Print results [y/N/Esc] >

This requires a single-keypress answer. The default is No (indicated by the

capital N), so you would need to press either Y or y in order to obtain a

printout. You also have the option to press Esc to abandon the process. The

main value of a printout is that it creates a log of SCALC's searches, which

may be useful if optimization is proving difficult - but be warned that this

option can use a lot of paper!

The next choice is:

No. of random searches [0-9/Esc] >

This refers to the facility of making random jumps in parameter values, to

ensure that SCALC has not optimized to a local maximum (see Section 14.1). Once

again this requires a single keypress. The default is zero (no random searches)

but if you press any key in the range 1-9, that number of random searches will

be made. Once again there is the additional option of pressing Esc to abandon

the optimization.

TIP

Normally you will not want printout, and will not want to make any random

searches. The defaults for these two options are set up so that you can simply

press Enter two times to skip them.

The optimization begins automatically. According to the speed of your PC, you

will see a flow of messages at the top-right of the screen, and the parameter

values rapidly change in the optimization list.

TIP

To pause or stop the optimization, press Q. You can continue by pressing O, but

if you wish to start again from the original parameter values, you must re-load

the network.

As optimization continues, the local gradient values will gradually diminish

until finally the optimization finishes with the following results:

ELEMENT    PARAMETER      LOCAL

type  num  num   value    GRADIENT

1  CPP   1    1   4.243pF    -.38

2  CPP   3    1   8.703pF    -.18

3  CPP   5    1   4.243pF    -.38

Figure 52:  The optimized 1GHz lowpass filter

To analyse the performance of the new network, press Esc to return to the Main

Menu, then press A. SCALC analyses the network at the frequencies specified in

the objectives file: 0.3, 0.6, 0.9, 1.4 and 1.7GHz. Here are the results using

option 4 of the Response menu (list S-parameters in dB).

Frequency        S11            S21           S12          S22

in GHz         in dB          in dB         in dB        in dB

0.3           -10.4         -.42          -.42          -10.4

0.6           -16.07        -.11          -.11          -16.07

0.9           -15.44        -.13          -.13          -15.44

1.4           -.04          -19.94        -19.94        -.04

1.7            0            -30.65        -30.65         0

Figure 53:  Performance analysis of optimized 1GHz lowpass filter

Recalling Figure 46, the objectives were a minimum of 10dB return loss in the

passband below 1GHz (S11 and S22), with no more than 0.1dB insertion loss

(S21); and at least 20dB attenuation (S21) at the two stopband frequencies

above 1GHz. The passband return loss targets have been reached, and the

insertion losses are close to target at 0.6 and 0.9GHz, with some increase at

0.3GHz. The stopband attenuation is very close to target at 1.3GHz and, as

you'd expect, it is much better at 1.7GHz.

You can analyse the filter in more detail by choosing a new range of

frequencies, as explained in Section 3 (in Part A of this manual).

How acceptable should you consider these results? That is the subject of the

next section.

19.   FINDING THE OPTIMUM COMPROMISE

19.1  Two kinds of problem

The 2F3F stub filter (Section 16) and the 1GHz lowpass filter (Sections 17-18)

were chosen to show the difference between two distinct kinds of optimization

problem.

The 2F3F stub filter case has a definite answer, which you could have worked

out using a Smith chart for example. You're only using SCALC's optimizer to

save yourself time. The final value of the local gradient was zero, which

indicates that SCALC has climbed to the `top of the mountain' and has found a

perfect solution with no compromises - as expected.

The 1GHz lowpass filter optimized in Sections 17-18 is rather different: the

optimization did not produce such a clear-cut result, and the performance was

not quite within specification. If you accept those results, you're accepting a

compromise.

Why is this? Basically because there is no perfect result in this case. We

decided not to change any inductor values, so SCALC has produced the best that

can be achieved within the constraints we ourselves imposed. If you go back to

the Optimization Editor and add extra lines to allow SCALC to alter the two

inductors as well, you'll find that the results are far better.

19.2  Guiding the optimization

You can guide the optimization towards the desired balance of performance by

altering the weightings in the Objectives Editor. Although these weightings

have a precise mathematical purpose, the absolute values chosen have no

particular significance to the user. What matters are the weighting values

relative to one another. If you find that the optimization is not paying

sufficient attention to a desired aspect of performance, simply increase its

weighting; you can then either re-optimize from the original file, or continue

from where you are.

Optimization can take you 'all around the mountain' in the search for the peak

of performance. Sometimes the network is changed so much that it scarcely

resembles the one you first thought of. In such cases it is often better to

back-track, re-loading the result of a previous stage and continuing from there

instead of always pressing forwards.

TIP

To be able to re-load previous results, make a habit of visiting the Network

Editor after each major stage of optimization, and save those results as a .NET

file under a unique name.

It's very easy to 'get lost on the mountain' if you allow the optimizer to make

random jumps. There may be no way back to the set of parameter values you had

before. Regrettably, SCALC does not provide the equivalent of the helpful local

resident who leans on the farm-gate, looks up at the mountain and says "If I

was going up there, I wouldn't start out from here."

19.3  Say `No' to optimizer dependency

Looking back at the changes we made to the original 1GHz lowpass filter, we

have to ask if that's a good way to develop a filter? We don't think so. More

time should have been spent on selecting a suitable LPF prototype design before

handing the problem over to SCALC. It's very tempting to use an optimizer as a

substitute for good RF design, but besides professional pride there are other

good reasons not to become over-dependent on the power of SCALC's optimizer.

One is that the network may not optimize at all. As we said earlier,

optimization by the `mountain climbing' algorithm is inherently short-sighted,

and if you start with a design that is too far away from the optimum, you may

never get a solution. If you're saying to SCALC, "Go climb that mountain!" it's

best not to start from Kansas. Start from somewhere well up in the foothills,

where you can already see a clear path to the summit.

Another problem with placing excessive dependence on the optimizer is that

optimization may stop with the message:

Error - diverging results!

This seldom happens, but when it does it's a fairly reliable indication that

you are asking too much of the optimizer. Maybe there isn't an answer at all.

19.4  Good candidates for optimization

Here are a few good candidates for SCALC's optimizer.

*   Networks where you want to determine which components are critical.

For this, you don't even need to run the optimization; a Sensitivity

Analysis will show you which components to concentrate on.

*   Networks where the prototype design shows some component values close to

industry-standard preferred values. If you change these values manually in

the Network Editor, you can then run the optimizer on the remaining

components to determine the values required. There may be some loss of

performance, but by careful use of the optimization objectives and

weightings you may be able to `guide' the process towards an acceptable

compromise design.

*   Other networks requiring compromises. Formal design methods generally

produce networks that meet one criterion perfectly, but have unspecified

performance in other respects. For example you may be able to design quite

easily for optimum matching or for maximum gain, but not for both together.

The optimizer allows you to seek a balanced compromise that works better

for your purposes than any of the formally-derived design options.

In all successful cases you will have to adjust SCALC's optimization objectives

and weightings with some knowledge of the potential performance of the network,

and some sympathy for its weaknesses. In such cases you will be using SCALC to

as a complement to own RF design capability, which is exactly how it should be.

20.   OPTIMIZATION OF AN AMPLIFIER

This final section gives you another guided tour, this time using an amplifier

which involves an active device file. We will analyse a prototype design, also

looking at SCALC's outputs of stability circles, and then optimize the

amplifier for gain and input/output impedance matching.

20.1  Prototype design

For this demonstration of the optimizing capabilities of SCALC, we'll use the

simple 1.3GHz amplifier shown in Figure 54. The initial design was done

conventionally, using approximate S-parameters (interpolated from the

manufacturer's data) and a paper Smith Chart. Input and outputs are matched to

50�.

80� 45� @ 1.3 GHz

35� 90� @ 1.3 GHz

�/��������������������������

��������������������������                     �

�    �->Ŀ                 �

in           3.3pF�����      � BFG96    2.2pF�����       out

�����      �               �����

�        �                 �

�����������������������������������������������������

Figure 54:  Inital design of the amplifier

The design target was a gain of at least 10dB and good input and output return

losses. Figure 55 shows the network representation of the amplifier and the

SCALC editor display of the network file AMP(.NET), which is provided on disk.

TRL          CPP         TWO         TRL          CPP

��������Ŀ   �������Ŀ   �������Ŀ   ��������Ŀ   �������Ŀ

�������������������������Ĵ       ���������������������������

�        �   � ����� �   �       �   �        �   � ����� �

�        �   � ����� �   �       �   �        �   � ����� �

�������������������������Ĵ       ���������������������������

����������   ���������   ���������   ����������   ���������

Element No.   #1       #2       #3    | Element No.   #1       #2       #3

TRL   1    35�      90�lng   1.3GHz

CPP   2    3.3pF

TWO   3   Filename = BFG96

TRL   4    80�      45�lng   1.3GHz

CPP   5    2.2pF

CAX   6    1el#     5el#

END   6

Figure 55:  Network representation of the amplifier and the

Network Editor display

20.2  Analysis

Although we are primarily interested in the 1.3GHz performance, it is important

to run the analysis over a wider frequency range, both to check out-of-band

performance and to look for frequency-dependent trends which may help the

optimization process. In the tables below, 1.3GHz has been marked with arrows.

For a first look, it is convenient to use the S-parameter magnitude (dB) table:

Frequency        S11            S21           S12          S22

in GHz         in dB          in dB         in dB        in dB

1.0           -7.35          12.59        -19.76        -9.71

1.05          -7.88          12.56        -19.26        -9.67

1.1           -8.19          12.48        -18.8         -9.63

1.15          -8.22          12.36        -18.37        -9.58

1.2           -7.94          12.16        -17.97        -9.51

1.25          -7.41          11.9         -17.61        -9.42

��> 1.3           -6.7           11.57        -17.29        -9.3  <��

1.35          -5.9           11.18        -17.01        -9.12

1.4           -5.09          10.74        -16.78        -8.89

1.45          -4.32          10.28        -16.6         -8.59

Although not too bad for a first attempt, the design could be improved. Gain is

above the 10dB target but the matching, particularly the input matching, isn't

too good. A look at the I/O impedance results confirms this:

Frequency           IMPEDANCE in �

in  GHz         Input            Output

1.0        76.5 +j 52.41     47.5 -j 33.63

1.05       96.66+j 39.84     45.54-j 32.87

1.1        110.9+j 15.92     43.85-j 32.15

1.15       111.3-j 13.59     42.46-j 31.55

1.2        97.91-j 38.1      41.36-j 31.11

1.25       77.9 -j 51.75     40.5 -j 30.89

��> 1.3        58.34-j 55.7      39.84-j 30.95  <��

1.35       42.32-j 53.54     39.28-j 31.32

1.4        30.26-j 48.23     38.75-j 31.99

1.45       21.6 -j 41.59     38.16-j 32.94

The input impedance is quite reactive and very frequency-dependent. Although

the output match is far less frequency-dependent, it too can be improved.

20.3  Stability circles

One of the best methods of determining the stability of an active device is to

plot its stability circles on a Smith chart, and to calculate its stability

factor K. Options 6 and 7 of the Response Menu produce tables of input and

output stability circles, and K, as a function of frequency. (The present

version of SCALC does not produce graphics displays of stability circles, but

this is planned as a future enhancement.)

K can take a range of values. If K is greater than or equal to 1, the active

device in its network will be unconditionally stable: whatever the terminations

at the input and output, it will not oscillate. If K lies between 1 and -1, the

device will oscillate with certain combinations of input and output impedances.

Broadly speaking, the lower the value of K in this region between 1 and -1, the

more likely it is to oscillate. If K is equal to or less than -1, the device

will be unconditionally unstable: it will definitely oscillate, regardless of

its terminating impedances.

Input and output stability considerations are analysed and plotted separately.

In brief, input stability represents conditions that result in a positive value

of the real part of the output impedance; and output stability implies a

positive value of the real part of the output impedance.

There are also several possible variations on the locations of stability

circles relative to the Smith chart. The circumference of the circle is the

boundary between regions of source or load impedance that will cause

instability, and those that will not. Depending on the relative magnitudes and

phases of S21 and S12, the stability circle may enclose either the stable or

the unstable region; and the circle itself may lie wholly inside the Smith

chart, partly inside it, or wholly outside of the chart. Figure 56 shows the

generic range of six possibilities.

[Sorry - this figure can only be displayed in the printed manual]

Figure 56:  Some options for stability circles on the Smith chart

There are two unconditionally stable cases: where the stability circle covers

the whole of the Smith chart, the stable region being inside the stability

circle; and where the stability circle lies wholly outside of the Smith chart

and the stable region is outside of the stability circle. In the conditionally

stable cases there is a stable-unstable boundary somewhere on the Smith chart,

so that certain impedances will cause instability.

Option 6 of SCALC's Response Menu produces data on input stability circles, and

the table for the prototype 1.3GHz amplifier looks like this:

Frequency    INPUT STABILITY CIRCLES       Stability

in GHz      dist    angle   radius   I/O   Factor

1.0        4.46    161.2   5.6       I     1.11

1.025      3.46    170.3   4.59      I     1.1

1.05       2.88    179.5   4         I     1.09

1.075      2.52   -171.0   3.63      I     1.08

1.1        2.29   -161.8   3.39      I     1.07

1.125      2.15   -152.7   3.24      I     1.06

1.15       2.08   -143.9   3.15      I     1.05

1.175      2.06   -135.5   3.12      I     1.04

1.2        2.09   -127.6   3.13      I     1.03

1.225      2.15   -120.0   3.18      I     1.02

1.25       2.26   -112.9   3.27      I     1.01

1.275      2.41   -106.2   3.4       I     1

��> 1.3        2.61   -99.96   3.59      I     .99   <��

1.325      2.87   -93.99   3.84      I     .98

1.35       3.21   -88.34   4.18      I     .98

1.375      3.65   -82.95   4.61      I     .97

1.4        4.24   -77.79   5.2       I     .97

1.425      5.03   -72.84   5.99      I     .97

1.45       6.13   -68.05   7.09      I     .96

1.475      7.7    -63.39   8.66      I     .96

Column 1 is the frequency, of course. The next three columns show the location

of the stability circle: its centre position (distance from the centre of the

Smith chart, and angle) and its radius, all as shown at the top of Figure 56.

A centre position with a magnitude greater than 1 means that the stability

circle is centred outside of the chart. Comparing the radius of the stability

circle with the distance column shows how much of the stability circle extends

into the Smith chart. For example, if the radius equals the distance, the

stability circle is centred on the edge of the Smith chart and reaches as far

as the centre. On the other hand, if radius � (dist + 1) the stability circle

covers the entire Smith chart.

However, that information is not sufficient to determine whether the amplifier

is stable or unstable, because Figure 56 shows that the stable region can lie

either inside or outside of the stability circle itself. The codewords `I' and

`O' in the fourth column of output give the answer. The final column shows the

stability factor K.

In the case of this 1.3GHz amplifier, the stable region lies inside a circle

which is centred outside of the Smith chart. Below the design frequency, the

stable circle completely covers the chart - radius is greater than (dist + 1) -

so no possible input impedance can provoke instability. Above 1.3GHz the stable

circle pulls away slightly from one edge of the chart, leaving a small region

of input impedances that would provoke instability.

Option 7 of the Response Menu produces output stability circles, and the table

for the prototype 1.3GHz amplifier looks like this:

Frequency       OUTPUT STABILITY CIRCLES     Gmax

in GHz      dist    angle   radius   I/O    in dB

1.0        1.64   -62.66   2.82      I     13.92

1.025      1.6    -61.86   2.76      I     13.85

1.05       1.56   -60.92   2.71      I     13.79

1.075      1.52   -59.83   2.66      I     13.74

1.1        1.49   -58.54   2.61      I     13.68

1.125      1.46   -57.05   2.56      I     13.63

1.15       1.43   -55.33   2.51      I     13.58

1.175      1.4    -53.36   2.46      I     13.53

1.2        1.36   -51.15   2.41      I     13.48

1.225      1.33   -48.66   2.35      I     13.42

1.25       1.28   -45.88   2.29      I     13.35

1.275      1.23   -42.8    2.23      I     0

��> 1.3        1.17   -39.38   2.15      I            <��

1.325      1.11   -35.58   2.08      I

1.35       1.03   -31.34   1.99      I

1.375      .95    -26.6    1.9       I

1.4        .87    -21.25   1.8       I

1.425      .79    -15.17   1.71      I

1.45       .7     -8.22    1.61      I

1.475      .62    -.21     1.52      I

The first five columns are the same as for the input stability circles. At

frequencies where the network is stable (K�1) the last column displays the

maximum available gain Gmax, this being the gain obtainable when both input and

output impedances are conjugately matched. If K is less than 1, the Gmax column

remains blank.

20.4  What to optimize?

To optimize the amplifier, we must first set our optimization objectives,

using the Objectives Editor as shown in Figure 45. Figure 57 shows the

objectives are included with the distributed files as AMP(.OPT):

FREQ     S11       S22       S21

in GHz  dB   WGT  dB   WGT  dB   WGT

1.3   -15   10  -15   10   20   10

Figure 57:  Performance objectives for the 1.3GHz amplifier

Since we are interested primarily in the amplifier's performance at 1.3GHz,

the objectives file consists of one line, referring to that frequency only.

If we were interested in broad-band performance, then up to 20 frequencies

could be entered. Targets of -15dB have been set for S11 and S22, with the

S21 (gain) target set at a rather unrealistic 20dB. Equal weighting has been

given to these three requirements.

Now we have to choose the elements that are to be optimized. The sensitivity

analysis option of the Optimization Editor is very useful here, in identifying

worthwhile subjects for the optimization process. In this case we have four

optimizable elements, two shunt capacitors and two transmission lines. That

gives a total of six possible optimizable parameters (which is also the maximum

number that SCALC can handle) but we don't necessarily have change all of them.

The input line is a 90� (quarter-wave) matching section; we'll leave that

length alone, because we know that it will be possible to achieve a match just

by varying the impedance of the line and the value of the shunt capacitor. So

we have five possible candidates for optimization, which are entered into the

optimization editor. By running the sensitivity analysis (press S) we get a

good idea of the most useful targets for the optimization process. The larger

the gradient, the more effect that parameter will have on meeting our

performance objectives.

Sensitivity

type  num  num   value    GRADIENT

1  TRL   1    1   35�         93.94

2  CPP   2    1   3.3pF      -221.76

3  TRL   4    1   80�         33.36

4  TRL   4    2   45�lng      48.74

5  CPP   5    1   2.2pF       748.73

Figure 58:  Sensitivity analysis for the 1.3GHz amplifier

Clearly, altering the values of the two capacitors will have the greatest

effect. We have two parameters for the second transmission line. It's not a

good idea to vary more than one parameter at a time on any given component.

So we'll delete the least sensitive parameter, the output line impedance (line

3 in Figure 58).

20.5  Optimization results

Run the optimizer (without any random jumps - in practice you seldom use this

option).

When the optimization ends, the screen shows the optimized values, as also will

the Network Editor (Figure 59).

Element No.   #1       #2       #3   �  Element No.   #1       #2       #3

TRL   1    33.799� 90�lng   1.3GHz

CPP   2    5.612pF

TWO   3   Filename = BFG96

TRL   4    80�     32.235�L 1.3GHz

CPP   5    .817pF

CAX   6    1el#     5el#

END   6

Figure 59:  The 1.3GHz amplifier after optimization

At this stage it is worthwhile to 'round off' the optimized values with the

Network Editor, as shown in Figure 60, before re-analysing the performance.

Note for example that the impedance of the input line is very little

different from the starting value.

Element No.   #1       #2       #3    � Element No.   #1       #2       #3

TRL   1    35�      90�lng   1.3GHz

CPP   2    5.6pF

TWO   3   Filename = BFG96

TRL   4    80�      30�lng   1.3GHz

CPP   5    .82pF

CAX   6    1el#     5el#

END   6

Figure 60:  Optimized 1.3GHz amplifier, edited to rounded and preferred values

Now the analysis can be re-run (don't forget to re-enter the frequency range).

First, let's examine the S-parameter display:

Frequency        S11            S21           S12          S22

in GHz         in dB          in dB         in dB        in dB

1.0           -9.88          13.26        -19.09        -13.29

1.05          -11.7          13.29        -18.53        -13.42

1.1           -13.63         13.28        -18           -13.43

1.15          -15.32         13.21        -17.51        -13.34

1.2           -16.23         13.08        -17.06        -13.21

1.25          -16.13         12.88        -16.63        -13.07

��>  1.3           -15.41         12.63        -16.23        -12.97  <��

1.35          -14.53         12.33        -15.86        -12.92

1.4           -13.73         12           -15.52        -12.95

1.45          -13.08         11.67        -15.21        -13.07

It is easy to see that we have now achieved the performance objectives. Both

S11 and S22 are better than -10dB, and the gain at 1.3GHz has increased to

a little over 12dB. A glance at the I/O impedances shows the better matches.

Frequency             IMPEDANCE in �

in GHz          Input             Output

1.0        74.55+j 33.25     63.92-j 20.84

1.05       79.87+j 16.31     59.5 -j 21.83

1.1        76.29+j .46       55.32-j 22.31

1.15       67.69-j 9.84      51.48-j 22.31

1.2        58.4 -j 14.65     48.07-j 21.87

1.25       50.47-j 15.88     45.1 -j 21.06

��>  1.3        44.31-j 15.17     42.55-j 19.93  <��

1.35       39.66-j 13.51     40.37-j 18.48

1.4        36.19-j 11.36     38.5 -j 16.69

1.45       33.59-j 8.87      36.89-j 14.51

If you check the input and output stability circles, you will find that these

have scarcely changed at all. Since the K values are either greater than 1 or

very close to 1, the amplifier will be stable with any reasonable values of

input and output terminations. However, you may still need to analyse the

stability at higher and lower frequencies, and possibly extend the analysis to

consider the larger network in which this amplifier is embedded.

CONCLUSION

This concludes the manuals for SCALC. We hope you have found the manuals clear

and readable, and that you enjoy using the software.

We have a long list of improvements that we'd like to add to SCALC. If you have

any helpful suggestions to make, please register and let us know. The more

people who register, the more we'll be encouraged to develop SCALC further.

SCALC ERROR CODES

Space is very much at a premium in the shareware release of SCALC. Error

handling is kept to the very minimum, so often SCALC will merely report an

error number. The most likely encountered error numbers are detailed below.

Error             Explanation

6, 11        Numeric Overflow or a divide by zero attempted.

Possibly a zero value for a component was entered.

7            Out of memory.

25            Printer is off-line.

27            Printer out of paper.

52, 64        Illegal filename - either you added a file suffix

such as .NET or used non-permitted character in the filename.

67            There is already the maximum permitted number of files on the

disk.

68            This drive does not exist.

70            The disk is write-protected.

71            Either there is no disk in the drive or the door is

open.

72            The floppy disk is faulty.

59, 62, 63    The file was not in SCALC format.

The registered version of SCALC has much improved error handling.

Why not register today?  See REGISTER.DOC for details.

#