List of demodulator interfacing modes for JVFAX 6.0:

====================================================

Note:

=====

If you should have an interface that does not run under JVFAX with one

of the settings described below you should consider to write an

external interface driver yourself. For more info about this see file

'JVTSR.PAS'.

A) asynchroneous data acquisition

=================================

Serial port:

============

This is the standard method for data acquisition in JVFAX.

Because nearly every PC has a built in serial interface I've chosen

this to be to be the standard.

In order to keep the demodulator as simple as possible, not the

serial data input line RxD is used for data acquisition, but

instead the four incoming handshake lines (DCD,RI,CTS and DSR) are

used.

These four input lines look to the program like a 4 bit parallel port.

In order to expand tthe data input capability via these 4 lines to

8 bit, a multiplexing technique is used by the programme.

The program fist reads in the 4 most significant data bits with the

RTS line set to high.

Then, the RTS line is toggled to the low state and after a (very)

short delay the lower 4 bits are read.

If your interface delivers less than 8 data bits, these must be left

(MSB) aligned within the 8 bit word.

When using this interface method you must enter the >base< address

of your serial port in the address field.

Standard port base addresses for serial ports are 03F8h (COM1),

02F8h (COM2), 03E8h (COM3) and 02E8h (COM4).

No interrupt from the serial port in used, so the IRQ-field in the

configuration screen is meaningless. The data rate field is as well

meaningless, unless you're using a demodulator that can be remotely

controlled by JVFAX.

In such cases, control bytes are sent to the interface via the TXD

line with the baud rate specified in the baud rate field. (8N1)

Some more 'intelligent' interfaces such as the "EASYFAX" can interpret

these control bytes and be set to the appropriate deviation, etc.

You should set the "bits" field to the number of significant bits

that your interface can actually deliver.

To receive SSTV, your interface must either deliver a SSTV-sync pulse

information in the two least significant bits of the input data byte

(active low) or it must be set to 800 Hz deviation.

In the first case, the LSB-SSTV-sync entry in the configuration menu

must be set to "yes", else to no. Two bits of data are used for

SSTV sync identification to keep the program open for future

extensions, especially VIS-code (=vertical interval signalling)

interpretation.

If you're planning to design a new demodulator that would detect

the SSTV sync pulses individually, it should deliver the following

bit patterns in bit 0 and 1:

Bit 1 0     identifies:

========================

H L     1050-1100 Hz

L L     1150-1250 Hz

L H     1250-1350 Hz

When LSB-SSTV sync is set to no, the program assumes that your

demodulator is set to 800 Hz deviation and will try to identify the

sync pulses itself.

Serial port/ser

===============

Works exactly like the above method with the difference that data is

read in serially via the RXD-line of the serial port.

In this case, the baud rate field must be set to the correct baud

rate of your interface. The interface must deliver data in the 8N1

format, and its baud rate should be set preferably to 57600 in order

to achieve a good picture resolution.

Comparator:

===========

This is by far the easiest way of getting a FAX-signal into the

computer.

Audio frequency from the receiver is shaped to a rectangular waveform

by means of a comparator and fed directly to one handshake line (DSR)

of a serial port.

Because the computer itself has to do all the signal decoding, this

is a very timing sensitive method that will fail on slow computers.

Even more, this method in some cases conflicts with some memory

managers such as EMM386. (But most interestingly not on every machine)

Results obtained with such a 'software demodulation' are from very

poor up to excellent.

Generally, you can prevent yourself from lots of frustrations when

not trying this kind of signal demodulation on a 8088/86 or

similarly slow computer.

A comparator circuit that could be used for software demodulation

might look like this one:

RS232 Pins

(25-pin receptacle)

100 nF         741        4 x 1N4148

�����������Ĵ#������O RTS        4

��           3 �\ �7    �          �

O����Ĵ�������������Ĵ+\�     �����Ĵ#Ŀ �

��   �         �  \ 6            � �

AF in         �         �   >������������������O DSR        6

�       2 �  /              � �

O��Ŀ      �    ����Ĵ-/�     ������#�ĳ��

�      �    �    �/ �4    �        �

���    ��   �       ������������#�������O DTR       20

� �  ���

47k � �

���

�            ��������������������O GND        7

���          ���

The 741 operates as a limiting amplifier with full free-run amplification.

It forms the audio frequency to a rectangular wave form with RS232 compatible

level.

Please notice that this simple circuit is only suitable for the reception

of FM-FAX transmissions and not for the reception of geostationary or low-

orbiting satellites.

RTS and DTR are programmed to serve as power supply leads for the

comparator, so that no external power supply is needed.

The 'bits' field can be set to 7 bits, but in some cases better

results are obtained when reducing this value to 6, or 4.

The port base address and the IRQ level of the serial port

must be enterd. Normally, IRQ is 4 for COM1 or COM3, and 3 for COM2

or COM4.

The program will not accept IRQ's above 7.

The data rate and baud rate fields are meaningless.

HAMCOMM:

========

This is just the same as the comparator interface with the

difference that the DTR and RTS lines are toggled to serve as a

transmit/receive switch.

The HAMCOMM interface is very popular for the reception of RTTY etc.

in conjunction with the famous HAMCOMM shareware program.

Parallal port:

==============

Data is being read from the specified port address periodically.

There's no handshake and all the timing is done by the program.

Despite the fact that this is by far the easiest way of interfacing,

as seen from the demodulator side, the problem is, that normally

there's no parallel input port available on PC's. Of course one could

use the game port or the 'modem status' port of any serial port, but

these all are only capable to deliver a 4-bit information.

The optimum solution would be a bidirectional printer port, where the

8 data lines could be used for input.

On some computers this is standard. (e.g. on Toshiba laptops, where

you can enable this option in the setup, or on some IBM-PS/2 models)

Fortunately, most older printer port cards (equipped with standard TTL

components) can be modified very easily for data input by simply

cutting the output enable pin of the data output latch.

No interrupt from the parallel port in used, so the IRQ-field in the

configuration screen is meaningless.

The baud rate field and the data rate field are as well meaningless.

Standard port addresses for printer ports are 0378h (LPT1:),

or 0278h (lpt2:).

If your interface delivers less than 8 data bits, these must be left

(MSB) aligned. You should set the "bits" field to the number of

significant bits that your interface can deliver.

In order to make a printer port bidirectional, or to prepare it for

input, bit 5 of the printer control port is set to high. before

reading any data from the data port.

Additionally, to get some IBM PS/2 machines set the printer port for

bidirectional operation, the follwing operations are performed

whenever JVFAX initializes:

020h is output to port 94h, then

bit 7 of port 0102h is reset, then

0A0h is output to port 94h.

Alternatively on a IBM Model 30, bit 7 of Port 065h is reset.

A&M design board, MEFAX:

========================

both are plug in cards which allow for the reception of both AM-

and FM-FAX signals.

Not all options of these cards are fully supported by the program,

especially regarding the A&M-board which allows for a synchroneous

buffered data acquisition.

Faxellite:

==========

is the interface that SSB-electronics supplies with their excellent

DIGISAT program.

I don't know whether they changed their interface design in the

meantime, so it might be that my program will not work with

the up-to-date version of this interface.

BPF2 (8 bit):

=============

is to interface to the BPF2 FM demodulator interface offered by

Ingenieurb�ro Becker & Poth (see below)

B) synchroneous data acquisition

================================

While all the above interfaces leave the complete signal timing up

to the program's task, the following drivers are to support interfaces

that can deliver some kind of pixel clock, besides the data itself.

This gives some advantages when, for example, receiving polar orbiting

satellites or when replaying tape recordings of such satellite passes.

All these drivers haven't been tested yet, and I'm not sure if they

will work at all.

SYNCHRON SERIAL:

================

works quite the same way as the serial port/ser interfacing with the

difference that the program timing is derived from the data rate of

the interface. (not the baud rate!!)

The correct data rate must be entered in the data rate field of the

configuration screen. the baud rate must be at least ten times the

data rate, because for every pixel to be read there are 10 bits to be

transferred (8 data bits and one stop bit).

The data rate itself normally is determined by the signal source,

e.g. 2400 or 4800 pixels/s for polar orbiting satellites.

Use af an 16550 FIFO UART is strongly recommended.

BidPrintPortIRQ:

================

same interfacing method as parallel port (see above).

Will only work on a bidirectional printer port (or a printer port

modified for data input).

Additionally, the ACK-line serves as a data strobe signal which must

be fed with the synchroneous clock delivered from the interface

(active low).

BidPrintPortPoll:

=================

same interfacing as parallel port.

You need a parallel printer port that is bidirectional or that has

been made bidirectional.

To use this option your demodulatormust be capable of buffering at

least 2 (or better more) complete picture lines.

Data is read in aperiodically by the program. The program will

read data from the the data port whenever and as long as the ACK line

is kept low by the interface.

For every byte read, a short pulse is generated on the strobe line.

Because no tests have ever been made with this driver (due to the lack

of an appropriate interface) I'm in doubt if it will work as it

should. If anyone would develop an interface that meets these

specifications and would leave it to me for some tests I would try to

get the whole thing running.

This interfacing technique should enable the use of the JVFAX program

under WINDOWS or OS/2, so it could become more and more interesting in

the future.

List of modulator types that can be selected for JVFAX 6.0:

===========================================================

In general, every transmission done by JVFAX has a maximum intensity

resolution of 64 levels (6 bit). When outputting digital intensity

data via a serial or a parallel port, these bits are right (LSB)

aligned within the output byte, just in oppposite to the demodulator

seide, where every data is MSB aligned.

You can use the following 'modulators':

Speaker:

========

audio is generated through the PC's built in speaker.

you can use a simple low pass filter to obtain a signal that can be

fed to your transmitter's microphone input jack:

100 nF

to              220     10k

speaker    ��  ���Ŀ   ���Ŀ

O����Ĵ��Ĵ   ���Ĵ   �������Ŀ

��  ����� � ����� �     � 10k

�       �    ��      v: depends on your TX's

(use the lead)       �       �    � �    ���Ŀinput characteristic

(that gives maximum)    �4n7    �4n7 � �#��Ĵ   ����O

(audio output)      ���     ���   � �    �����     to

���     ���   � �    1k-100k   transmitter

�       �    ���    ��������O

O��Ŀ           �       �     �     �   100 nF means .1    �F

�           �       �     �     �   4n7    means .0047 �F

GND  ���         ���     ���   ���   ���

Serial audio:

=============

works like the above one, but audio is generated on the TxD line of a

serial port. You must enter the base address of your serial port

into the address field.

You can use the same low pass filtering as above to connect the TxD

pin of the serial port to your transmitter's input.

Serial port:

============

when having selected this option, no audio is generated by the

computer. Instead, the digital intensity information is being output

to the TxD line of the serial port (0 for black, 63 for white, etc.)

The format 8N1 is used, with the baud rate selected in the baud rate

field.

Your modulator must parallelize this information, convert it to an

analogue signal and feed a voltage controlled audio generator with it.

An example modulator that works this way is shown in the file

"MODULAT.GIF"

Output values above 63 indicate a remote control command to a

demodulator being eventually connected to the same port, with the

exception of the following:

125:   generate 1100 Hz audio  ;as SSTV sync signal

126:      "     1200 Hz   "    ;all values are in decimal

127:      "     1300 Hz   "

The differentiation between 1100,1200 and 1300 Hz is to enable VIS

signalling.

To simplify things, you can generate a 1200 Hz tone whenever values

greater than 63 and less then 128 are being output.

If you don't want to transmit in SSTV you can forget about all that

1200 Hz stuff.

Parallel port:

==============

like above, but data is being output via a parallel port, e.g. a

printer port.

IBM parallel port 25 pin connector:

===================================

/Strobe     1 ���O

D0         2 ���O  O��� 14       /Auto feed

D1         3 ���O  O��� 15       /Fault

D2         4 ���O  O��� 16       /Reset

D3         5 ���O  O��� 17       /Select in

D4         6 ���O  O��� 18        GND

D5         7 ���O  O��� 19        GND

D6         8 ���O  O��� 20        GND

D7         9 ���O  O��� 21        GND

/ACK       10 ���O  O��� 22        GND

Busy      11 ���O  O��� 23        GND

PE        12 ���O  O��� 24        GND

Select    13 ���O  O��� 25        GND

Signals which are preceeded with a "/" are active low.

IBM serial port 25 pin connector:

=================================

Prot.GND   1 ���O

TXD        2 ���O  O��� 14

RXD        3 ���O  O��� 15

RTS        4 ���O  O��� 16

CTS        5 ���O  O��� 17

DSR        6 ���O  O��� 18

GND        7 ���O  O��� 19

DCD        8 ���O  O��� 20        DTR

9 ���O  O��� 21

10 ���O  O��� 22        RI

11 ���O  O��� 23

12 ���O  O��� 24

13 ���O  O��� 25

IBM serial port 9 pin connector:

================================

DCD        1 ���O

RXD        2 ���O  O��� 6         DSR

TXD        3 ���O  O��� 7         RTS

DTR        4 ���O  O��� 8         CTS

GND        5 ���O  O��� 9         RI

Configuration hints on some demodulators

========================================

Here is shown what you should enter in the demodulator/modulator

field of the configuration menu when using the following devices:

EASYFAX:

========

Demodulator: 8 bits on serial port;      Baud rate 57600

Modulator:   6 bits on serial port       LSB-SSTV-sync: yes

Do not use the "serial port/ser" option for demodulator. This would

work too, but using the "serial port" interfacing gives better results.

Port addresses depend on what serial port your Easyfax is

connected to.

HARI-FAX:

=========

Demodulator: 8 bits on serial port/ser. Baud rate 38400

LSB-SSTV-sync: no

Modulator:   use Speaker or serial audio with the circuit

described above

Port address for "demodulator" depends on what serial port your

HARI-FAX is connected to.

DL5JM demodulator/modulator:

============================

- no longer available -

WRAASE MR-9202:

===============

Demodulator: 8 bits on serial port;      Baud rate: doesn' matter

LSB-SSTV-sync: doesn't matter

Modulator:   use Speaker or serial audio with the circuit

described above

Port address for "demodulator" depends on what serial port your

WRAASE RECEIVER is connected to.

NSK-demodulator:

================

Demodulator: 7 bits on serial port      Baud rate: doesn't matter

LSB-SSTV-sync: no

Modulator:   use Speaker or serial audio with the circuit

described above

Port address for "demodulator" depends on what serial port your

NSK demodulator is connected to.

BPF2 (8 bit version):

=====================

Demodulator: 7 bits on BPF 2 (8 bit)    Baud rate 57600

LSB-SSTV-sync: no

Modulator:   use Speaker or serial audio with the circuit

described above

Port address for "demodulator" depends on what serial port your

BPF 2 is connected to. Becker & Poth deliver a "8 bit" version

of the BPF 2, specially designed for use with JVFAX.

There is a "9 bit" version of the BPF 2, too, which will currently

not work with JVFAX.

A short list of (german) demodulator/modulator suppliers known to me:

=====================================================================

This list will certainly be incomplete, and the ordering of the

addresses will not tell anything about the quality of the interfaces

being offered.

All these interfaces should co-operate with JVFAX, but in no case

I can guarantee for that.

"HamComm" interface (comparator type) by Dieter Dippel, DF4RD:

Ready-to-run  HamComm-Interfaces  can  be ordered from Dieter

Dippel,     DF4RD.      They     are     build    with    SMD

(Surface-Mounted-Device)  components  and  housed  in  a case

similar to a mouse-adapter.  There are two versions available

for 9-pin or 25-pin COM ports.

__                   __                 __

_|  |_________________|  |_

| |                       | |

COM-Port   | |      SMD-HamComm      | |   RX/TX      app.

9-pin female | |       Interface       | | 9-pin male   35mm

|_|   _________________   |_|

|__|                 |__|                __

|       app. 60mm       |

__                                      __

_|  |_____

| |         \

| |           \          __

| |             \_______|  |_

| |                        | |

COM-Port   | |      SMD-HamComm       | |   RX/TX     app.

25-pin female | |       Interface        | | 9-pin male  55mm

| |               ______   |_|

| |             /       |__|

| |           /

|_|   _____ /              |

|__|                     |                __

|

|       app. 60mm        |

The  interface  also  works with SSTVFAX, PKTMON, EASYFAX and

the well-known HamComm (rtty/amtor/cw) program by Django Schroeder,

DL5YEC.

For more information please request an updated price list and

terms of delivery from

Dieter Dippel, DF4RD

Fenitzerstr. 33

D-90489 Nuernberg

Phone/Fax: (0) 911 / 55 92 96  between

Mo. - Fr. 18:00 and 20:00 german local time

or 17:00 and 19:00 UTC

"EASYFAX" kit from  Ulrich Bangert, DF6JB:

Ulrich developed a superb �P-based transmit/recieve interface that is

easy to assemble and that needs absolutely no alignment. It can handle

up to 256 intensity levels, AM and FM FAX as well as SSTV.

The interface is offered as a complete kit including a preassembled

case and can be easily updated by EPROM exchange.

Ulrichs address:

Ulrich Bangert

DF6JB

Ortholzer Weg 1, Klein Ippener

27243 Gro� Ippener

Tel.: 04224/431

"MMI" by C-Data:

Address:

C-Data

Hohenwarter Str. 6

85276 Pfaffenhofen

Tel.: 08441/6145

"DK8JV-FAX-Konverter" by NSK:

A 64 grey scale demodulator for AM and FM FAX

Address:

Ingenieurb�ro f�r Satellitentechnik

Dipl.-Ing. B. Ewert

M�hlenweg 11

24217 Stakendorf

Tel.: 04344/6758

"Meteosat-Empfangs-Set"

Complete rig for the reception of geostationary weather satellites

such as Meteosat, including receiver, antenna and interface.

Available from

Fa. Bogerfunk

Funkanlagen GmbH

Grundesch 15

88326 Aulendorf

Tel.: 07525/451

"WX-Satelliten-Empf�nger MR-9202"

A 137-Mhz receiver for polar orbiting satellites with a built-in

interface.

Available from:

Volker Wraase Elektronik

Kronsberg 10

24161 Altenholz

Tel.: 0431/32528

"BPF 2"

A �C controlled subminiature FM demodulator is supplied by

Ingenieurb�ro Becker & Poth GmbH

D�sselthaler Str. 49

40211 D�sseldorf

Tel: 0211/9061340

"HARI-FAX"

A �C controlled AM/FM fax demodulator that can deliver up to 256

intensity levels. Available as kit or as assembled board.

Fa. Siegfried Hari

Spessartstr. 80

63448 Seligenstadt

Both the Easyfax and the Hari-Fax interface support remote control

of the interface by the JVFAX program.