NSCC Serial Communication Card

1.Presentation

This board has been realized to free serial ports of PC and eliminate TNCs

that dont seem to me the right choice for high speed. Often, to find 10MHz

Z80 CPU is a worst problem respect building this board.

Most of today BBS need a PC always on, since a TNC isn't enough, the same thing is

valid for node, so for my point of view, it's better to use a board inside PC

with internal and/or external modem.

Many project has already been realized by several groups, I can remember the

PI2 Interface from Ottawa Packet Group, which use DMA (Linux driver has been

written too), and OptoPcScc by PA0HZP to which I inspired myself.

The only advantage, according to my point of view, about NSCC is its

simplicity: I've designed NSCC in order to reduce the number of ICs and to get

a PCB suitable for home made (double sided since PC bus edge connector is

double).

You don't need metallized hole for via, you can solder some piece of wire

instead.

Version 1.2 is provided, on board, by a 1200AFSK modem based on TCM3105, you can

decide to insert relative components or not.

2. Hardware

This board has been tested on a 486DX50 with ISA BUS at CLK/4, so I think it

should work on every ISA BUS operating up to 12-13MHz.

It's an AT board but connection on AT part of edge connector regard only some

IRQs (it's an 8 bit board), so, apart from mechanical problem, it should

been inserted in XT BUS (you will not have IRQs 10, 11, 12, 15 obviously).

By means of dip-switch (called PORTSEL) you  can select each hardware address

(from 0x000 to 0x3ff) and by a jumper the IRQs 5, 6, 7, 9, 10, 11, 12, 15.

Further you can disable the board simply switching one of the dip-switches

without removing board from slot.

Here the layout of dip-switches and jumper when you hold the edge connector

down.

-------------------------------   1

| E |   |   |   |   |   |   |   |  ON      -------------------------------

| n |   |   |   |   |   |   |   |         |  1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

|   | A | A | A | A | A | A | A |         |    |   |   |   |   |   |   |   |

| D | 3 | 4 | 5 | 6 | 7 | 8 | 9 |         |    |   |   |   |   |   |   |   |

| i |   |   |   |   |   |   |   |         | 15 |12 |11 |10 | 5 | 6 | 7 | 9 |

| s |   |   |   |   |   |   |   | OFF      -------------------------------

-------------------------------   0

1   2   3   4   5   6   7   8

----- PORTSEL -----                         ----- IRQ -----

3. Software

Actually the board has been tested only with TFPCX with very good results, as

soon as possible it will be tested with NET/NOS and under Linux.

To configure properly the driver use value for OptoPcScc (as described in

TFPCX manual), except for INTACK logic that is not present (the only driver I

know using the logic is the one in NET/NOS, so use 0 instead of 0x168 and refer

to NET/NOS manual).

Here is an example of configuration for TFPCX for my setup: I use 0x300 as

base address and IRQ9, according to clock source the third field must be 4

TFPCX -POSCC:300:09:4444 -B1200:1200:9600:9600 -NL

I've configured the board to have 1200 baud for port A & B and 9600 baud for

port C & D.

Two file START.BAT and SCC.BAT are also present in distribution package, from

those you can see the difference between standard configuration for KISS mode

TNC and SCC (the terminal program I use is TSTHOST).

Further I've write a simple program for asyncronous communication for such

board, so modems (who will have TTL I/O level) can be reused for such

purpose. This program (ASYSCC10.ZIP) is into the distribution package for

version 1.2.

4. Schematic and printed circuit board

Schematic, artwork of PCB, layout and bill of material are included in

this package as separate file.

5. Other

All the file in this package are public domain, could be copied and

distributed free, but no charge should be added.

The author is NOT responsible for any malfunctioning, damage from using the

board realized and used in any way, he rest available for every other

information.

No parts of the board could be asked to the author, and the author don't know

if someone can supply such parts.

If you want contact me, please use one of the following modes:

- by mail:  Lapo Pieri  via A.dei Corbizi 9   50127  FIRENZE - ITALY

- by phone:  +39 55 410209

- via AX25:  ik5nax@iw5afk.itos.ita.eu

- via e-mail :  icesb@iroe.fi.cnr.it

Florence 27/9/1995