ROTATOR ALGORITHM

Rotator Setup is required when using the IBM, AD200, and ADA1100

interface boards.  All the listed functions are used in the IBM and

AD200 boards, and most are used for the ADA1100.  You do not need the

Rotator Setup for the Kansas City Tracker except to indicate whether

your antenna is north- or south-breaking.  (North-breaking means that

the antenna goes from 0-360� and the center of the indicator is South;

South-Breaking means that the rotator goes from 180�, through west to

360�, then through east to 180�; the middle of the indicator is North).

The rotator setup screen may appear confusing at first.  Here's

how it works.  A zone is established around the current calculated

Moon (or other celestial object) position.  That zone is called the

dead band.  So long as the antenna position is within the dead band,

the antenna is not commanded to move.  If the antenna is swayed by a

gust of wind, or if time passes and the target (Moon, satellite, etc.)

drifts, then the antenna may be outside the dead band and the antenna

will be commanded to move in either azimuth or elevation.  In my

system, I set the azimuth dead band to 2.0�, and the elevation dead

band to 1.0�.  This means that if the target's current position is,

say, 180� azimuth and 45� elevation, that any antenna position within

178-182� azimuth and 44-46� elevation is OK.

Perhaps you want the antenna to aim for the middle of the dead

band, rather than being satisfied with just being inside it.  The

azimuth (or elevation) dead band narrowing factor establishes a

narrower dead band for aiming the antenna than for telling it whether

to move or not.  Numerically, it is the portion of the normal dead

band to be used as a target region while moving the antenna.  Let's

say you have set the azimuth dead band to 2�.  Once you have acquired

the target, the antenna will be pointing somewhere within +/- 2� from

the current calculated target position.  As the target drifts, the

trailing edge of the dead band will eventually cross the azimuth or

elevation where your antenna is pointed.  The antenna will be

commanded to move at this time.  If the dead band narrowing factor is,

say, 0.5, then the antenna will try to guide itself to within +/- 1�

[2 * 0.5 = 1.0] of the current target position, instead of being

satisfied with being merely inside the dead band.  In other words, the

dead band is narrower when the antenna is moving than when it sitting

still.  It is unwise to make the dead band narrowing factor too small;

your system will rebel by "hunting" an excessive amount.  A narrowing

factor of 1.0 means that the dead band is not narrowed at all.  I use

a value of 0.5.

Big antennas develop considerable momentum when they are turning

fast.  This can cause them to overshoot their target when first trying

to acquire the target.  RealTrak takes this into account by widening

the dead band when the antenna is initially "way off" the target.

Coasting functions only come into play when the antenna has to slew a

considerable amount; their effect is proportional to the amount of

slewing (just as the antenna's momentum is).  When the antenna is "on

target," the coasting functions are zero, meaning that the dead bands

you set are used (perhaps narrowed by the narrowing factors, above).

When the antenna is more than twice the dead band width away from its

target position, RealTrak temporarily widens the dead band by a factor

called the coasting angle constant.  The larger this constant (and the

greater the angle between the antenna and the target) the wider the

dead band.  The relationship is :

CoastAngle = AngleK * DifAngle

where:    CoastAngle = Angle added to current dead band width;

AngleK = Coasting angle constant, set in Rotator Setup;

DifAngle = Abs(target position - antenna position) - dead band.

[Difangle ranges between minimum (2*dead band)� and maximum

30�]

For example, if DifAngle is 50�, the coasting angle constant is

1.0, and the nominal dead band is 1.5�, then the temporary dead band

will be +/- 31.5� [(30� * 1.0) + 1.5 = 31.5�].  If the coasting angle

constant had been 0.5, then the temporary dead band would have been

16.5� [(30�*0.5) + 1.5 = 16.5�].  CoastAngle is recalculated after the

antenna gets within this dead band so the new dead band will become

progressively smaller.  After a few iterations, the antenna will have

acquired the target.  At this point, CoastAngle will be zero, meaning

no added width to the dead band.  No coasting angle is added if the

antenna is within twice the width of the dead band, so the coasting

function is disabled when you have acquired the target.

A coasting time constant is also employed in the azimuth rotator.

It is calculated in a manner similar to the angle constant and is

proportional to the slew distance (DifAngle):

AzCoastTime = AzTimeK * (DifAngle�30)

where:    AzCoastTime = Time delay before setting of azimuth brake;

AzTimeK = Azimuth coasting time constant (in Rotator Setup).

The azimuth rotator will hold off setting its brake for a time

proportional to DifAz.  If DifAz is, say, 15� and azimuth coasting

time is set at 3 seconds, then the brake will hold off for 1.5 seconds

[3 * (15�30) = 1.5 sec.].  The brake will delay the full amount if

DifAz is >= 30�.

Elevation coasting time is set directly.  In other words, if you

set it to 2.0, that's what you will get regardless of the elevation

DifAngle.  This is because elevation rotators are usually much slower

than azimuth rotators.

Minimum time for elevation rotator represents, oddly enough, the

minimum time that the elevation rotator will remain on.  This feature

was necessary because my elevation rotator interacts a bit with its

position indication potentiometer; the rotator would chatter on and

off when it was making small adjustments.  Now it doesn't.

The A/D conversion smoothing factor is the number of times that a

single analog input channel is read for each displayed value.  Setting

it to 50 means that the channel is scanned 50 times and the values

averaged.  This smooths out small fluctuations and keeps the last

digit in the antenna position from jumping around.  The setting of

this value depends on the speed of your computer.  Set it at the

smallest value that gives a tolerable display.

MRO

rev. 7/21/93