VK3UM EME PLANNER         Current from Version 9:00 November 2000

MAIN MENU OPTIONS

Selection of the three options is achieved by moving the highlighted bar with

the cursor keys or by typing the first letter of that option required.

Calculations.

This Sub Menu allows you to select each of the 7 main calculations provided by

the software.  Note that to run Common Windows you must first select a DX

Location from the DX Location Sub Menu.

DX Locations.

This Sub Menu provides for the selection of 5 options where you may choose a DX

Location for use as either Home or DX from the DXCC Data Base. You may also

select a Location by Grid Square. In addition you may also Add or Edit your own

Data Base and calculate distances. Current settings are displayed.

General Set Up.

This Sub Menu provides the default parameters that are stored as your own Home

Location and are used on start up. You may also set your Computer Time from

within this Menu. Remember to save your changes when you are finished.

ooo OOO ooo

CALCULATIONS - MENU OPTIONS

Real Time Track.

This option provides real time calculations for Azimuth, Elevation, Doppler

Shift for any DX location, frequency, Tracking Target (Moon - Sun - Noise), as

selected from within the DX Location Menus or General Set Sub Menus. You may

also select any other Target by [F2]. The screen will indicate both the

Transmit (red) and Receive (blue) periods when the Moon is selected and is

preceded with an alarm sound if the latter is turned on. If you first select

a DX Station then this screen will also provide you with an Azimuth, Elevation,

Doppler Shift, and Spacial Offset to the DX Station. A further option is

available via the [F3] and [F4] keys where you may input a DX Station and then

display that Stations Azimuth, Elevation, Doppler Shift and Polarisation

offset (Spatial offset). The [F3] key enables direct entry of a Grid Square

whilst the [F4] key allows you to select a QTH from the Data Base.

Moon Position.

This option provides calculations for UTC, Local Time for the Moon at your own

Location or any other selected from the DX Locations Sub Menu. Azimuth,

Elevation, RA, DEC, GHA, Sky Noise, Doppler Shift, Path Loss (ref. to Moon

Perigee) as well as the Sun Elevation and Azimuth when visible. A "Paged" print

out is provided by selecting this option from the General Setup Menu.

Common Moon Windows.

This option provides a means of calculating Common Moon Windows between two

Stations. The Home or DX stations may be selected from the DX Locations Sub

Menu. In addition to the outputs provided in the Moon calculation option, the

Local Time, Elevation, Azimuth, DX Doppler and Polarisation are given.

Sun Position.

This option provides the means of calculating the Sun Position for Noise

measurements. Should you wish real time measurements it is suggested that you

use the Real Time Track option with a Sun "Target". (refer General Setup)

Noise Source Position.

This option provides calculations for Azimuth, Elevation of both Quiet and

Noise Sources. Again, as with previous Calculation Options you may obtain a

hard copy print out by selecting this option from the General Set Up Menu.

Should you wish real time measurements it is suggested that you use the Real

Time Track option with a Noise Source as a "Target". (refer General Setup)

EME Budget Calculator.

This program calculates the S/N ratio of the returned Echo that may be expected

by varying any or all of the following parameters for Stations A and B.

- frequency band   - Transmitter Output Power - Transmitter feed line loss

- RX Noise Figure  - RX feed line loss        - Receiver Bandwidth

- Antenna Gain in dBi (referenced to isotropic)

Note ... [F2] Will Save this Data.

[F3] Provides a screen which will allow you to calculate your Antennae

Gain (in dBi) from your measured -3dB Beam Width. When converting

from Antennae Gain back to Antennae Beam Width the figures shown

assume equal E and H Plane angles. In the converse situation you

may input both E and H Plane angles and the combined Antennae

gain will be calculated.

Move around the screen with the [Arrow] keys and select the item to change with

the [Return] key. Change the data and complete the entry with [Return]. You may

change any of the variables for both Station A and Station B and so determine

the respective returned S/N ratio of EACH Station and the S/N ratio between the

Stations. Accordingly, it is possible to determine if communication is possible

between stations of varying capabilities.

Note ... The calculations are based upon Perigee Path Loss (minimum loss) and

relate to antenna temperatures of :-

2200 degrees Kelvin for  50 MHz

290 degrees Kelvin for 144 MHz.

50 degrees Kelvin for 432 MHz and all higher frequencies.

Y Factor Calculator.

This calculation provides a means of comparing a Noise Source to a Quiet Source

and relate such measurements to your overall receive system performance.

Reference should be made to YU1AW article in VHF Communications of 1/84 to

obtain a detailed understanding of the calculation. In brief, the operation is

as follows. When you select this calculation the cursor will rest at the Y

Factor position. (Dummy figures have been added to provide the initial

calculation). The Y Factor is a ratio in dB measured between two sources.

(generally a Quiet and Noise source). Thus, from the coordinate obtained from

the Noise Source Position option, aim your array first at a selected Quiet

Source. (choose one that is well elevated from the Horizon). Use this as your

reference and calibrate the level indicator. Next aim your array at a Noise

Source, (again choose one well elevated from the Horizon), and then measure the

increase in noise to that as referenced to the Quiet source. The ratio between

the Quiet and Noise Source is the Y Factor. Enter this value into the program

or other variables ( [arrow] keys to position and [CR] to enter), what you wish

to change. Don't forget to enter your Antenna gain (in dBi). Similarly change

the noise sources to those you have chosen for your measurement. The Receiver

Noise Temperature and Noise Figure will then be displayed.

Note ...

The Y Factor is a most compressed figure where a small change in value will

result in a large change in the RX Noise Figure and RX Noise Temperature.

Accordingly, the results should be treated with appropriate caution.

ooo OOO ooo

DX LOCATIONS - MENU OPTIONS

Select a DX QTH from the Data Base.

This option allows the user to select any one of the DXCC locations and use the

data to calculate common Moon Windows. You may also recall any of your own

added locations (Page 8 onwards), for similar calculations. You may also select

any Grid Square Location via the [F2] option.

Select a Home QTH from the Data Base.

As explained above, this facility provides the ability to recall a DXCC QTH but

in this option the data is used as a HOME Location. You may also select any

Grid Square Location via the [F2] option.

Edit the User Data Base.

This option allows the you to Edit any of the user added DX Locations.

Add a New QTH to the User Data Base.

This option allows you to add a new QTH to the Data Base. This may be done in

either of two ways. Firstly a detailed entry can be made, or secondly by

selecting [F2], and entering a Grid Square, the computed data will be stored.

Delete a QTH from the User Data Base.

This option allows you to delete a user added QTH from the User Data Base.

Grid and Distance Calculator.

This accurate calculator allows you to calculate both Grid and Distances by

either selecting a Home and DX Station from the Data Base, or typing in the

Data. Use the [Arrow] keys to select the option and the [CR] to input Data.

It will also provide the direct path bearing.

Current Settings - Indicator Only

Home ... displays the current setting of the Home Location.

DX   ... displays the current setting of the DXLocation.

ooo OOO ooo

GENERAL SETUP - MENU OPTIONS

Home Location Change.

This option provides you with the ability to set this program for your own Home

Location. The Data you enter here is used on initiating the Program. The input

is in degrees, minutes and seconds. The UTC Zone is the value in hours ahead or

behind Greenwich Meridian. ie Germany +1, England 0 etc.

Calculation Interval in Minutes.

This option allows you to select the calculation interval as desired.

ie. 60 would provide a calculation every hour. (Maximum  1440 minutes).

Printer On/Off.

This option allows you to select the Printer and Paper size as desired or

set a User Defined page length, top and bottom margins. You may cycle through

the paper size options with a [CR]. When you reach the User Defined option a

screen will be displayed allowing you to set this user option as desired.

11" paper is set for 66 lines per page with 5 lines top and bottom margins.

A4  paper is set for 67 lines per page with 1 line top and bottom margins.

Selection of your print option is also available in all the calculation screens

prior to entering the day, month, start time and year. Note .. you cannot

change the user defined parameters except from within the General Setup Menu.

Remember to save the user defined printer parameters as they are stored and

then loaded into the program on start up.

Local Winter/Summer Time.

This option allows you to select either Summer (+1 hour), or Normal Time.

Operating Frequency.

This option allows you to select any frequency from 50 MHz to 24 GHz.

The frequency is selected by cycling through the band options with a [CR].

Antennae Beam Width in Degrees.

Enter your beam width of your array. (Used for coincident Sun calculations).

A provision for calculating this value is available in the EME Budget Screen

by selecting the [F3] key.

Antennae Mount.

If you have Polar Mount then a display of G.H.A and Declination (in lieu of

the normal Azimuth and Elevation), is displated in the Real Time Track Screen.

The DX Station in this screen, (if selected), will be the normal Az/El.

Save Set up Parameters.

This option allows you to save all the above options to Disk.

Local/UTC Computer Time.

This option allows the user to run your Computer Time as UTC if desired.

The options are selected with a [CR].

Transmit Sequence.

This option allows the user to select either First or Second TX sequencing.

The options are selected with a [CR].

Period to Transmit.

The user may select Transmit periods of 1/2,1, 2 or 2 1/2 Minutes as desired

for use in the Real Time Track Program. The options are selected with a [CR].

Display Source Target.

The user may select either the Moon, Sun or any Noise Source for use in the

Real Time Track calculation routine.

Alarm Sound On/Off.

The user may turn the Alarm On/Off that is used in the Real Time Track Program.

The options are selected with a [CR].

Save Set up Parameters.

This option allows you to save all the above options to Disk.

*** Change Computer Time ***

You may correct your own Computer Time in this option. Simply select the

highlighted figure you wish to change, with the [arrow] keys, and use the [+/-]

keys to increment or decrement the value as required. Use [Esc] to exit back to

return to the Sub Menu.

ooo OOO ooo